3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230013

e-meeting, 11th – 20th January 2023

3GPP TSG-SA WG1 Meeting #100	S1-223734

14 - 18 November, 2022, Toulouse, France

# Title:	Reply LS on QoS Sustainability analytics and V2X service adaptations

# Release:	Rel-18, Rel-19

# Work Item:	-

Source:	SA1

To:	SA

Cc:	SA2, SA6

Contact person:	Yang Xu <xuyang@oppo.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA1 thanks to 5GAA’s LS and have discussed the questions in the LS based on the Section 6.1 and 6.2 in the 5GAA Technical Report, SA1 would like to provide the following answer to Q.1.2 and Q.1.3:

Q.1.2 in 5GAA LS: In the context of Requirement 2 “Improve freshness of QoS predictions (e.g. notice period)”, it is a key 5GAA requirement that notifications on potential QoS changes are delivered to the application sufficiently in advance with respect to the QoS change. Such in-advance time (or notice period) should give the application sufficient time to implement the application reaction before the QoS change happens. The notice period (usually between 6 and 18 seconds according to current PRESA analysis) determines how early - before the predicted QoS change - should the notification be delivered to the consumer. 5GAA understands that notice period can be supported for QoS Sustainability analytics when the consumer includes the parameter “Time when the analytics are needed” in the subscription/request. 5GAA understands that the value for such parameter should be set to a time in the future so that “Time when the analytics are needed” + “notice period” is earlier or equal to the start time of the “Analytics target period” in the future. Can 3GPP confirm that the parameter “Time when the analytics are needed” can be used for such purpose for QoS Sustainability, if there are any limitations or constraints to this approach and if the notice period values of 6-18 seconds resulting from PRESA analysis are compatible with Release 18?

SA1 answer: In Rel-18 TS 22.261 clause 6.40, there are two relevant requirements as below.

-  Based on operator policy, the 5G system shall be able to provide an indication about a planned change of bitrate, latency, or reliability for a QoS flow to an authorized 3rd party so that the 3rd party AI/ML application is able to adjust the application layer behaviour if time allows. The indication shall provide the anticipated time and location of the change, as well as the target QoS parameters

- 5G system shall be able to provide event alerting to an authorized 3rd party, together with a predicted time of the event (e.g., alerting about traffic congestion or UE moving into/out of a different geographical area).

NOTE 3:	A 3rd party AI/ML application may use the prediction information to minimize disturbance in the transfer of learning data and AI/ML model data

Q.1.3 in 5GAA LS: In the context of Requirement 6 “Investigate harmonization of QoS change notifications for V2X application (e.g., QoS change notification sent to the UE)” and 3GPP study item [3], KI#2, some of the proposed solutions include the possibility to share QoS Sustainability with the UE directly:

SA1 answer: TS22.261 clause 6.10 contains the following requirements from the Rel18 work on Subscriber aware northbound API access (SP-200797) addressing the issue in Q.1.3 as below

The 5G system shall be able to:

-	provide a third-party with secure access to APIs (e.g. triggered by an application that is visible to the 5G system), by authenticating and authorizing both the third-party and the UE using the third-party's service.

-	provide a UE with secure access to APIs (e.g. triggered by an application that is not visible to the 5G system), by authenticating and authorizing the UE.

-	allow the UE to provide/revoke consent for information (e.g., location, presence) to be shared with the third-party.

-	preserve the confidentiality of the UE's external identity (e.g. MSISDN) against the third-party.

-	provide a third-party with information to identify networks and APIs on those networks.

2. Actions:

To SA

SA1 kindly asks SA to take the above answer into account.

3. Date of Next SA1 Meetings:

SA1#101	13-17 February 2023	EU

SA1#102	15-19 May 2023	China, CN