3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230038

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0155 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New AC-EEC procedure to invoke UE ID request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new procedure is needed to support the retrieval of the UE ID for AC’s (and hence EAS’s) dealing with IPv4 NATed IP address issue. That is when EAS’s direct invocation of UE Identifier API procedure (i.e. Eees_UEIdentifier API) due to NATed IP address failes to identify the UE then the EAS may use the AC to ask the EEC for the UE ID. Therefore, this procedure is provided to enable the AC interact with the EEC to obtain the UEID and behind the scene pass on the UE ID to the EAS. In turn the EEC would rely on its interaction with the EES to obtain the UEID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clause 8.14.1 & 8.14.2.1 to list the new UE ID retrieval procedure. Also, adding the detales of the new procedure (Edge UE ID request) in the new clause 8.14.2.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retrival of the UE ID corresponding to the UE’s NATed IP address is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.1, 8.14.2.1, 8.14.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

## 8.14	EDGE-5 APIs

### 8.14.1	General

EEC exposes EDGE-5 APIs corresponding to EEC's capabilities, for the AC to request EEC's services for edge enablement. Using these APIs, ACs request the EEC for EEL services. EDGE-5 APIs include one-time request/response operations for EAS discovery, retrieval of Edge UE ID and ACR operations. Additionally, the AC can request for an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required.

NOTE 1:	Details on how the AC and EEC communicate with each other is out of scope.

NOTE 2:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

### 8.14.2	Procedures

#### 8.14.2.1	General

Following procedures are specified for EDGE-5:

-	Registration;

-	EAS discovery;

-	ACR request;

-	EEC services subscription; and

-	Edge UE ID request

# * * * Change 2 * * * *

#### 8.14.2.x	Edge UE ID request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 8.14.2.x-1: Edge UE ID request procedure

1.	The AC sends an Edge UE ID request to the EEC. The request includes AC's security credentials, and AC profile.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC process the request from the AC. The EEC uses the UE Identifier API procedure (see clause 8.6.5) to query the EES.

NOTE:	This procedure may be used to support the retrieval of the UE ID for AC’s (and EAS’s) dealing with IPv4 NATed IP address issue (i.e. EAS’s direct invocation of UE Identifier API procedure due to NATed IP address failes to identify the UE). Under such a scenario then it is understood that, the AC through application signaling which is outside the scope of this document, would pass on the the Edge UE ID (received in step 3) to the EAS so that it can be used to invoke capability APIs over EDGE-3 or EDGE-7.

# * * * End of changes * * * *