3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230039

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0156 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating UE Identifier API procedure to enable EEC invoke UE ID request for NATed IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the existing UE Identifier API procedure and hence the EES-exposed Eees_UEIdentifier API is used by the EAS only. However, Eees_UEIdentifier API doesn’t support the retrieval of the UE ID for a NATed IP address. The UE Identifier API procedure is required to change so that EEC can also be a new consumer of this API in order to support the retrieval of the UE ID for AC’s (and hence EAS’s) dealing with IPv4 NATed IP address issue (as stated above). So long as EAS’s direct invocation of Eees_UEIdentifier API with UE NATed IP address failes due to lack of support of NATed IP address by NEF, the EAS (through application signaling) may use the AC to ask the EEC for the UE ID. Under such circumstances, EEC would need to interact with the EES to reteive the UE ID; hence the need to update the UE Identifier API procedure in order to allow such an interaction from the EEC. As an example, EEC invocation of Eees_UEIdentifier API may rely on the dual-stack capability of the UE and the usage of the IPv6 in its API request and hence allowing the unique identification of the UE by the NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clauses 8.6.5.1 and 8.6.5.2 to allow the EEC to also be a new consumer of the UE Identifier API. Updating clause 8.6.5.3.2 (making user information IE Optional) in order to allow source IP address of the EEC to be used instead of requiring IP address as an IE in the request body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Retrieval of the UE ID corresponding to the UE’s NATed IP address by EEC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5.1, 8.6.5.2, 8.6.5.3.2, 8.6.5.4.1 & 8.6.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 8.6.5	UE Identifier API

#### 8.6.5.1	General

EES exposes UE Identifier API to the EAS and EEC in order to provide an identifier uniquely identifying a UE. This API is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. hasn’t already cached). This identifier, called UE ID, is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 or EDGE-7. The UE ID is specific to the given EAS and it is represented as a GPSI assigned by the 3GPP Core Network.

#### 8.6.5.2	Procedure

Figure 8.6.5.2-1 illustrates the interactions between the EES and the EAS.

Pre-conditions:

1.	The EAS or EEC is authorized to discover and to use UE Identifier API provided by the EES.

Figure 8.6.5.2-1: UE Identifier API

1.	The EAS or EEC invokes UE Identifier API exposed by the EES.

2.	The EES uses the received user information in the step 1 (e.g. IP address) and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3].

NOTE 1:	If EEC is the one invoking the UE Identifier API, the EES may use the source IP address in the request to obtain the associated UE ID by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [3].

NOTE 2:	If EEC is the one invoking the UE Identifier API, the Eees_UEIdentifier needs to be an IPv6 service endpoint in order to overcome the private IP address overlap issue.

3.	The EES provides the obtained UE identifier as UE ID to the EAS to EEC (i.e. whichever invoked the API).

Editor's note:	[SA3] Whether and how user's consent is obtained to share the UE identifier with a particular EAS is SA3's responsibility.

4.	The EAS uses the UE ID received in step 3 to invoke capability exposure API(s) provided by the EES over EDGE-3.

#### 8.6.5.3	Information flows

##### 8.6.5.3.1	General

The following information flows are specified for UE Identifier API:

-	UE Identifier request and response.

##### 8.6.5.3.2	UE Identifier API request

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| User information | O | Information about the User or UE available in the EAS or EEC, e.g. IP address. |
| EAS ID | M | Identifier of the EAS providing the application services. |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Security Credentials | M | Security credentials of the EAS. |
| NOTE: This IE shall be present when the API is invoked by EAS. When EEC invokes the API this IE may or may not be present (if present it shall be ignored as source IP address of the request takes precedence). |  |  |

Editor's Note: [SA3] Whether the EAS ID and the EAS Provider ID are part of the security credential is SA3's responsibility.

##### 8.6.5.3.3	UE Identifier API response

Table 8.6.5.3.3-2: UE Identifier API response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the UE identifier request was successful. |
| > UE ID | M | Identifier uniquely identifying the UE. |
| Failure response | O | Indicates that the UE identifier request failed. |
| > Cause | O | Indicates the cause of UE identifier request failure |

#### 8.6.5.4	APIs

##### 8.6.5.4.1	General

Table 8.6.5.4.1-1 illustrates the APIs for UE Identifier.

Table 8.6.5.4.1-1: Eees_UE_Identifier API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_UEIdentifier | Get | Request/Response | EAS, EEC |

##### 8.6.5.4.2	Eees_UEIdentifier_Get operation

API operation name: Eees_UEIdentifier_Get

Description: The consumer requests identifier of a UE.

Inputs: See clause 8.6.5.3.2.

Outputs: See clause 8.6.5.3.3.

See clause 8.6.5.2 and 8.14.2.x for details of usage of this operation.