3GPP TSG-SA6 Meeting #52-bis-e	S6-230086

# Online, , 11th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0158 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding a CAPIF deployment for exposure of EAS Service APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FS_eEDGEAPP study, the enablement of Service APIs exposed by EAS was studied as Key Issue #2 in TR 23.700-98; and concluded to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs within the EDGEAPP architecture by exploiting CAPIF as specified in clause 7.8 of TR 23.700-98. As per the conclusion of the study, EES may provide CAPIF core function to support discovery and publish of the EAS Service APIs, which needs to be provided in TS 23.558 as a CAPIF deployment in the Annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs within the EDGEAPP architecture, the following changes are proposed: - Clase 4.5: A.5.4 is added as a reference for the details of supporting exposure of EAS Service APIs Annex A.5.4: new clause is added to provide a CAPIF deployment to support for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Exposing EAS Service APIs within the EDGEAPP architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5, A.5.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI #2 - Solution #8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

4.5	Capability exposure to EAS

The edge enabler layer exposes services towards the EASs. The exposed capabilities include the services of the Edge Enabler Layer and the re-exposed and enhanced services of the 3GPP core network. The capabilities exposed by the edge enabler layer are specified in clause 8.6 and the 3GPP network capability exposure is specified in clause 8.7. Other application layer capabilities like application enabler services and SEAL services may be exposed via edge enabler layer as per CAPIF as illustrated in Annex A.4.

The edge enabler layer also supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs via CAPIF as specified in 3GPP TS 23.222 [6] by providing CAPIF core function within the EES to support publish and discovery of EAS Service APIs. The details are provided in Annex A.5.4.

2nd CHANGE

A.5.4	Supporting Exposure of EAS Service APIs using CAPIF

The EES supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs by providing CAPIF functions as shown in Figure A.5.4-1.

Figure A.5.4-1: EES supporting CAPIF functions for exposure of EAS Service APIs

In EDN 1, the EES and the EAS are in the different ECSP trust domain. In EDN 2, the EAS (API invoker), EAS 2 (API provider), and EES are within the same ECSP trust domain, whereas the EAS 3 (API provider) is not.

The EES of an EDN provides the following functions for exposure of EAS Service APIs:

-	The CAPIF core function as specified in 3GPP TS 23.222 [6] to support onboarding of EASs (API invokers), publish of EAS Service APIs, discovery of EAS Service APIs, and charging of EAS Service APIs invocations.

The following procedures are performed as specified in 3GPP TS 23.222 [6]:

-	The EAS (API provier) acts as an API provider by supporting API provider domain functions (i.e., API exposing function, API publishing function, and API management function), and its Service APIs are published to the EES (CAPIF core function 1 or CAPIF core function 2) within the same EDN. The EESs (CAPIF core function 1 or CAPIF core function 2) further publishes the EAS Service APIs to the EES supporting a centralized CAPIF core function (CAPIF core function 3) to support discovery of the EAS Service APIs across different EDNs (EDN 1 and EDN 2).

-	The EAS (API invoker) acts as an API invoker and is onboarded to the EES (CAPIF core function 1 or CAPIF core function 2) within the EDN.

-	The EASs (API invokers) are authenticated with EES (CAPIF core function 1 or CAPIF core function 2).

-	The EAS (API invokers) discovers the EAS Service API(s) via the EES (CAPIF core function 1 or CAPIF core function 2) within the EDN including the end point address of the API exposing function where the service API invocation is to be performed.

-	The EAS (API invokers) obtains authorization to invoke the EAS service APIs from the EES (CAPIF core function 1 or CAPIF core function 2).

-	The EAS (API invokers) invokes the EAS Service APIs after authorization by the EAS (API provider).

NOTE:	EES supporting CAPIF core function may also provide the support for logging, audit, and access control of EAS Service API(s) as specified in 3GPP TS 23.222 [6].

END OF CHANGES