3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230097

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0164 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Acquiring user consent for each AC through EDGE-5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 22023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the AC can obtain user consent directly, the EEC can provide this along with the AC information that has obtained user consent to the EEL so that the EEL can use the functionality for the AC-specific UE. In the 23.558 8.6.5 UE Identifier API, the EES can provide the UE ID to the EAS. Here, it is not yet clear how to obtain user consent for sharing the UE ID with the EAS. And it has EN that ‘[SA3] Whether and how user's consent is obtained to share the UE identifier with a particular EAS is SA3's responsibility.’ However, as the AC-EEC API is defined through EDGE-5 from the SA6 scope and there is responsible for how to use user consent where 23.700-98 4.4 Key issue #4: EDGE-5 ‘NOTE 2: The aspects of the usage of end-user consent/authorization over APIs is in the scope of SA6.’ the EES does not have to perform additional procedures to issue user consent if the EEC can receive user consent from each AC and provide user consent to the EES to send the UE ID to the EAS requested by the EES. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating clause 8.14.1 - 8.14.3 to list the new User consent application. And removing the following EN: SA3 recommendations, if any, on how the user or the AC can consent, e.g., to the disclosure of location information and the use of the AC ID in the signalling towards the network are FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User consent per AC cannot be applied to EEL. EES cannot provide UE ID to EAS without user consent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.1, 8.14.2, 8.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.14	EDGE-5 APIs

8.14.1	General

EEC exposes EDGE-5 APIs corresponding to EEC's capabilities, for the AC to request EEC's services for edge enablement. Using these APIs, ACs request the EEC for EEL services. EDGE-5 APIs include one-time request/response operations for EAS discovery, obtains user consent and ACR operations. Additionally, the AC can request for an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required. The EEC obtains user consent for each AC for the EEL service that requires user consent.

NOTE 1:	Details on how the AC and EEC communicate with each other is out of scope.

NOTE 2:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

* * * Next Change * * * *

8.14.2.2.2	AC registration

Figure 8.14.2.2.2-1 illustrates AC registration procedure.

Pre-conditions:

- AC can communicate with the EEC.

- AC can obtain user consent for the AC service to be used at the edge while being installed and created.

Figure 8.14.2.2.2-1: AC registration procedure

1.	The AC sends an AC registration request to the EEC. The request includes the AC profile, AC's security credentials, user consent and optionally the EAS characteristics. The request may also include a list of EEC's services that AC requires the EEC to handle. The request additionally includes ECS configuration information if the AC is edge-aware and configured with the ECS configuration information.

NOTE 1:	The ASP providing the AC and the ECSP providing the ECS can have edge computing service provider service agreement as described in Annex B. The ECS configuration information configured in the AC is based on the service agreement.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC registers the information provided in the request and responds back to the AC with AC registration response. The AC registration response includes the list of EEC's services that AC is authorized for.

NOTE 2:	The mechanisms used for authentication and authorization between AC and EEC is out of scope of this specification. EEC can use local policies, user preferences, ASP services agreement(s) (see Annex B) to authorize the request from the AC.

NOTE 3:	When the ECS configuration information is provided from an AC, the EEC can use the ECS configuration for initial service provisioning for the AC that provided the ECS configuration information if there is no ECS configuration information is provided from the 5GC.

8.14.2.2.3	AC registration update

Figure 8.14.2.2.3-1 illustrates AC registration update procedure.

Pre-conditions:

- AC is registered with the EEC.

- AC can obtain user consent for the AC service to be used at the edge while being installed and created.

Figure 8.14.2.2.3-1: AC registration update procedure

1.	The AC sends an AC registration update request to the EEC. The request includes the registration ID, AC's security credentials, user consent, and may include the updated AC profile, EAS discovery filters, list of requested EEC services and list of ECS information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC sends a successful registration update response, which includes an updated list of EEC services that AC is authorized for.

* * * Next Change * * * *

8.14.2.3	EAS discovery

Pre-conditions:

- The AC can communicate with the EEC.

- AC can obtain user consent for the AC service to be used at the edge while being installed and created.

Figure 8.14.2.3-1: EAS discovery request procedure

1.	The AC sends an EAS discovery request to the EEC. The request includes AC profile and AC's security credentials, user consent and may include EAS discovery filters.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC determines if the required EAS is available or not. The EEC may use information cached or preconfigured at the EEC or may use the EAS discovery procedures to query the EES. If step 1 includes the AC profile or EAS discovery filters, then the EEC may utilize the provided AC profile and filters, to form the EAS discovery request towards EES. If step 1 does not include any of the optional IEs of the AC profile and EAS discovery filters, and AC registration was performed, the EEC may utilize the AC profile provided by the AC during AC registration. The EEC also needs to take user privacy requirements, e.g., regarding the disclosure of location information towards the network into account. If required, e.g., when EAS discovery procedures returns a list of EASs, the EEC performs EAS selection based on the information received in step 1 and the AC profile. The EEC can perform EAS discovery with different EESs before selecting an EAS.

NOTE 1:	If required, the EEC can perform service provisioning procedure, or EEC registration procedure or both, before performing the EAS discovery procedures. EEC may already have captured EESs and EASs availability for present location; so that the AC's request (step #1) can be replied to quickly and efficiently.

NOTE 2:	The EEC can include AC profiles of more than one AC in the EAS discovery request sent to the EES.

4.	The EEC responds back to the AC with the EAS discovery response. The response includes the EAS profile(s) of the available EAS(s).

* * * Next Change * * * *

8.14.2.5.2	Subscribe

Pre-conditions:

- The AC can communicate with the EEC.

- AC can obtain user consent for the AC service to be used at the edge while being installed and created.

Figure 8.14.2.5.2-1: EEC services subscription procedure

1.	The AC sends an EEC services subscription request to the EEC. The request includes AC profile, AC's security credentials, user consent, a list of EEC's services that AC requires the EEC to handle, and related parameters as described in 8.14.3.10. If the subscription request includes:

-	EAS discovery or EAS dynamic information subscription, then the request may include a list of EAS characteristics and a list of EAS dynamic information filters respectively;

-	ACR, then the request includes a list of S-EAS information and corrosponding type of ACR operations:

-	ACR monitoring, where the EEC monitors the need for ACR and notifies the AC as and when required e.g., on receiving ACR related notifications on EDGE-1 interface.

-	EEC managed ACR, where the EEC monitors the need for ACR. If need for ACR is detected, then the EEC decides and initiates ACR using one of the EEC initiated ACR scenarios. The EEC notifies the AC about the imminent ACR and may include the target information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC creates the subscription and sends an EEC services subscription response message to the AC. The response includes the list of services that the EEC will handle and related details.

4.	The EEC executes the services e.g., EAS discovery, ACR, and notifies the AC with information as necessary. The EEC may use locally cached information or configurations while providing services to the AC. The EEC may provide user consent to the EEL when performing the EEC services.

* * * Next Change * * * *

8.14.2.5.4	EEC services subscription update

Figure 8.14.2.5.4-1 illustrates EEC services subscription update procedure.

Pre-conditions:

- The AC has subscribed to the EEC.

- The EEC may provide user consent to the EEL when performing the EEC services.

Figure 8.14.2.5.4-1: EEC services subscription update procedure

1.	The AC sends an EEC services subscription update request to the EEC. The request includes the subscription ID, AC's security credentials, and may include updated notification related details or updated list of required EEC services.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC updates the subscription and sends a successful subscription update response.

4.	The EEC executes the services e.g., EAS discovery, ACR, and notifies the AC with information as necessary. The EEC may use locally cached information or configurations while providing services to the AC. The EEC may provide user consent to the EEL when performing the EEC services.

* * * Next Change * * * *

8.14.3	Information flows

8.14.3.1	General

8.14.3.2	AC registration request

Table 8.14.3.2-1 describes information elements of the AC registration request sent by the AC to the EEC .

Table 8.14.3.2-1: AC registration request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile of the AC sending the registration request. |
| Security credentials | M | Security credentials of the AC sending the registration request. |
| User consent | O | User consent to AC |
| > UE ID | O | AC ID to which user content is applied |
| > Expiration time | O | Indicates the expiration time of the user consent. This is the time that expires in EEL when it is provided as EES by EEC. |
| List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |
| List of requested EEC services | O | One or more EEC services requested by the AC e.g., EAS discovery, ACR. |
| List of ECS information | O | One or more ECS configuration information |

* * * Next Change * * * *

8.14.3.4	AC registration update request

Table 8.14.3.4-1 describes information elements in the AC registration update request sent by the AC to the EEC.

Table 8.14.3.4-1: AC registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | AC registration identifier provided by the EEC during AC registration. |
| Security credentials | M | Security credentials of the AC sending the registration update request. |
| User consent | O | User consent to AC |
| > UE ID | O | AC ID to which user content is applied |
| > Expiration time | O | Indicates the expiration time of the user consent. This is the time that expires in EEL when it is provided as EES by EEC. |
| AC profile | O | AC profile of the AC sending the registration update request. |
| List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |
| List of requested EEC services | O | One or more EEC services requested by the AC e.g., EAS discovery, ACR. |
| List of ECS information | O | One or more ECS configuration information |

* * * Next Change * * * *

8.14.3.8	EAS discovery request

Table 8.14.3.8-1 describes information elements for the EAS discovery request sent by the AC to the EEC.

Table 8.14.3.8-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile of the AC sending the registration request. |
| Security credentials | M | Security credentials of the AC. |
| User consent | O | User consent to AC |
| > UE ID | O | AC ID to which user content is applied |
| > Expiration time | O | Indicates the expiration time of the user consent. This is the time that expires in EEL when it is provided as EES by EEC. |
| List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |

* * * Next Change * * * *

8.14.3.10	EEC services subscription request

Table 8.14.3.10-1 describes information elements for the EEC services subscription request sent by the AC to the EEC.

Table 8.14.3.10-1: EEC services subscription request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile of the AC requesting subscription |
| Security credentials | M | Security credentials of the AC. |
| User consent | O | User consent to AC |
| > UE ID | O | AC ID to which user content is applied |
| > Expiration time | O | Indicates the expiration time of the user consent. This is the time that expires in EEL when it is provided as EES by EEC. |
| Notification details | M | Details on how the EEC can notify the AC. |
| Required EEC services | M | EEC handled services required by the AC |
| > EAS discovery | O | Indicates that AC requires EEC to handle EAS discovery. |
| >> List of EAS characteristics | O | List of EAS characteristics as described in EAS discovery filters (see Table 8.5.3.2-2). |
| > EAS dynamic information | O | Indicates that the AC requires EAS dynamic information subscription |
| >> List of EAS dynamic information filters | M | List of EAS dynamic information filters as described in Table 8.5.3.4-2. |
| > ACR | O | Indicates that the AC requires ACR related services for the following EASs |
| >> List of S-EAS information | M | S-EAS information corresponding to the ACR operation |
| >>> ACR operation | O | Indicates if AC requires ACR monitoring or EEC managed ACR corresponding to each S-EAS |

* * * Next Change * * * *

8.14.3.13	EEC services subscription update request

Table 8.14.3.13-1 describes information elements for the EEC services subscription update request sent by the AC to the EEC.

Table 8.14.3.13-1: EEC services subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the update request. |
| Security credentials | M | Security credentials of the AC. |
| User consent | O | User consent to AC |
| > UE ID | O | AC ID to which user content is applied |
| > Expiration time | O | Indicates the expiration time of the user consent. This is the time that expires in EEL when it is provided as EES by EEC. |
| Notification details | O | Details on how the EEC can notify the AC. |
| Required EEC services | O | EEC handled services required by the AC |
| > EAS discovery | O | Indicates that AC requires EEC to handle EAS discovery. |
| >> List of EAS discovery filters | M | List of EAS discovery filters as described in Table 8.5.3.2-2. |
| >>> Preferred ECSP list | O | Preferred ECSP list corresponding to each set of EAS discovery filters. |
| > EAS dynamic information | O | Indicates that the AC requires EAS dynamic information subscription |
| >> List of EAS dynamic information filters | M | List of EAS dynamic information filters as described in Table 8.5.3.4-2. |
| > ACR | O | Indicates that the AC requires ACR related services for the following EASs |
| >> List of S-EAS information | M | S-EAS information corresponding to the ACR operation |
| >>> ACR operation | O | Indicates if AC requires ACR monitoring or EEC managed ACR corresponding to each S-EAS |

* * * End of Change * * * *