3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230115

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0165 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update ACR parameter information procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NOTE in ACR parameter information procedure contains normative operations for sending acknowledgement message for ACR management notification. Thus, that normative operation needs to be moved into the dedicated step and the procedure should be update accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update ACR parameter information procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACR parameter information procedure remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to KI#3 / Sol #21 in TR 23.700-98. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

8.8.3.9	ACR parameter information procedure

Figure 8.8.3.9-1 illustrates the procedure for sending ACR parameters from S-EES to the T-EES and T-EAS. The procedure may be used by the S-EES at the beginning of the ACR execution to provide ACR parameters, e.g. prediction expiration time, to the T-EES and T-EAS. The procedure may also be used during an ongoing ACR to update ACR parameters.

Pre-condition:

1.	The ACR has been launched to the S-EES.

Figure 8.8.3.9-1: ACR parameter information procedure

1.	The S-EES sends the ACR parameter information to the T-EES.

2.	The T-EES checks if the requestor is authorized for this operation. If authorized, the T-EES processes the request

3.	If the request is authorized and if the T-EAS has subscribed to receive ACR notifications, the EES shall notify the T-EAS by sending an ACR management notification, with "ACT start" event including ACR parameters from the request in step 1, e.g. Prediction expiration time.

4.	The T-EAS sends the Acknowledgement message for the received ACR management notification to the T-EES.

The T-EAS may use the ACR parameters to determine whether to accept the ACR for service continuity planning. If the prediction expiration time is provided in step 3, the T-EAS considers prediction expiration time in deciding whether and how long to wait for the AC of the UE to connect to it. If the T-EAS decides not to wait for the time period indicated by Prediction expiration time, the T-EAS sends the Acknowledgement message including rejection to the ACT.

NOTE:	The T-EAS can use the ACR parameters to handle the ACR. For example, the T-EAS can consider prediction expiration time in deciding whether and for how long to wait for the AC of the UE to connect to it. If the ACT is performed, and the AC does not connect to the T-EAS by "Prediction expiration time", the EAS can send ACT failure with the appropriate cause to the EES. In that case, the T-EAS can delete the transferred application context.

5.	The EES responds to the requestor's request with an ACR parameter information response message.

Editor's note: The changes to the ACR scenarios in clause 8.2.2 to include ACR parameter information is FFS.

* * * End of Change * * *