3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230139

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0168 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS discovery support for constraint UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EAS discovery procedure can be enhanced for constraint UE considering the study sol#18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Improved EAS discovery procedure for constraint UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No discovery procedure support for constraint UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.2, 8.5.3.2, 8.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#15, Sol#18 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.5.2.2	Request-response model

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.5.2.2-1: EAS Discovery procedure

1.	The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator or UE type.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location. If the EEC indicates that service continuity support is required, the EES shall take the indication which ACR scenarios are supported by the AC and the EEC and which of these are preferred by the AC into consideration. The EES may select one EAS and determine to perform application traffic influence for this AC based on AC’s service KPI or EAS’s service KPI in desired response time, when the EES does not perform traffic influence in advance.

When EAS discovery filters are not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

Upon receiving the request from the EEC, the EES may trigger the EAS management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12.

If EAS selection request indicator is set or UE type indicates constraint device, the EES shall select an EAS among discovered EAS(s) and the selected EAS will be included in the discovered EAS list.

3.	If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs. For discovered EASs, this includes endpoint information. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc. The EAS discovery response may contain a list of EASs. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required.

If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request.

Upon receiving the EAS discovery response, the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE.

NOTE 4:	Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command).

NOTE 5:	The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document.

NOTE 6:	The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required.

If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request. If the failure cause indicated that no suitable EAS is available, the EEC resends the EAS discovery request to next candidate EES retrieved during service provisioning.

NOTE X:	As long as a proper EAS is discovered and selected, EEC of a constraint UE can stop repeating EAS discovery to rest candidate EES(s).

* * * Next Change * * * *

#### 8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |
| Target DNAI (NOTE) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support (NOTE) | O | The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR. |
| EAS Service Continuity Support (NOTE) | O | The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR. |
| EAS selection request indicator | O | Indicates the request for EAS selection support from the EES (e.g., for constrained device). |
| UE type | O | Indicates UE or device type (e.g. constrained device) |
| NOTE: This IE shall not be included when the request originates from the EEC. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

* * * Next Change * * * *

#### 8.5.3.3	EAS discovery response

Table 8.5.3.3-1 describes information elements for the EAS discovery response from the EES to the EEC.

Table 8.5.3.3-1: EAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EAS discovery request was successful. |
| > Discovered EAS list | O | List of discovered EAS(s). Each element includes the information described below. If the EAS selection request indicator is set or UE type indicates constraint device in the EAS discovery request, then Discovered EAS list shall contain only one selected EAS. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4 |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| Failure response | O | Indicates that the EAS discovery request failed. |
| > Cause | O | Indicates the cause of EAS discovery request failure. |

* * * END of Change * * * *