3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230164

e-meeting, 11th – 20th January 2023	(revision of S6-223591, S6-223440, S6-223205)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0093 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | usage of device triggering before store and forward |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, one2many B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The device triggering capability is used in message store and forward before the Message expiration time is expired. It can also be used before the store and forward is used to reduce the delay of delivery. This CR is proposed to add this feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | usage of device triggering before store and forward |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the device is not available and S/F is used, the message can be delivered to the device only if the device goes online or device triggering is used befored the Message expiration time is expired. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6, 8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: S6-223440 is a merger of S6-223205 and S6-223111. The Store and Forward procedure is updated to make it clearer. Rev2: Delete pre-condition 2, add a new pre-condition as pre-condition 2. Editorial changes. Rev3: pre-condition added in Rev2 is removed again. The Pre-condition 1 is modified. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.6	MSGin5G Store and Forward

Figure 8.3.6-1 shows the procedure for providing store and forward services for MSGin5G message requests.

This procedure applies when an MSGin5G inbound message is received at the MSGin5G Server and the recipient UE is not available; otherwise, the procedure detailed in clause 8.3.2 applies.

Pre-conditions:

1.	The MSGin5G Client or Application Server has registered to the MSGin5G Server at least once and has indicated that store and forward services are requested in the newest registration.

NOTE:	In addition to UE registration status, the MSGin5G Server can use e.g, UE reachability status monitoring specified in clause 8.9.2 or the recipient's Communication Availability information specified in clause 8.2 to determine whether the recipient is available, i.e. reachable for message delivery.

Figure 8.3.6-1: Store and forward procedure

1.	Inbound MSGin5G message handling, see steps 1-3 in clause 8.3.2. The value of the Store and forward flag IE (see Table 8.3.2-1) in the MSGin5G message indicates that store and forward services are requested by the sender.

2.	The MSGin5G Server checks the registration information of the recipient UE.

a.	If the recipient UE is not registered, the store and forward processing procedure specified from step 3 to step 6 applies.

b.	If the recipient UE is registered, the MSGin5G Server delivers the message as specified in clause 8.3.3:

i)	If the delivery succeeds, the procedure ends; or

ii)	if the delivery failed, e.g. based on the transport layer response etc, the store and forward processing procedure specified from step 3 to step 6 apply.

3.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the delivery had been deferred.

4.	The MSGin5G Server stores the message and tries to deliver it again until the Message expiration time is expired.

NOTE:	The Message expiration time is configured by the MNO and implementation specific.

The MSGin5G Server may deliver the message to the recipient when:

a.	The MSGin5G Server has triggered the Recipient UE successfully as per the MSGin5G device triggering procedure in clause 8.9.3 based on the service policy and the recipient UE has registered again;

Editor's note:	the Pre-condition 2 in clause 8.9.3.2 is needed to be modified according to the procedure changes in this clause, i.e. the triggering procedure can be used when the MSGin5G Client is not registered.

b.	the retry condition of message delivery is fulfilled as per the implementation; or

c.	the recipient UE registered again.

5.

If the the message delivery is still failed until the expiration time is reached, the MSGin5G Server discards the stored message;

6.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the message was discarded.

* * Second Change * *

8.3.2	MSGin5G inbound messages into the MSGin5G Server

Figure 8.3.2-1 shows the procedure for an MSGin5G UE that initiates an MSGin5G message request.

Figure 8.3.2-1: New MSGin5G message request from UE

Figure 8.3.2-2 shows the procedure for an Application Server that initiates an API request specified in clause 9.1.1.1 for sending an MSGin5G message to UE.

Figure 8.3.2-2: Application Server initiates a request for sending an MSGin5G message

Figure 8.3.2-3 shows the procedure for a Legacy 3GPP Message Gateway or a non-3GPP Message Gateway that sends a new MSGin5G message request to the MSGin5G Server on behalf of a Legacy 3GPP UE or Non-3GPP UE.

Figure 8.3.2-3: New MSGin5G message request sending from Message Gateway

The following procedure applies to the above figures 8.3.2-1, 8.3.2-2 and 8.3.2-3 with the exception that step 1 only applies to figure 8.3.2-1.

If the value of the Store and forward flag IE in the MSGin5G message request indicates that store and forward services are requested by the sender, the procedure in 8.3.6 applies instead.

1.	The Application Client in the UE sends a request to the MSGin5G Client for invoking the MSGin5G Client to send a new MSGin5G message to a recipient or to multiple recipients.

Editor's note:	Whether the APIs provided by the MSGin5G Client to the Application Client is to be specified in another clause of the TS is FFS.

2.	As shown in figure 8.3.2-1 or 8.3.2-3, the MSGin5G Client or Message Gateway sends the MSGin5G message request to the MSGin5G Server and includes the IEs as listed in table 8.3.2-1 in the request; or as shown in figure 8.3.2-2, the Application Server sends an API request to the MSGin5G Server for sending an MSGin5G message, the API request includes the IEs as listed in table 8.3.2-1.

Table 8.3.2-1: Request to MSGin5G Server for sending MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Recipient UE Service ID/AS Service ID (see NOTE 1, NOTE 2) | O | The service identity of the receiving MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. This IE is mandatory for Point-to-Point messaging, AS-to-Point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. |
| Group Service ID (see NOTE 1) | O | The service identifier of the target MSGin5G Group. This IE is mandatory for a Group Message and is not present in other message scenarios. |
| Broadcast Area ID (see NOTE 1) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic (see NOTE 1) | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application IDs IE may be included when the message is sent to an Application Server or to an Application Client. MSGin5G Server is unaware of the content. |
| Message ID | M | Unique identifier of this message. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. |
| Payload | O | Payload of the message. MSGin5G Server/Client is unaware of the content. If the request is sent from MSGin5G Client or Message Gateway to the MSGin5G Server, the maximum size of this IE is a configurable value that shall not exceed 2048 octets. |
| Priority type (see NOTE 3) | O | Application priority level requested for this message. The application priority levels include High, Normal and Low. The default Priority type of an MSGin5G message is Normal. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Segmentation set identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages |
| Last segment flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| Store and forward flag | M | An indicator of whether store and forward services are requested for this message. If the value indicates that store and forward services are requested by the sender, the store and forward procedure in clause 8.3.6 applies. |
| Store and forward parameters | O | Parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2. This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. The MSGin5G store and forward procedure is detailed in clause 8.3.6. |
| NOTE 1: Only one of these IEs shall be included to represent the type of message request. The MSGin5G client may construct the related IEs based on the information received from Application Client, e.g. adds the MSGin5G service domain. NOTE 2: When the originator is an Application Server, (i.e. Originating AS Service ID is present), this IE shall be a UE Service ID. NOTE 3: The MSGin5G message with high priority should not be aggregated. The other usages of the priority level of the message is implementation specific and is out of scope of this document. |  |  |

Table 8.3.2-2: Store and forward parameters

| Value | Status | Description |
| --- | --- | --- |
| Message expiration time | O | Indicates message expiration time used for providing store and forward services if the destination is not available for communications, The MSGin5GServer attempts delivery at or before the message expiration time, or when the recipient becomes available. |
| Application specific store and forward information | O | Application specific information about store and forward handling, e.g. a delivery time/date. |

3.	The MSGin5G Server verifies that the sender is authorized to send the message.

4.	The MSGin5G Server may send a Message response to the originating entity if the message is rejected or stored and includes the IEs as listed in table 8.3.2-3 in the response.

Table 8.3.2-3: Message Response

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The identity of the MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the identity of the Application Server that initiated the previous Request. |
| Message ID | M | Identifier of the initiating Request. |
| Delivery Status | O | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |
| Failure Cause | O | The reason for failure |