3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230176

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0184 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Service API Registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the EAS Service API Registration in supported in TR 23.700-98 v18.0.0, the function is transposed to TS 23.558. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 8.4.5, the EAS Service API Registration in TR 23.700-98 v18.0.0 is transposed to TS 23.558. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EdgeAPP and ETSI MEC Alignment is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Change 1 * * * *

8.4.5	EAS Service API Registration

8.4.5.1	General

The EAS service API Registration procedure allows an EAS to publish its exposing Service API information to EES.

The EAS service API registration update procedure allows an EAS  to update the published EAS Service API information on the EES.

The EAS service API Registration procedure shall provide mechanisms for the EAS to publish and update KPIs of its Service APIs when available.

The EAS uses the discover service APIs procedure to discover available EAS Service APIs.

The EAS service API subscription and notification procedure allows an EAS to receive changes in dynamic information of EAS Service APIs from an EES.

The EAS service API subscription and notification procedure allows an EAS to receive changes in availability of EAS Service APIs from an EES.

8.4.5.2	Procedures

The procedures for API publication, API discovery, API invocation, API subscription and notification are as specified in TS 23.222 [16].

* * * End of changes * * * *