3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230179

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0359 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating migration status of MC service user at the partner MC system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 10.6.3.3 contains the following Editor`s Note: Editor’s note: How the partner MC system gets informed that the MC service client no longer receives MC services from that partner MC system is FFS. This CR updates the service authorization for migration procedure to allow the partner MC system to be informed once an MC service user has migrated out and no longer receives MC services from that partner MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the service authorization for migration procedure to allow the partner MC system to update the migration status of a migrated MC service server once it is no longer available at that partner MC system. - Migration service de-authorization notification message is defined to inform the partner MC system that an MC service user is no longer available at that partner MC system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper routing and private call redirection is done since the migration status of migrated user is not updated. Hence, consecutive migration is not correctly supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.3.3, 10.6.3.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.6.3.3	Procedure

The procedure for service authorization for migration to a partner MC system C is shown in figure 10.6.3.3-1.

Pre-conditions

-	The MC service user is migrated to a partner MC system B, i.e., receives MC services from the partner MC system B.

-	The MC service user wishes to migrate to the partner MC system C.

-	The MC service client has been configured with an MC service user profile that contains the necessary parameters needed for connectivity with the partner MC system C.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service authorization to take place in the partner system C.

Figure 10.6.3.3-1	Service authorization for migration to partner MC system

1.	The MC service client requests migration service authorization with the partner MC service server C indicating the selected MC service user profile to be used during migrated MC service. The MC service client provides both the MC service ID provided during user authentication in the partner MC system C, and the MC service ID of the MC service user in the primary MC system A.

NOTE 1:	The migrating MC service client also provides authentication credentials which are specified in 3GPP TS 33.180 [25].

2.	The partner MC service server C performs an initial authorization check to verify that the MC service user is permitted to migrate to the partner MC system C from the primary MC system A.

NOTE 2:	The criteria for the initial authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request migrated service authorization.

3.	The partner MC service server C identifies the primary MC system A of the MC service user of the MC service client by use of the MC service ID of the MC service user in the primary MC system, areA which was presented by the MC service client in step 1, and sends a migration service authorization request to the gateway server in the partner MC system C.

4.	The partner MC system C gateway server identifies the primary MC system A of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to the gateway server of the primary MC system A.

5.	The gateway server in the primary MC system A identifies the primary MC service server of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to that MC service server A.

6.	The primary MC service server A of the MC service user performs an authorization check, to verify that migration is permitted to that partner MC system C by this MC service user using the indicated MC service user profile.

7.	The primary MC service server A marks the MC service user as having migrated, and records the partner MC system C as the migrated MC system.

8.	The primary MC service server A sends a migration service authorization response to the gateway server in the primary MC system A.

9.	The gateway server in the primary MC system A sends the migration service authorization response to the gateway server in the partner MC system.

10.	The gateway server in the partner MC system C sends the migration service authorization response to the partner MC service server C.

11.	The partner MC service server C stores the necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system A mapped to the MC service ID of the migrated MC service user provided by partner MC system C).

12.	The partner MC service server C sends the migration service authorization response to the MC service client, confirming that successful migration and service authorization has taken place.

NOTE 3:	If topology hiding is not used, the migration service authorization request and migration service authorization response messages are sent between the MC service servers in the primary and partner MC system without the need to be sent via an MC gateway server.

13.	As the MC service user has been migrated at the partner MC system B, the primary MC service server A sends a migration service de-authorization notification to the gateway server of the primary MC service server A.

14. The gateway server in the primary MC service server A sends the migration service de-authorization notification to the gateway server of the partner MC system B.

15. The gateway server in the partner MC system B sends the migration service de-authorization notification to the partner MC service server B. Hence, the latter is informed that the MC service user has migrated out and receives MC services from a different MC system.

NOTE 4:	If topology hiding is not used, the migration service de-authorization notification message is sent between the MC service servers in the primary and partner MC system without the need to be sent via an MC gateway server.

16. The MC system B updates the MC service user’s migration status accordingly and keeps the mapping of its assigned MC service ID by partner MC system B with the MC service user`s MC service ID assigned by its primary MC system A to allow proper redirection of services via the primary MC service server A, as described in clause 10.16.5.

NOTE 5:	The period of how long MC system B will maintain the mapping information is left for implementation.

# * * * * Second change * * * *

##### 10.6.3.2.3	Migration service de-authorization notification

Table 10.6.3.2.3-1 describes the information flow migration service de-authorization notification sent from the primary MC service server to the partner MC service server, which its MC service user has migrated out from that partner MC system. Hence, the partner MC system is informed once the MC service user has migrated out. This information flow is sent individually addressed on unicast or multicast.

Table 10.6.3.2.3-1: Migration service de-authorization notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating MC service user provided by the partner MC system. |
| MC service ID | M | The MC service ID of the migrating MC service user in the primary MC system of the MC service user. |

# * * * * End of changes * * * *