3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230209

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-78 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for activation and deactivation of PIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_PINAPP |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently as part of PIN management, there are procedures for creation and deletion of the PIN. But there is no procedure to activate and deactivate the PIN. When a PIN is being created, there need to exist a PIN configuration (PIN profile) at the PIN server which contains the information about the PIN. When the PIN is deleted the related configuration will be deleted. But in some cases, the PIN might need to be active for a particular duration each day and after that it becomes deactive. For e.g., the PIN owner/PIN admin might configure that PIN needs to be active for particular duration each day e.g., from 10 am to 5 PM. After 5 PM the PIN needs to be deactivated/suspended instead of deletion. When the PIN is deactivated the PIN configuration(PIN profile) should not be deleted and only the PIN dynamic profile information will be deleted. pCR S6-222750 was introduced during SA6#51e and it is decided to send a LS to get more clarification from SA1 on the requirements related to valdidity duration of the PIN and whether the PIN owner or admin can temporarily suspend the PIN. SA1 in their reply LS (S6-230014) stated that it is left to the discretion of SA6. The requirement is not intended to include or exclude any possible technical realization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds a procedure for PIN activation and deactivion as part of PIN management procedures. Considering the reply from SA1 it is better if we have these procedures related to PIN activation and deactivation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Need to include this as part of study and handle the same during the normative phase. If not we will not be able to have the flexibility of handling the PIN duration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (all new text)* * * *

#### 7.3.2.5	Procedures of PIN activation and deactivation

The procedures in this clause describes how a PEMC can activate and deactivate the PIN. PEMC being an authroized entity to manage the PIN can request to activate and deactivate the PIN. When the PIN is in deactivated state services offered by the PIN are inaccessible and no PIN elements can join the PIN.

##### 7.3.2.5.1	PIN activation by PEMC

Pre-conditions:

1.	PEMC(PINE-3), PEGC, PINE-1, PINE-2 all are part of same PIN

2.	The PIN being activated has already been created earlier.

3.	The PINE-3 is capable of acting as PEMC

4.	The PEMC (PINE-3) in a PIN has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

Figure 7.3.2.5.1-1: Activation of PIN by PEMC

1.	PEMC (PINE-3), PEGC, PINE-1 and PINE-2 all are part of same PIN, which is created earlier and is now in deactivated state.

2.	PEMC requests to the PIN server to register itself as PEMC as described in the clause 7.13.2.2.

3.	PEMC requests the PIN server to activate the PIN and this request contains the PIN ID, PIN Element ID/PINAPP ID of PINE-3, duration of the PIN to be in active state etc.,

4.	PIN server activates the PIN and sends the response to PEMC.

5.	PIN server and PEMC creates the dynamic PIN information with the details such as PIN ID, PIN validity duration.

6.	Other PIN elements including PEGC joins the PIN as described in clause 7.4.2.2.1.

* * * Next Change (all new text)* * * *

##### 7.3.2.5.2	PIN deactivation by PEMC

Pre-conditions:

1.	The PINE-3 is authorized as PEMC

2.	PEMC (PINE-3) decides to deactivate the PIN

Figure 7.3.2.5.2-1: Deactivation of PIN by PEMC

1.	PEMC (PINE-3), PEGC, PINE-1, PINE-2 all are part of same PIN, which is created earlier and is now in activated state.

2.	PEMC (PINE-3) decides to deactivate the PIN and sends the request to deactivate the PIN to PIN server. Decision reason by PEMC to deactivate the PIN could be PIN validity duration is expiring or all the PIN elements have left the PIN or for any other reasons which could be implementation specific. This request carries the PIN identifier of the PIN to be deactivated, PIN Element identifier of the PEMC and the reason for de-activating the PIN.

3.	PIN server checks whether PINE-3 is authorized to deactivate the PIN by checking the PIN profile.

4.	PIN server sends the deactivaton response to the PEMC (PINE-3) for the request to deactivate the PIN.

5a-5c.	PEMC (PINE-3) on receiving the PIN deactivation response from the PIN server notifies all the PIN elements which are currently attached/joined/registered to the PIN that PIN is being deactivated.

6.	PIN server, PEMC and PEGC cleansup the dynamic PIN profile information of the PIN being deactivated.

* * * End Change * * * *