3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230219

e-meeting, 11th – 20th January 2023	(revision of S6-23abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0332 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows for the ad hoc group call procedures involving multiple MCPTT systems - Participants list determined by the MCPTT servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the Ad hoc group communication study, the TR 23.700-76 V1.3.0 has defined the procedures for the adhoc group call procedure involving multiple MCPTT system. The conclusion in the clause 9 for the Key issue #1 indicates that the procedures in the clause 7.1.3.2 of solution 1 are agreed. There is a companion CR, trying to provide the procedures for MCPTT server to modify the ad hoc group call participant’s procedure involving multiple MCPTT systems, procedure to notify the partner system to stop determining the participants due to ad hoc group call termination. This CR is trying to propose the information elements which are used newly in the proposed procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new information flows Ad hoc group call add user notification, Ad hoc group call remove user notification and Ad hoc group call release notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Detailed information about the content of newly defined information elements which are used in the procedures will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (10.19.2.K, 10.19.2.G and 10.19.2.A) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.19.2.K	Ad hoc group call add user notification (MCPTT server – MCPTT server)

Table 10.19.2.K -1 describes the information flow Ad hoc group call add user notification from one MC service server to another MC service server. This notification is to provide the list of MC service IDs meeting the criteria specified in the ad hoc group communication get userlist request from one MC service server to another MC service server.

Table 10.19.2.K-1 Ad hoc group call add user notification information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ad hoc group ID | M | The MC service group ID associated with the ad hoc group communication |
| MC service ID list | M | List of MC service IDs meeting the criteria specified in the ad hoc group communication get userlist |

#### 10.19.2.G	Ad hoc group call remove user notification (MCPTT server – MCPTT server)

Table 10.19.2.G -1 describes the information flow Ad hoc group call remove user notification from one MC service server to another MC service server. This notification is to provide the list of MC service IDs no longer meeting the criteria specified in the ad hoc group communication get userlist request from one MC service server to another MC service server.

Table 10.19.2.G-1 Ad hoc group call remove user notification information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ad hoc group ID | M | The MC service group ID associated with the ad hoc group communication |
| MC service ID list | M | List of MC service IDs no longer meeting the criteria specified in the ad hoc group communication get userlist |

#### 10.19.2.A	Ad hoc group call release notification (MCPTT server – MCPTT server)

Table 10.19.2.A -1 describes the information flow Ad hoc group call release notification from one MC service server to another MC service server. This notification is to indicate to stop evaluating the criteria to determine the participants list if another MC service server is having the criteria specified in the ad hoc group communication get userlist request.

Table 10.19.2.A-1 Ad hoc group call release notification information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ad hoc group ID | M | The MC service group ID associated with the ad hoc group communication |

* * * End of Changes * * * *