3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230221

e-meeting, 11th – 20th January 2023	(revision of S6-23abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0333 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ad hoc group call procedures involving multiple MCPTT systems - Participants list determined by the MCPTT servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the Ad hoc group communication study, the TR 23.700-76 V1.3.0 has defined the procedures for the adhoc group call procedure involving multiple MCPTT system. The conclusion in the clause 9 for the Key issue #1 indicates that the procedures in the clause 7.1.3.2 of solution 1 are agreed.This CR is trying to propose the corresponding normative text in the normative work phase. In addition, the CR is trying to provide the procedures for MCPTT server to modify the ad hoc group call participant’s procedure involving multiple MCPTT systems, procedure to notify the partner system to stop determining the participants due to ad hoc group call termination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new clauses to provide the procedures for the adhoc group call. Added the procedures as defined in the clause 7.1.3.2 of TR 23.700-76 V1.3.0 into new clause 10.19.3.2.K of TS 23.379 as a normatie text. Added new procedures for MCPTT server to modify the ad hoc group call participant’s procedure involving multiple MCPTT systems. Added new procedures for MCPTT server to stop determining the ad hoc group call participant’s involving multiple MCPTT systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group call procedures will be missing in multiple MCPTT systems where participants list determined by the MCPTT servers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (10.19.3.2, 10.19.3.2.K, 10.19.3.2.G, and 10.19.3.2.B) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.19.3.2	Ad hoc group call procedures involving multiple MCPTT systems

##### 10.19.3.2.K	Ad hoc group call setup – Participants list determined by the MCPTT server

Figure 10.19.3.2.K-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user wherein the list of participants is determined by the MCPTT server based on the citeria received from the MCPTT client and determined MCPTT users are from multiple MCPTT systems.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initate ad hoc group call.

2.	End-to-End encryption is supported for this ad hoc group call.

Figure 10.19.3.2.K-1: Ad hoc group call setup involving multiple MCPTT systems

1-4.	Same as described in subclause 10.19.3.1.3.

5.	MCPTT server 1 if it needs to involve the partner system based on the agreement and based on the criteria for determining the participants list, sends the ad hoc group call get userlist request to the MCPTT server 2. This request carries the criteria specified in the step 1.

6.	MCPTT server 2 evaluates the criteria and determines the particpants satisfying the criteria and sends the response containing the list of MCPTT users satisfying the criteria

7.	The MCPTT server 1 sends the ad hoc group call request towards the MCPTT server 2. This request carries the list of MCPTT users who needs to be invited for this ad hoc group call and are registered with MCPTT server 2. It also carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested.

8a-8b.	The MCPTT server 2 sends the ad hoc group call request towards the MCPTT client 3 and MCPTT client 4.

9.	The MCPTT server 1 sends the ad hoc group call requests towards the MCPTT client 2. While sending the ad hoc group call request, the MCPTT server shall remove the information elements that are not required to be conveyed to the target MCPTT clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. The MCPTT server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

10a-10c.	The receiving MCPTT clients are notified about the incoming ad hoc group call.

11.	The MCPTT client 2 accept the ad hoc group call request and send ad hoc group call response to the MCPTT server 1.

12a-12b.	The MCPTT client 3 and MCPTT client 4 accepts the ad hoc group call request, and sends ad hoc group call responses to the MCPTT server 2.

13a-13b.	The MCPTT server 2 forwards the responses to the MCPTT server 1.

14.	The MCPTT server 1 sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

15.	The MCPTT server 1 may notify the initiating MCPTT user of all MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call. The MCPTT server 1 more than once during the call may send this notification to the initiating MCPTT user whenenver an MCPTT user joins or leaves the MCPTT ad hoc group call.

16.	The MCPTT client 1, MCPTT client 2, MCPTT client 3 and MCPTT client 4 establish media plane and floor control resources.

* * * Next Change * * * *

##### 10.19.3.2.G	MCPTT server modifying ad hoc group call participants procedure involving multiple MCPTT systems

Figure 10.19.3.2.G-1 below illustrates the MCPTT server modifying the ad hoc group call participants procedure involving multiple MCPTT systems.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initate ad hoc group call.

2.	The primary and partner MCPTT servers determined the participants for the ad hoc group call based on the criteria specified by the MCPTT client 1 while initiating the ad hoc group call.

3.	The primary and partner MCPTT servers continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

4.	The primary MCPTT server detects that the MCPTT client 3 satisfies the criteria and MCPTT client 2 stops to meet the criteria specified by the MCPTT client 1.

5.	The partner MCPTT server detects that the MCPTT client 5 satisfies the criteria and MCPTT client 4 stops to meet the criteria specified by the MCPTT client 1.

Figure 10.19.3.2.G-1: MCPTT server modifying ad hoc group call participants involving multiple MCPTT systems

1.	The ad hoc group call is established and on-going with the participants MCPTT client 1, MCPTT client 2, and MCPTT client 4. The participants list is determined by both primary and partner systems MCPTT server based on the criteria specified by the MCPTT client 1 while initiating the call.

2.	The primary MCPTT server detects that MCPTT client 3 now meets criteria and successfully added to call.

3.	The primary MCPTT server detects that MCPTT client 2 does not meet criteria any more and successfully removed from a call.

4.	The partner MCPTT server detects that the MCPTT client 5 satisfies the criteria specified by the MCPTT client.

5.	The partner MCPTT server sends the notification to primary MCPTT system to add the MCPTT user at MCPTT client 5 to on-going ad hoc group call.

6.	The primary MCPTT server sends the ad hoc group call request to the MCPTT client 5.

7.	The MCPTT client 5 notifies the user about the incoming ad hoc group call.

8.	The MCPTT client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the primary MCPTT server.

9.	The on-going ad hoc group call is updated by adding MCPTT client 5 which satisfies the criteria specified by the MCPTT client 1.

10.	The partner MCPTT server detects that the MCPTT client 4 is no more satisfying the criteria to be the participant of the ad hoc group call.

11.	The partner MCPTT server sends the notification to primary MCPTT system to remove the MCPTT user at MCPTT client 4 from on-going ad hoc group call.

12.	The primary MCPTT server sends the ad hoc group call leave request to the MCPTT client 4 and removes it from the on-going ad hoc group call.

13.	The MCPTT client 4 notifies the user of the ad hoc group call leave request.

14.	The MCPTT client 4 sends the ad hoc group call leave response to the primary MCPTT server.

15.	The on-going ad hoc group call is updated by removing MCPTT client 4, which no more satisfies the criteria specified by the MCPTT client 1.

* * * Next Change * * * *

##### 10.19.3.2.B	Stop determining the ad hoc group call participants procedure involving multiple MCPTT systems

Figure 10.19.3.2.B-1 below illustrates the stoping of MCPTT server determining the ad hoc group call participants procedure involving multiple MCPTT systems.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initate ad hoc group call.

2.	The primary and partner MCPTT servers determined the participants for the ad hoc group call based on the criteria specified by the MCPTT client 1 while initiating the ad hoc group call.

3.	The primary and partner MCPTT servers continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

Figure 10.19.3.2.B-1: MCPTT server stopping the determination of ad hoc group call participants involving multiple MCPTT systems

1.	The ad hoc group call is established and on-going with the participants MCPTT client 1, MCPTT client 2, MCPTT client 3, and MCPTT client 4. The participants list is determined by both primary and partner systems MCPTT server based on the criteria specified by the MCPTT client 1 while initiating the call.

2.	The primary MCPTT server detects that the ad hoc group call which is ongoing is to be released e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MC service group members are not present.

3a-3d.	The primary MCPTT server sends ad hoc group call release request to all the participants of the ad hoc group call.

4a-4d.	The MCPTT clients notifies the user about the release of the ad hoc group call.

5.	The primary MCPTT server sends the notification to partner MCPTT system about the ad hoc group call release to stop determing the participants list by partner MCPTT system.

6a-6d.	All the participants of the ad hoc group call recieves the ad hoc group call release request and sends the ad hoc group call release response to the primary MCPTT server.

* * * End of Changes * * * *