3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230225

e-meeting, 11th – 20th January 2023	(revision of S6-23abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0363 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Migration service de-authorization procedure initiated by MC service server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During #52 meeting, CR S6-223510 was agreed which was proposed to overcome the situation in which a private communication request may fail when the MC service ID assigned by a partner MC system is used by an MC service user in that MC system and the targeted MC service user has already left the MC system towards another MC system. The proposed solution is to store the necessary information for communication redirection. This information is stored during service authorization for migration to partner MC system and the call will be routed accordingly. And an EN is introduced Editor’s note: How the partner MC system gets informed that the MC service client no longer receives MC services from that partner MC system is FFS. This CR is trying to propose the solution for above Editor note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new information elements (Migration service de-authorization notification and MC service authorisation notification) and procedure (Migration service de-authorization procedure initiated by MC service server) are introduced. The newly introduced procedure to help the partner MC service server from which the user is migrating out, to update the stored necessary information related to the migrated MC service user (e.g., migration status of the MC service user as migrated MC service user is no longer migrated on this partner MC system) and notify the MC service authorisation completion to the primary MC system of the migrated MC service user at the partner MC system to which user is migrating into it. In 10.6.3.3.1: The step 7 and 11 is updated to include the storing of migration status of the MC service user along with the existing migration information. The stored necessary information related to the migrated MC service user is used for proper communication redirection only after successful completion of MC service authorisation at partner MC system (i.e. migrated MC system). Removed the Editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Communication redirection back to the primary MC system is not possible once the migrated MC service user is no longer migrated on this partner MC system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.3.2, 10.6.3.2.Y(new), 10.6.3.2.X(new), 10.6.3.3, 10.6.3.3.1, 10.6.3.3.2(new), 10.6.3.3.2.K(new), and 10.6.3.3.2.G(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.6.3.2	Information flows

* * * Next Change * * * *

##### 10.6.3.2.Y	Migration service de-authorization notification

Table 10.6.3.2.Y-1 describes the information flow migration service de-authorization notification sent by a MC service server of Primary MC system of migrated MC service user to a migrated partner MC system from which MC service user is migrating out.

Table 10.6.3.2.Y-1: Migration service de-authorization notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID (see NOTE 1) | M | The MC service ID of the migrated MC service user provided by the partner MC system. |
| MC service ID | M | The MC service ID of the migrated MC service user in the primary MC system of the MC service user. |
| NOTE 1: The MC service ID is provided by the identity management server in the partner MC system during authentication of the migrated MC service user. |  |  |

##### 10.6.3.2.X	MC service authorisation notification

Table 10.6.3.2.X-1 describes the information flow about the notification for successful completion of MC service authorisation after migrating to the partner MC system.

Table 10.6.3.2.X-1: MC service authorisation notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the migrating MC service user provided by the partner MC system. |
| Service authorisation Indication | M | Indication to provide the successful completion of MC service authorisation after migrating to the partner MC system |

* * * Next Change * * * *

#### 10.6.3.3	Procedures

##### 10.6.3.3.1	Migration service authorization procedure

The procedure for service authorization for migration to a partner MC system is shown in figure 10.6.3.3.1-1.

Pre-conditions

-	The MC service user wishes to migrate to the partner MC system.

-	The MC service client has been configured with an MC service user profile that contains the necessary parameters needed for connectivity with the partner MC system.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service authorization to take place in the partner MC system.

Figure 10.6.3.3.1-1	Service authorization for migration to partner MC system

1.	The MC service client requests migration service authorization with the partner MC service server indicating the selected MC service user profile to be used during migrated MC service. The MC service client provides both the MC service ID provided during user authentication in the partner MC system, and the MC service ID of the MC service user in the primary MC system of the MC service user.

NOTE 1:	The migrating MC service client also provides authentication credentials which are specified in 3GPP TS 33.180 [25].

2.	The partner MC service server performs an initial authorization check to verify that the MC service user is permitted to migrate to the partner MC system from the primary MC system of the MC service user.

NOTE 2:	The criteria for the initial authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request migrated service authorization.

3.	The partner MC service server identifies the primary MC system of the MC service user of the MC service client by use of the MC service ID of the MC service user in the primary MC system of the MC service user, which was presented by the MC service client in step 1, and sends a migration service authorization request to the gateway server in the partner MC system.

4.	The partner MC system gateway server identifies the primary MC system of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to the gateway server of the primary MC system.

5.	The gateway server in the primary MC system of the MC service user identifies the primary MC service server of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to that MC service server.

6.	The primary MC service server of the MC service user performs an authorization check, to verify that migration is permitted to that partner MC system by this MC service user using the indicated MC service user profile.

7.	The primary MC service server marks the MC service user as having migrated, and records the partner MC system as the migrated MC system.

NOTE 3:	The stored necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system and the migration status information of the MC service user) is used for proper communication redirection only after successful completion of MC service authorisation at partner MC system (i.e. migrated MC system).

8.	The primary MC service server sends a migration service authorization response to the gateway server in the primary MC system.

9.	The gateway server in the primary MC system sends the migration service authorization response to the gateway server in the partner MC system.

10.	The gateway server in the partner MC system sends the migration service authorization response to the partner MC service server.

11.	The partner MC service server stores the necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system and the migration status of the MC service user). The migration status of the MC service user allows proper communication redirection back to the primary MC system once the migrated MC service user is no longer migrated on this partner MC system.

NOTE 4:	The stored necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system and the migration status information of the MC service user) is used for proper communication redirection only after successful completion of MC service authorisation at partner MC system (i.e. migrated MC system).

12.	The partner MC service server sends the migration service authorization response to the MC service client, confirming that successful migration and service authorization has taken place.

NOTE 5:	If topology hiding is not used, the migration service authorization request and migration service authorization response messages are sent between the MC service servers in the primary and partner MC system without the need to be sent via an MC gateway server.

* * * Next Change * * * *

##### 10.6.3.3.2	Migration service de-authorization procedure initiated by MC service server

###### 10.6.3.3.2.K	Migrated MC service user return back to its primary MC system

The procedure for service de-authorization of migration from a partner MC system is shown in figure 10.6.3.3.2.K-1.

Pre-conditions

-	The MC service user migrated to the partner MC system.

-	The MC service user wishes to migrate back to primary MC system.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service de-authorization to take place in the partner MC system.

-	The MC service user has successfully completed the MC service authorisation at primary MC system to which the MC service user returning back from migrated MC system.

Figure 10.6.3.3.2.K-1	Service de-authorization of migration from partner MC system

1.	The primary MC service server sends an Migration service de-authorization notification to the primary MC system gateway server.

2. The primary MC system gateway server sends the Migration service de-authorization notification to the partner MC system gateway server.

3. The partner MC system gateway server sends the Migration service de-authorization notification to the partner MC service server.

4. The partner MC service server updates the stored necessary information related to the migrated MC service user e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system and the migration status information of the MC service user) of the MC service user accordingly.

###### 10.6.3.3.2.G	Migrated MC service user migrate to another partner MC system

The procedure for service de-authorization of migration from a partner MC system is shown in figure 10.6.3.3.2.G-1.

Pre-conditions

-	The MC service user migrated to the partner MC system B.

-	The MC service user wishes to migrate back to another partner MC system C.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service de-authorization to take place in the partner MC system B.

Figure 10.6.3.3.2-1	Service de-authorization of migration from partner MC system

1.	The MC service client completes the service authorization for migration to a partner MC system C as defined in the clause 10.6.3.3.1.

2.	The MC service client performs an MC service authoriation at partner MC system C and succesfuly completes it.

3.	The partner MC service server of partner MC system C notifys the primary MC system of the MC service user of the MC service client by sending the MC service authorisation notification

4.	The primary MC service server of primary MC system A, sends an Migration service de-authorization notification to the primary MC system A gateway server.

5. The primary MC system gateway server of of primary MC system A, sends the Migration service de-authorization notification to the partner MC system gateway server of partner MC system B.

6. The partner MC system gateway server of partner MC system B sends the Migration service de-authorization notification to the partner MC service server of partner MC system B.

7. The partner MC service server of partner MC system B updates or clears the stored necessary information related to the migrated MC service user e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system and the migration status information of the MC service user) of the MC service user accordingly.

* * * End of Changes * * * *