3GPP TSG-SA WG6 Meeting #52	S6-230228

Toulouse, France 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.434 | CR | 0161 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL server provisioning for Location Management Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 9.3.3.2 – location management client fetches the location reporting configuration. Location management server needs to provide default triggers if the specific triggers are not provided by VAL server for the VAL user. Further, as specfieid in clause 9.3.13.1, the VAL service areas are configured by the VAL server to the location management server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a procedure and information flows for VAL server to provision requried configuration to the SEAL LMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL LMS will not be aware of the default triggers and also VAL service areas. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.xx (NEW), 9.3.2.yy (NEW), 9.3.zz(NEW), 9.3.zz.1 (NEW), 9.3.zz.2 (NEW), 9.4.1, 9.4.a (NEW), 9.4.a.1 (NEW), 9.4.a.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.xx	VAL server provisioning request

Table 9.3.2.xx-1 describes the information flow from the VAL server to the location management server for providing provisioning configuration.

Table 9.3.2.xx-1: VAL server provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Endpoint information | O | Information of the endpoint of the VAL server. |
| List of VAL service specific information | M | Provides the list of VAL service specific information |
| > VAL service ID | M | Identify of the VAL service for which the configuration information is provided. |
| > default triggering criteria | O | Identifies the default triggers when the VAL server is expecting to receive the location report from the requested VAL user/ VAL UE. |
| > List of VAL Service Areas | O | List of VAL Service Areas identified distinctly through geographical area |
| >> VAL Service Area ID | O | VAL Service area Identity |
| >> Geographical area | O | Identifies the geographical information of the user(s) |

#### 9.3.2.yy	VAL server provisioning response

Table 9.3.2.yy-1 describes the information flow from the location management server to the VAL server as a response for providing provisioning configuration.

Table 9.3.2.yy-1: VAL server provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * Next Change * * * *

### 9.3.zz	VAL server provisioning for location management service

#### 9.3.zz.1	General

The high level procedure for VAL server to provision required information to SEAL location management server is described in the following subclause.

#### 9.3.zz.2	Procedure

The procedure for VAL server to provision required information to SEAL location management server is illustrated in figure 9.3.zz.2-1.

Figure 9.3.zz.2-1: VAL Server provisioning to SEAL Location Management Server

1.	The VAL server sends a request message to location management server to provision required information. The request message includes identity of the VAL server, endpoint information of the VAL server, security credentials of the VAL server, and service provider specific information like default location reporting trigger and list of VAL service areas.

2.	Upon receiving the request, the location management server authorizes the request based on the security credentials provided in the request and considering the service level agreement between VAL service provider and SEAL service provider. If VAL server is authorized to use the SEAL service, then the location management server stores the details about the VAL server including the default location reporting trigger and list of VAL service areas. The location management server sends the response message to the VAL server.

Editor’s note:	Whether the VAL server registers with location Management Server is FFS.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_VALServerProvisioning | Provide_Configuration | VAL server | Request /Response |

* * * Next Change * * * *

### 9.4.a	SS_VALServerProvisioning API

#### 9.4.a.1	General

API description: This API enables the VAL server to provision configuration for the VAL service to the SEAL LM-S.

#### 9.4.a.2	Provide_Configuration operation

API operation name: Provide_Configuration

Description: Provisioning of VAL service configuration to LM-S.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.xx

Outputs: See subclause 9.3.2.yy

See subclause 9.3.zz.2 for the details of usage of this API operation.

* * * End of Change * * * *