3GPP TSG-SA WG6 Meeting #52 bis	S6-220265

e-meeting, 11th – 20th January 2023	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 217 | rev | - | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CCr procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023/01/10> |
| Category: | B |  |  |  |  |  | Release: |  |  | <Rel-18> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service provisioning triggering via SMS over NAS is not supported in Rel18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#17, sol 27, 30 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## * * * First Change * * * *

## a.b  CCR APIs

### a.b.1	General

CCR provides services for storage and query of CEAG information. These srevices are exposed to EESs via EDGE-19 and to ECSs via EDGE-20.

EDGE-19 APIs to EESs include requests for CEAG information store operations, with the the CCR providing coresponding responses. Additionally, EESs can request CEAG information queries, with the CCR providing corresponding responses.

EDGE-19 APIs to EESs include requests for CEAG information queries, with the the CCR providing corresponding responses.

### a.b.2	Procedures

#### a.b.2.1	General

The procedures supporting the APIs exposed by CCR are:

- The CEAG information store procedure

- The CEAG information query procedure

#### a.b.2.2 CEAG information store procedure

The CEAG information store procedure allows a requestor EES to store CEAG information in CCR.

Pre-conditions:

- The requestor EES is pre-configured with addressing and security information for communicating with a CCR.

Figure a.b.2.2-1: CEAG information store procedure

- The requestor sends a CEAG information store request to the receiving CCR. The request message includes one or more CEAG ID(s) associated with the requestor EES, i.e. with the requestor’s EES ID.

- Upon receipt of the CEAG information store request, the CCR authorizes the request and stores the list of tuples (EES ID, CEAG ID) formed with the requestor’s EES ID and the CEAG IDs in the request.

- The CCR sends a CEAG information store response to the requestor which contains a status of the sore operation.

#### a.b.2.3	CEAG information query procedure

The CEAG information query procedure allows a requestor ECS or EES to query CEAG information stored in CCR.

Pre-conditions:

- The requestor EES or ECS is pre-configured with addressing and security information for communicating with a CCR.

Figure a.b.2.3-1: CEAG information query procedure

- The requestor sends a CEAG information query request to the receiving CCR. The request message includes one or more CEAG ID(s) for which EES information is queried

- Upon receipt of the CEAG information query request, the CCR  authorizes the request and retrieves the list of EESs which are associated with the CEAG IDs in the request.

- The CCR sends a CEAG information query response to the requestor which contains, for each requested CEAG ID, a list of associated EESs.

### a.b.3 Information flows

#### a.b.3.1	General

The following information flows are specified for the CCR APIs:

- CEAG information store request/ response; and

- CEAG information query request/response.

#### a.b.3.2	CEAG information store request

Table a.b.3.2-1 describes the information elements for the CEAG information store request.

Table a.b.3.2-1: CEAG information store request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The EES ID of the requestor. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| CEAG ID list | M | List of CEAG IDs receiving services from an EAS registered to the requestor EES. |

#### a.b.3.3	CEAG information store response

Table a.b.3.3-1 describes the information elements for the CEAG information store response.

Table a.b.3.3-1: CEAG information store response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Failure response | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |

#### a.b.3.4	CEAG information query request

Table a.b.3.4-1 describes the information elements for the CEAG information query request.

Table a.b.3.4-1: CEAG information query request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The EES ID or ECS ID of the requestor. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| CEAG ID list | M | List of CEAG IDs being queried. |

#### a.b.3.5	CEAG information query response

Table a.b.3.5-1 describes the information elements for the CEAG information query response.

Table a.b.3.5-1: CEAG information query response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > CEAG information list | M | List of CEAG information returned. Each list element is a (EES ID, CEAG ID) tuple. |
| Failure response | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |