3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230278

e-meeting, 11th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0221 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deployment options for EDGEAPP and ETSI MEC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Technology India Pvt Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An operator may want to deploy both EDGEAPP and ETSI MEC architectures in its environment or one of the architectures might be present already and it is desirable to support the another architecture as well in the network. This CR captures such deployment possiblities for an operator to use both the architectures in its environment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides different deployment and evolution options for EDGEAPP and ETSI MEC architectures in an operator environment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This will be limiting for for the operator because the two architectures will not be able to coexist in single environment. It will create non-interoperable scenario any may cause business loss to the operator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | None |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

# C.2	Relationship between EDGEAPP and ETSI MEC architectures

Figure C.2-1 provides the relationship of ETSI ISG MEC architecture with EDGEAPP architecture.

Figure C.2-1: Relationship between EDGEAPP and ETSI MEC architectures

Editor's note: Whether and how to enhance EDGE-9 or Mp3 is FFS.

# * * * Change 2 * * * *

# C.3	Deployment options for EDGEAPP and ETSI MEC

## C.3.1	Deployment Option-1: Collocated Platforms

Based on Figure C.2-1, the two platforms (EES and MEC Platform) are co-located are deployed as two different Application Functions (AFs) on a single Physical or NFV infrastructure. However, the actual deployment details of two platforms are implementation specific (Figure C.3.1-1).

Figure C.3.1-1: EES and MEC Platform as two different AFs

## C.3.2	Deployment Option-2: Converged architecture

### C.3.2.1	General

The two platforms i.e. EES and MEC Platform (MEP) are co-located and implemented as a single AF (e.g. realized as one VNF) compliant with both standards.

The converged architecture as in Figure C.3.2.1-1 is expected to satisfy the following requirement:

-	The collocated platforms (MEP+EES) is able to satisfy all the functionalities of MEP defined in ETSI and EES defined in SA6.

-	A uniform API is defined for the EAS and MEC App, i.e. EDGE-3 and Mp1 are unified into one interface and the EAS and MEC App will consume the same service from the MEP+EES.

-	EDGE-9 and Mp3 are unified into one interface.

NOTE: Management of MEP+EES is under the scope of SA5.

Figure C.3.2.1-1 Converged architecture for EDGEAPP and ETSI MEC alignment

### C.3.2.2	Evolution Options

#### C.3.2.2.1	General

In this clause the converged architecture is the architecture described in clause C.3.2.1 and satisfies the requirements listed in that clause.

#### C.3.2.2.2	Evolution Option #1- Enhancement of a deployed MEP to support the functionality of EES

If an operator has already deployed ETSI MEC architecture, the MEP needs to be upgraded to the MEP+EES for supporting the functionality of EES defined in 3GPP SA6. The deployment and evolution scenario are depicted in Figure C.3.2.2.2-1. In this scenario, both EDN 1 and EDN 2 are deployed by the same operator.

Figure C.3.2.2.2-1 MEP enhancement to support EES functionality

#### C.3.2.2.3	Evolution Option #2 Enhancement of a deployed EES to support the functionality of MEP

An operator may opt to upgrade/evolve the EDGEAPP architecture to the converged architecture if the operator deployed EDGEAPP architecture in its EDNs to provide edge services. The deployment and evolution scenario are depicted in Figure C.3.2.2.3-1. In this scenario, both EDN 1 and EDN 2 are deployed by the same operator.

Figure C.3.2.2.3-1 EES enhancement to support MEP functionality

## C.3.3	Deployment Option-3: non-Collocated Platforms

The two platforms (EES and MEC Platoform) are non-collocated, and reside in two different data networks, where EES is in the Mobile Network Operator (MNO) domain while the ETSI MEC platform is in another MNO domain (Figure C.3.3-1).

Figure C.3.3-1: EES and MEC Platform as two different AFs in two different EDNs

# * * * End of changes * * * *