3GPP TSG-SA WG6 Meeting #52 bis	S6-230281

e-meeting, 11th – 20th January 2023	(revision of S6-230266)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 223 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS provisioning merge |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To provide support for Common EAS in service provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note that the initial version includes comments from offline discussions to help further them. They are planned to be deleted when no longer useful. A CEAG Profile is introduced, containing information necessary for the feature to be provided. The CEAG Profile proposed may have 2 types: dynamic and pre-provisioned, each with accompanying filter criteria. The IE has been structured as such to show the parallelism between different proposals. However, removing the type and keeping the IEs from Dynamic CEAG filter criteria only would be sufficient, given that it is a superset of those in Pre-grouped CEAG filter criteria, and that the differentiating IDs are all optional. In other words, the pre-grouped type is equivalent with the dynamic type where only the Group ID is provided. The Group ID in the filter criteria has been proposed to be different than CEAG ID because the latter needs to be unique within an EdgeApp deployment, while the Group ID is assumed to be assigned by an external server. The AC ID list and the AC type in the filter criteria provide the means to support multi-AC ID/type grouping, which is a usecase discussed in the TR and which several TR solutions support. The EAS list, with its KPIs, is provided for the cases in which the EAS KPIs necessary to support Dynamic CEAG filter criteria a group are different than the EAS KPIs neccseary to support single ACs, even for the same service. The list is optional, so if the same KPIs apply this IE can be omitted. In addition to CEG Profile, the following changes to exiting text are proposed: - EES Profile is enhanced with a list of CEAG ID, representing the CEAGs the EES is supporting. This list is formed/updated by EES as EECs discover its EASs and choose one as Common EAS. Therefore, this list is not pre-provisioned at the EES, and it needs to be updated only when the first AC in a CEG starts services and when the last AC in a CEG stops services from the Common EAS at this EES. Starting and stopping EAS services is determined by EES based on EAS selection, ACR, etc.This list of served CEAG IDs is useful in order to optimize EES selection for subsequent EECs. - The Service Provisioning request/response have been modified to show the procedure at ECS, which uses the information above about EESs which may already provide services to CEAGs, in addition to the other algorithms. There are 2 ways for ECS to obtain the information about which EESs already provide services to a CEAG: (a) directly from EES by receiving registration updates (b) via CCR, where EES stores and ECS queries the information. - The Service Provisioning request has been enhanced with a list of CEAG profiles which can be provided for the ACs that require common EAS. They are used by ECS to perform the functionality detailed in point 2 above. - The Service Provisioning response has been enhanced so that for the EESs provided to the EEC, it is possible to also indicate which CEAG IDs the EES may be currently supporting, if any. This is the same information discussed above, and it is necessary to be provided to the EEC so it can choose the EES to conduct Common EAS discovery with. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS lacks service provisioning enhancements for Common EAS support based on the merge of the TR solutions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6; 8.3.3.3.2; 8.3.3.3.3; New: 8.2.Y1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#17, sol 27, 28,29,30,31 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## * * * First Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered with the EES. |
| instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| EES Provider Identifier | O | The identifier of the ECSP that provides the EES Provider. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| List of CEAG IDs the EES currently supports via Common EASs | O | This list may be maintained only in deployments without CCR. The EES updates the list when it determines that a registered EAS provides services as Common EAs to at least one AC member of a CEAG. |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

## * * * Next Change * * * *

### 8.2.Y1	CEAG Profile

The CEAG Profile includes information about a group of ACs which require the use of a common EASs.

Table 8.2.Y1-1: CEAG Profile

| Information element | Status | Description |
| --- | --- | --- |
| CEAG ID | M | Identifier of the AC group requiring common EAS (i.e. CEAG) corresponding to the profile |
| CEAG type | M | Indicates CEAG type (dynamic or pre-grouped) |
| Dynamic CEAG filter criteria (NOTE 1) | O | Information for dynamic CEAG. |
| > Group ID | O | If present, it indicates a 3GPP Group ID pre-provisioned (e.g. as External Group ID) to the UEs with ACs in the CEAG. |
| > CEAG service area | O | If provided, it indicates the Service area for determining other UEs part of the CEAG. The UE location is described in clause 7.3.2. The optional additional EAS selection criteria describe criteria for the EAS selection (e.g. "same latency for all" or "lowest latency for the own UE location"). |
| > ACID list | O | List of AC IDs for multi-AC grouping. If provided, the AC IDs of the ACs in the group match the list elements. |
| > ACs Type list | O | List of AC IDs for multi-AC grouping. If provided, the AC types of the ACs in the group match the list elements. |
| Pre-grouped CEAG filter criteria (NOTE 1) | O | Information for pre-grouped CEAG. |
| > Group ID | M | Indicates a 3GPP Group ID pre-provisioned (e.g., as External Group ID) to the UEs with ACs in the CEAG. |
| List of Common EASs | O | List of EAS and/or the service KPIs required by the CEAG. |
| > EASID | O | Identifier of the EAS |
| >Common EASs aggregate Service KPIs | O | Service characteristics provided by the common EAS, detailed in Table 8.2.5-1. The characteristics are described to meet the requirements for the CEAG. |
| NOTE 1: Either “Dynamic CEAG filter criteria” or “Pre-grouped CEAG filter criteria” IEs shall be included, corresponding to the value of CEAG type. |  |  |

## * * * Next Change * * * *

##### 8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location.

If the request includes CEAG Profile information the ECS uses this information (e.g., CEAG IDs) to  query the (EES ID, CEAG ID list) tuples stored by CCR to determine whether an EES is already serving the CEAG(s). The query procedure is described in clause a.b.

When neither AC profiles nor CEAG profiles are provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

If the ECS is not provisioned with any EDN configuration information or is unable to determine the EES information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively. When service provisioning responses are used to determine  EES(s) with which to perform EAS discovery for a common EAS (i.e.an EES for providing CEAG service), and the response provides more than one EES, the EEC can use List of CEAG IDs in the service provisioning response to determine an EES for common EAS discovery.

NOTE 4:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 5:	Even after the EEC establishes a connection to the EES using information received in step 3, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

## * * * Next Change * * * *

##### 8.3.3.3.2	Service provisioning request

Table 8.3.3.3.2-1 describes the information elements for service provisioning request from the EEC to the ECS.

Table 8.3.3.3.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| CEAG profile(s)s | O | Information about CEAG services the EEC wants to connect to, as described in Table 8.2.Y1-1 |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |

Editor's Note: [SA3] Whether the EECID and the UE ID included in request of EDGE-1 & 4 interactions is part of the security credential is SA3's responsibility.

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (NOTE 2) | O | List of EASIDs registered with the EES. |
| > EES Provider identifier | O | The identifier of the EES Provider (such as ECSP) |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > List of CEAG IDs the EES currently supports via Common EASs (NOTE 3) | O | List of CEAG IDs of the CEAGs the EES is serving, i.e., CEAG IDs for which at least one member AC is connected to a Common EAS, where the Common EAS can be discovered via this EES. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles or CEAG Profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: It is FFS whether the list needs to contain the entire CEAG Profile, or the IDs are sufficient. |  |  |