3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230304

e-meeting, 11th – 20th January 2023	(revision of S6-230022)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0035 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of API for change of USS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The API related with change of USS has an Editor’s Note indicating that the API is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The API for all messages related with related with change of USS is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API will be undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 8.2.1, 8.2.4 (new), 8.2.4.1 (new), 8.2.4.2 (new), 8.2.4.3 (new), 8.2.4.4 (new), 8.2.4.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.255 CR #0041 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## * * *   First Change   * * *

# 8	APIs

## * * *   Next Change   * * *

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Manage_C2OperationMode | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| UAE_ChangeUSSManagement API | Manage_USSManagement | UAS application specific server | Request/ Response |
|  | Notify_USSManagementComplete | UAS application specific server | Subscribe/notify |
|  | Manage_USSChange | UAS application specific server | Request/ Response |
|  | Notify_USSChange | UAS application specific server | Subscribe/notify |
|  | Notify_USSChangeTrigger | UAS application specific server | Subscribe/notify |
| NOTE: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Configure_C2OperationMode |  |  |  |

## * * *   Next Change   * * *

### 8.2.4	UAE_ChangeUSSManagement API

#### 8.2.4.1	General

API description: This API enables the UAS application specific server to communicate with the UAE server to send policies, requests for change of USS for the UAS and receive notifications from the UAS UEs (i.e. UAV) about change of USS.

#### 8.2.4.2	Manage_USSManagement operation

API operation name: Manage_USSManagement

Description: Manage (initiate, change or delete) the configuration of the policies for change of USS for the UAS UEs.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.6.3.1.

Outputs: Refer clause 7.6.3.2.

See clause 7.6.2.1 for the details of usage of this API operation.

#### 8.2.4.3	Notify_USSManagementComplete operation

API operation name: Notify_USSManagementComplete

Description: Notification about the USS management completion by UAE server.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.6.3.3.

Outputs: None.

See clause 7.6.2.1 for the details of usage of this API operation.

#### 8.2.4.4	Manage_USSChange operation

API operation name: Manage_USSChange

Description: Manage change of USS on request from UAS application specific server.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.6.3.6.

Outputs: Refer clause 7.6.3.7.

See clause 7.6.2.3 for the details of usage of this API operation.

#### 8.2.4.5	Notify_USSChange operation

API operation name: Notify_USSChange

Description: Notification about the change of USS required by UAS UE.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.6.3.8.

Outputs: None.

See clause 7.6.2.4 for the details of usage of this API operation.

#### 8.2.4.6	Notify_USSChangeTrigger

API operation name: Notify_USSChangeTrigger

Description: Notify a trigger event related to the recommended change of USS.

Known Consumers: UAS application specific server.

Inputs: Refer clause 7.6.3.9.

Outputs: None.

See clause 7.6.2.5 for the details of usage of this API operation.

## * * *   End of Change   * * *