3GPP TSG-SA WG6 Meeting #52bis-e	S6-230321

e-meeting, 11th – 20th January 2023	(revision of S6-230205)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0196 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update service provisioning to support the UE mobility to another ECSP or PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#6, KI#10, Sol#4, Sol#5: In Rel. 18 study in TR 23.700-98, enhancements to ECS provisioning has been introduced, support the UE mobility to another ECSP or PLMN. This CR specifies the enhancements based on solutions #4 and #5 as per clauses 7.4 and 7.5 in TR 23.700-98. Accordingly, the service provisioning procedure needs to be enhanced to support the followings in case that the ECS cannot identify suitable EES(s): - The ECS redirects the requesting EEC to another ECS for service provisioning request. - Alternatively, the ECS communicates to another ECS and retrieve the information of EES(s) registered on that ECS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.2a (new): define roaming architecture with EDGE-10 between ECSs. 6.5.Z (new): define EDGE-10. 6.6.3.Z (new): define cardinality rules for EDGE-10. 8.3.3.2.2, 8.3.3.2.3.3, 8.3.3.3.3, 8.3.3.3.6: update the procedures and information flows for service provisioning. 8.X (new): add a clause for support for inter-ECSP roaming and federation 8.X.Y (new): define new procedures and informations flows for “EES discovery” over EDGE-10 between two ECSs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Provisioning in roaming case and when UE moves from its ECSP to another ECSP is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2a (new), 6.5.Z (new), 6.6.3.Z (new), 8.3.3.2.2, 8.3.3.2.3.3, 8.3.3.3.3, 8.3.3.3.6, 8.3.X (new8.X (new), 8.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#6, KI#10, Sol#4, Sol#5 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: clause 8.X added |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

## 6.2a	Roaming architecture

### 6.2a.1	General

This clause describes the architecture for roaming UEs.

### 6.2a.2	Local breakout roaming architecture: Local breakout to access H-ECS

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. In this architecture, the H-ECS is associated with HPLMN, while the V-ECS and the EDN which the UE accesses are associated with VPLMN. A reference point EDGE-10 is defined between ECSs. Figure 6.2a.2-1 shows the architecture for this model.

NOTE:	H-ECS and V-ECS can be provided by the same ECSP.

Figure 6.2a.2-1: Local breakout roaming architecture: Local breakout to access H-ECS

### 6.2a.3	Home-routed EDGE-4 access to H-ECS

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. Figure 6.2a.3-1 shows the architecture for home routed roaming architecture for this model.

Figure 6.2a.3-1: Home-routed access to H-ECS

In the HR roaming scenario, the roaming architecture is valid if the UE is supported to access the EDN in the VPLMN (i.e. the local access to the EDN of the VPLMN is supported in the HR roaming scenario). The traffic toward the EDN in the VPLMN (i.e. EDGE-1 traffic and application data traffic between AC and EAS) is not home routed to the HPLMN while the traffic between the EEC and H-ECS over is home routed via VPLMN and HPLMN.

NOTE:	How to support local access to the EDN for the VPLMN in HR roaming scenarios is SA2's responsibility.

## 6.2b	Architecture with ECS-ER support

# * * * Change 2 * * * *

### 6.5.Z	EDGE-10

EDGE-10 reference point enables interactions between the ECS and the ECS acting as edge repository. It supports:

a)	registration and de-registration of the ECS to the ECS acting as edge repository; and

b)	retrieval or discovery of information about other ECS(s) of the federation.

# * * * Change 3 * * * *

#### 6.6.3.Z	EDGE-10 (Between ECS and ECS)

Following cardinality rules apply to the ECS-ER that acts as edge repository:

a)	One ECS-ER per ECSP may be deployed to support the federation;

Following cardinality rules apply for EDGE-10:

a)	One or more ECS may communicate with the ECS that acts as edge repository.

# * * * Change 4 * * * *

##### 8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

5.	For inter-ECSP case, the partner ECS information is available at the ECS (see clause 8.X).

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

For inter-ECSP case, e.g. due to mobility of the user to the area not covered by the service area of the ECSP, based on service level agreements between the ECSPs, the ECS of the first ECSP may discover suitable EES(s) via the ECS of the second ECSP. In this case, the ECS of the first ECSP discovers the EES(s) whose service area can cover the UE location via the ECS of the second ESCP using the ECS seivice provision procedure specified in clause 8.X.

If the ECS cannot discover a suitable EES to serve the UE at the received or retrieved UE location based on the received information (e.g. all the EESs registered on the ECS do not cover the given UE location), the ECS may determine potential ECS(s) which may have suitable EES based on the information such as the UE location and service level agreements with the other ECSP. The ECS may determine a potential ECS information (e.g. address, endpoint or service API information) based on pre-configuration. In case that the request in step 1 is from a roaming UE to the ECS in HPLMN, i.e. H-ECS, if the request does not contain the serving PLMN ID, the ECS may interact with 3GPP core network to retrieve serving PLMN ID. The ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the roaming agreement on edge computing services between PLMNs or service agreement between ECSPs.

3.	If the processing of the request was successful, and the ECS identifies the EES(s) in step 2, the ECS responds to the EEC's request with a service provisioning response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

If the ECS identifies the EES(s) in step 2, but alternative ECS(s) has been identified in step 2, the ECS sends a redirect response which includes the list of ECSs information indicating redirection to another ECS for service provisioning request.

If the ECS is not provisioned with any EDN configuration information or is unable to determine the EES information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

When the EES profile(s) include instantiable EAS information, the EEC can select one EES for EAS discovery to mitigate the waste of EDN resources considering the instantiable EAS information, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated.

NOTE 4:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 5:	Even after the EEC establishes a connection to the EES using information received in step 3, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

NOTE 6:	If the EAS instantiation fails based on the selected EES, the EEC may retry the EAS discovery request to another EES.

# * * * Change 5 * * * *

###### 8.3.3.2.3.3	Notify

Figure 8.3.3.2.3.3-1 illustrates the service provisioning notification procedure between the EEC and the ECS.

Pre-conditions:

1.	The EEC has subscribed with the ECS for the provisioning information as specified in clause 8.3.3.2.3.2.

Figure 8.3.3.2.3.3-1: Service provisioning notification

1.	An event occurs at the ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC. If UE's location information is not available, the ECS may obtain the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) were provided by the EEC during subscription creation, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. If AC profiles(s) were not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 1:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 2:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

For inter-ECSP case, e.g. due to mobility of the user to the area not covered by the service area of the ECSP, based on service level agreements between the ECSPs, the ECS of the first ECSP may discover suitable EES(s) via the ECS of the second ECSP. In this case, the ECS of the first ECSP discovers the EES(s) whose service area can cover the UE location via the ECS of the second ESCP using the ECS seivice provision procedure specified in clause 8.X.

If the ECS cannot discover a suitable EES to serve the UE at the received or retrieved UE location based on the received information (e.g. all the EESs registered on the ECS do not cover the given UE location), the ECS may determine potential ECS(s) which may have suitable EES based on the information such as the UE location and service level agreements with the other ECSP. The ECS may determine a potential ECS information (e.g. address, endpoint or service API information) based on pre-configuration. In case that the request in step 1 is from a roaming UE to the ECS in HPLMN, i.e. H-ECS, if the request does not contain the serving PLMN ID, the ECS may interact with 3GPP core network to retrieve serving PLMN ID. The ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the roaming agreement on edge computing services between PLMNs or service agreement between ECSPs.

2.	The ECS sends a provisioning notification to the EEC with the list of EDN configuration information determined in step 1.

If the EDN configuration information in the service provisioning notification includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area, which can be discovered using the UE Registration Procedure.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

NOTE 3:	Even after the EEC establishes a connection to the EES using information received in step 2, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

# * * * Change 6 * * * *

##### 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |
| Redirect | O | Indicates redirection to (an)other ECS(s). |
| > ECS(s) information | M | Endpoint address of ECS(s) to which the UE is redirected for service provisioning. |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (NOTE 2) | O | List of EASIDs registered with the EES. |
| > instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| > EES Provider identifier | O | The identifier of the EES Provider (such as ECSP) |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). |  |  |

# * * * Change 7 * * * *

##### 8.3.3.3.6	Service provisioning notification

Table 8.3.3.3.6-1 describes the information elements for service provisioning notification from the ECS to the EEC.

Table 8.3.3.3.6-1: Service provisioning notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the ECS for the request |
| List of EDN configuration information (NOTE) | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Redirect (NOTE) | O | Indicates redirection to (an)other ECS(s). |
| > ECS(s) information | M | Endpoint address of ECS(s) to which the UE is redirected for service provisioning. |
| NOTE: One of the IEs shall be present. |  |  |

NOTE:	When SEAL NMS is used, Service provisioning notification is the Notification data in SEAL Notification message of clause 17.3.2.4 in 3GPP TS 23.434 [13].

Editor's note:	Similar updates to other Subscription and Notification messages is FFS.

# * * * Change 8 * * * *

### 8.X	Support for inter-ECSP roaming and federation

#### 8.X.1	General

In inter-ECSP case, e.g. due to mobility of the user to the area not covered by the service area of the ECSP, the ECS might not be able to discover any EES. In that case, based on service level agreements between the ECSPs, the ECS of the first ECSP may determine or communicate to the ECS of another ECSP.

#### 8.X.2	ECS discovery using ECS-ER

# * * * Change 9 * * * *

#### 8.X.Y	Fetch service provisioning information

##### 8.X.Y.1	General

The following clauses specify procedures, information flows and APIs for ECS service provisioning.

##### 8.X.Y.2	Procedures

###### 8.X.Y.2.1	General

Following procedures are supported for service provisioning:

-	Request-response procedure;

Editor's note:	subscribe-notify model for ECS service provisioning is FFS.

###### 8.X.Y.2.2	Request-response model

Figure 8.X.Y.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-condition(s):

1.	ECS1 has received a sevice provisioning request from an EEC and the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g. all the EESs registered on the ECS1 do not cover the given UE location),

2.   ECSP1 and ECSP2 have a service level agreement to share edge services in a way that if the ECS1 cannot discover a suitable EES to serve the UE at the current location, the ECS1 may contact another, i.e. ECS2, which may have suitable EES for service provisioning.

NOTE:	To configure sufficient information to the ECS, the ECS(s) information related to other ECSPs may be available at the OAM system due to the inter-ECSP relationship establishment, which is then used by the OAM system of an ECSP to configure its ECS. If required, the information of available applications in a partner ECSP and the corresponding service areas are included in the configured information. Inter-ECSP relationship establishment is according to the business relationship between the ECSPs and is out of the scope of SA6. The OAM to configure its ECS for inter-ECSP relationship is under the scope of SA5.

Figure 8.X.Y.2.2-1: Fecth Service provisioning information – Request/Response

1.	The ECS1 sends an Fetch service provisioning information request to another ECS, i.e. ECS2. The request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information.

2.	Upon receiving the request, the ECS2 performs an authorization check to verify whether the ECS1 has authorization to perform the operation. If AC profile(s) are provided in step 1, the ECS2 identifies the EES(s) based on the provided AC profile(s) and the UE location. When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS2 and the UE location;

-	ECS2 identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

The ECS2 also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

3.	If the processing of the request was successful, the ECS2 responds to the ECS1's request with a Fetch service provisioning information response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

If the ECS2 is not provisioned with any EDN configuration information or is unable to determine the EES information using the inputs in the request in step 1 or the ECSP's policy, the ECS2 shall reject the request and respond with an appropriate failure cause.

The ECS1 may cache the fetched service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Fetch service provisioning information response from ECS2, then the ECS1 may cache and reuse the fetched service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

Editor's note:	Information flows for Fetch service provisioning information is FFS. It is FFS whether this procedure and its information flows can be merged to the existing service discovery procedure and flows.

# * * * End of changes * * * *