3GPP TSG-SA WG6 Meeting #52	S6-230339

Toulouse, France 14th – 18th November 2022	(revision of S6-230227)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0160 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL server provisioning for Key Management Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following requirements needs to be implemented for KMS - TS 33.434 - Clause 5.3.1 General - “A VAL server is provisioned with an access token scoped for SEAL key management services and is provided with each and every key management request to the SKM-S. ” - TS 33.434 - 5.3.2 SEAL KM Request message - “The format and content of a key management record is defined and securely provisioned into the SEAL KMS by the VAL application or VAL service owner/operator. ” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a procedure and information flows for VAL server to provision requried configuration to the SEAL KMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEAL KMS will not be aware of the key format and scope for the VAL server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13,3,2 (NEW), 13.3.2.1 (NEW), 13.3.2.2 (NEW), 13.3.x (NEW), 13.3.x.1 (NEW), 13.3.x.2 (NEW), 13.4.1, 13.4.3 (NEW), 13.4.3.1 (NEW), 13.4.3.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 13.3.2	Information flows

#### 13.3.2.1	VAL server provisioning request

Table 13.3.2.1-1 describes the information flow from the VAL server to the key management server for providing provisioning configuration.

Table 13.3.2.1-1: VAL server provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL service specific information | M | Provides the list of VAL service specific information |
| > VAL service ID | M | Identify of the VAL service for which the configuration information is provided. |
| > key-format | M | Provides the format and content of a key management record |

#### 13.3.2.2	VAL server provisioning response

Table 12.3.2.2-1 describes the information flow from the key management server to the VAL server as a response for providing provisioning configuration.

Table 13.3.2.2-1: VAL server provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * Next Change * * * *

### 13.3.x	VAL server provisioning for key management service

#### 13.3.x.1	General

The high level procedure for VAL server to provision required information to SEAL key management server in order to provide the format and content of a key management record is described in the following subclause.

#### 13.3.x.2	Procedure

The procedure for VAL server to provision required information to SEAL key management server in order to support VAL user authentication is illustrated in figure 13.3.x.2-1.

Figure 13.3.x.2-1: VAL Server provisioning to SEAL Key Management Server

1.	The VAL server sends a request message to key management server to provision required information. The request message includes identity of the VAL server, endpoint information of the VAL server, security credentials of the VAL server, and service provider specific information like key format per VAL service.

2.	Upon receiving the request, the key management server authorizes the request based on the security credentials provided in the request and considering the service level agreement between VAL service provider and SEAL service provider. If VAL server is authorized to use the SEAL service, then the key management server stores the details about the VAL server including the key format per VAL service. The key management server sends the response message to the VAL server.

Editor’s note:	Whether the VAL server registers with Key Management Server is FFS.

* * * Next Change * * * *

### 13.4.1	General

Table 13.4.1-1 illustrates the SEAL APIs for key management.

Table 13.4.1-1: List of SEAL APIs for key management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_KmsVALServerProvisioning | Provide_Configuration | VAL server | Request /Response |

The other SEAL APIs for Key Management are specified in subclauses 5.7.1 and 7.6.1 of 3GPP TS 29.549 [30].

* * * Next Change * * * *

### 13.4.3	SS_KmsVALServerProvisioning API

#### 13.4.3.1	General

API description: This API enables the VAL server to provision configuration for the VAL service to the SEAL KM-S.

#### 13.4.3.2	Provide_Configuration operation

API operation name: Provide_Configuration

Description: Provisioning of VAL service configuration to IM-S.

Known Consumers: VAL server.

Inputs: See subclause 13.3.2.1

Outputs: See subclause 13.3.2.2

See subclause 13.3.x.2 for the details of usage of this API operation.

* * * End of Change * * * *