3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230342

e-meeting, 11th – 20th January 2023	(revision of S6-230232)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0204 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACR Scenario selection by EEC for T-EES and T-EAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.15 specifies procedure to select supported ACR scenario list based on AC, EEC, EES and EAS. However, when UE moves to new location, due to service continuity procedures, EEC may connect to new EES (i.e. T-EES) and AC may get service from new EAS (i.e. T-EAS). It is possible that both S-EES and T-EES are from differnet service providers and may not support exactly same ACR scenario list. Similarly, It is possible that both S-EAS and T-EAS are from differnet service providers and may not support exactly same ACR scenario list. In such case, it is required to select supported ACR scenario list again based on T-EES and T-EAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a clarification for EEC to either select ACR scenarios based on T-EES and T-EAS supported scenarios or request T-EES to select ACR sccenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACR scenarios will not be selected for T-EEs and T-EAS and so T-EES or T-EAS will not be aware of ACR detection and execution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to KI#19, Sol #38 of 3GPP TR 23.700-98 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.8.3.5.3	Notify

Figure 8.8.3.5.3-1 illustrates the ACR information notification procedure between the EEC and the EES, which can be used by the EES to notify the EEC of the following:

-	target information, i.e. the details of the selected T-EAS and, if required, the selected T-EES, during ACR as described in clauses 8.8.2.4 and 8.8.2.5;

NOTE:	The T-EAS and T-EES information can be used to determine the PDU session(s) to provide connectivity to the T-EAS and the T-EES. If the ACR does not require change in EES, i.e. T-EES is same as S-EES, then the T-EES information can be skipped.

-	ACR complete events.

Pre-conditions:

1.	The EEC has subscribed with the EES for the ACR information as specified in clause 8.8.3.5.2.

Figure 8.8.3.5.3-1: ACR information notification

1.	An event (e.g. ACR complete, or Target information notification) occurs at the EES that satisfies trigger conditions for providing ACR information to a subscribed EEC.

2.	The EES sends an ACR information notification to the EEC with the ACR information determined in step 1. The ACR information notification may include ACID to indicate the application context relocation of the AC is complete. If the S-EES has received the successful EEC Context Push response from T-EES, along with registration ID and the registration expiration time in the EEC Context Push relocation procedure, then the ACR information notification towards EEC also includes the registration ID and registration expiration time under EEC context relocation status (for successful status). Upon receiving the target information notification to indicate start of the ACR execution, the EEC avoids triggering a second ACR execution for the same ACR identity (ACID, UE ID, S-EAS endpoint and T-EAS endpoint) until the current ACR execution is completed. For ACR scenarios as specified in clause 8.8.2.2 and clause 8.8.2.3, upon receiving the ACR complete notification with T-EES supported ACR scenarios, the EEC may either selects ACR scenario considering the supported ACR scenarios of AC, EEC, T-EES and T-EAS or requests T-EES to select list of ACR scenarios as specified in clause 8.15.

Editor's note: In order to start detect or decide, till how long the detection entity should wait for current ACR to complete, is FFS

* * * End of Change * * * *