3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230346

e-meeting, 11th – 20th January 2023	(revision of S6-230236)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0208 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN related to ACR combination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are Editor nots in clause 8.6.3.2.3 and clause 8.8.3.5.3 as follows: Editor's note: In order to start detect or decide, till how long the detection entity should wait for current ACR to complete, is FFS As per the clause 8.6.3.2.3 and clause 8.8.3.5.3, upon receiving the notification, the EAS or EEC avoid initiating another ACR for the same pair of AC and EAS, till the ACR complete notification is received. There can be a case when ACR complete notification (of ongoing ACR) is lost and due to this EAS or EES will never be able to initiate new ACR. It is required to start a timer once notification indicating start of ACR is received, and if the ACR is not completed by the time the timer is expired then EAS or EEC can initiate ACR if required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed ENs Specified to start timer upon receiving the notification indicating start of the ACR execution, and stop the timer upon receving the ACR complete notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EEC and EAS may not be able to execture ACR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.2.3, 8.8.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related to KI#18, Sol#38 of 3GPP TR 23.700-98 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.8.3.5.3	Notify

Figure 8.8.3.5.3-1 illustrates the ACR information notification procedure between the EEC and the EES, which can be used by the EES to notify the EEC of the following:

-	target information, i.e. the details of the selected T-EAS and, if required, the selected T-EES, during ACR as described in clauses 8.8.2.4 and 8.8.2.5;

NOTE:	The T-EAS and T-EES information can be used to determine the PDU session(s) to provide connectivity to the T-EAS and the T-EES. If the ACR does not require change in EES, i.e. T-EES is same as S-EES, then the T-EES information can be skipped.

-	ACR complete events.

Pre-conditions:

1.	The EEC has subscribed with the EES for the ACR information as specified in clause 8.8.3.5.2.

Figure 8.8.3.5.3-1: ACR information notification

1.	An event (e.g. ACR complete, or Target information notification) occurs at the EES that satisfies trigger conditions for providing ACR information to a subscribed EEC.

2.	The EES sends an ACR information notification to the EEC with the ACR information determined in step 1. The ACR information notification may include ACID to indicate the application context relocation of the AC is complete. If the S-EES has received the successful EEC Context Push response from T-EES, along with registration ID and the registration expiration time in the EEC Context Push relocation procedure, then the ACR information notification towards EEC also includes the registration ID and registration expiration time under EEC context relocation status (for successful status). Upon receiving the target information notification to indicate start of the ACR execution, the EEC avoids triggering a second ACR execution for the same ACR identity (ACID, UE ID, S-EAS endpoint and T-EAS endpoint) until the current ACR execution is completed.

NOTE:	The EEC may start a timer upon receiving the notification from the EES about the start of the ACR by EAS or EES, and terminates the timer upon receiving the notification from the EES about the completion of the ACR. If the notification from the EES about the completion of the ACR is not received, the EEC may initiate another ACR, if required, after timer expires.

* * * End of Change * * * *