3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230365

# E-meeting, 11th January – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0143 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configure and Update service operations for the SS_VALServiceAreaConfiguration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The VAL service area ID support was introduced for SEAL Location Mangament APIs in CR #0136 of 23.434 (agreed in SA6 52 meeting). Thus, the related service operations shall be specified to enable the VAL service area ID functionality. The Configure/Update operations are part of the CRUD operations that only apply to a single VAL server operation and required for the stage 3 implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces new service operations and related information flows for the SS_VALServiceAreaConfiguration API: - Configure; and - Update. It is proposed to correct “Geographical coordinates” to “Geographical location” to enable a flexible definition of the VAL service area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CRUD service operations for a single VAL server with the related procedures and information flows are unspecified for the SS_VALServiceAreaConfiguration API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.21, 9.3.2.22, 9.3.2.23(new), 9.3.2.24(new), 9.3.13.2, 9.3.13.3(new), 9.4.1, 9.4.8.2, 9.4.8.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - In Revision 1, the CR scope is reduced to single VAL server use case, i.e., CR proposes the CRUD operations only. |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.21	VAL service area configuration request

Table 9.3.2.21-1 describes the information flow from the the VAL server to the location management server for VAL service area configuration request.

Table 9.3.2.21-1: VAL service area configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server. |
| List of VAL service area information | M | List of one or more VAL service area identifiers. |
| > VAL service area ID | M | The VAL service area identifier. |
| > Geographical locations | M | List of one more geographical locations (e.g., co-ordinates) corresponding tothe VAL service area ID. |

* * * Next Change * * * *

#### 9.3.2.22	VAL service area configuration response

Table 9.3.2.22-1 describes the information flow from the location management server to the VAL server for VAL service area configuration response.

Table 9.3.2.22-1: VAL service area configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL Service area configuration status | M | It indicates the result of VAL service area configuration request. |
| List of VAL service area identifiers | O (see NOTE) | If configuration is successful, identifies the list of the configured VAL service area identifiers. |
| NOTE: This IE shall be present only if the configuration status is set to success. |  |  |

* * * Next Change * * * *

#### 9.3.2.23	VAL service area update request

Table 9.3.2.23-1 describes the information flow from a VAL server to a location management for update VAL service area identifiers at the LM server.

Table 9.3.2.235-1: Update VAL service area request

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL service area information | M | List of one or more VAL service area identifiers that shall be updated. |
| > VAL service area ID | M | Indicates the VAL service area identifier. |
| > Geographical locations | M | Indicates the list of the updated geographical locations (e.g., the geographical coordinates) corresponding to the VAL service area ID. |

* * * Next Change * * * *

#### 9.3.2.24	VAL service area update response

Table 9.3.2.24-1 describes the information flow from a location management server to the VAL server for update VAL service area identifiers at the LM server.

Table 9.3.2.24-1: Update VAL service area response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation. |
| VAL service area IDs | O (see NOTE) | Indicates the list of the updated VAL service area identifiers. |
| NOTE: This information element shall be present only if the update VAL service area identifier(s) is successful. |  |  |

* * * Next Change * * * *

#### 9.3.13.2	Configure VAL service area identifier procedure

Figure 9.3.13.2-1 illustrates the high level procedure of the VAL service area identifier configuration.

Figure 9.3.13.2-1: Configure VAL service area identifier procedure

- The VAL server sends the VAL service area configuration request to the location management server to configure a new VAL service area identifier. In the request message, the VAL server includes the information as specified in table 9.3.2.21-1.

- The VAL server checks the consistency of the provided identifiers (e.g., all VAL service area identifiers shall be unique) and configures the provided VAL service area identifiers for the VAL server.

- The location management server sends the VAL service area configuration response with the information as specified in table 9.3.2.22-1.

* * * Next Change * * * *

#### 9.3.13.3	Update VAL service area identifier procedure

Figure 9.3.13.3-1 illustrates the high level procedure of the VAL service area update.

Figure 9.3.13.3-1: Update VAL service area procedure

- The VAL server sends the VAL service area update request to the location management server to update the VAL service area identifier(s) configured by the VAL server. In the request message, the VAL server includes the information as specified in table 9.3.2.23-1;

2. 	The location management server checks whether the VAL service area IDs exist and then updates the VAL service area identifier(s), and provisions the updated VAL service area identifier(s) across SEAL APIs for location management.

NOTE:	The provisioning of the updated VAL service area identifier(s) across the SEAL APIs for location management is implementation specific.

3.	The location management server sends the VAL service area update response with the information as specified in table 9.3.2.24-1.

* * * Next Change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Update_VAL_Service_Area | VAL server |  |

* * * Next Change * * * *

#### 9.4.8.2	Configure_VAL_Service_Area

API operation name: Configure_VAL_Service_Area

Description: Configuration of VAL service area identifiers.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.21

Outputs: See subclause 9.3.2.22.

See subclause 9.3.13.2 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.3	Update_VAL_Service_Area

API operation name: Update_VAL_Service_Area

Description: Update of the VAL service area identifier.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.23

Outputs: See subclause 9.3.2.24.

See subclause 9.3.13.3 for the details of usage of this API operation.

* * * End of changes * * * *