3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230376

e-meeting, 11th – 20th January 2023	(revision of S6-230026)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0039 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of procedures for DAA configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of procedures for DAA configuration by the UAE-layer is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of procedures for DAA configuration by the UAE-layer is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The addition of procedures for DAA configuration by the UAE-layer will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 (new), 7.7.1 (new), 7.7.2 (new), 7.7.2.1 (new), 7.7.2.2 (new), 7.7.3 (new), 7.7.3.1 (new), 7.7.3.2 (new), 7.7.3.3 (new), 7.7.3.4 (new), 7.7.3.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## * * *   Start of Change   * * *

## 7.7	UAE layer support for DAA services and applictions

### 7.7.1	General

This feature enables the UAS application enablement services for assisting the UAS application with DAA handling. In particular, the UAE layer provides support for the following operations:

-	Support of the registration of the UAE clients DAA assistance capability to the UAE server as described in clause 7.1a.

-	Support the distribution of the DAA policies from the UAS application specific server to the UAE server and the UAE client, as described in clause 7.7.2.1 and clause 7.7.2.2.

Editor’s Note:	The procedures for assistance by the UAE layer are FFS.

A DAA policy has two components:

- The DAA application policy. This policy is used by the application and transparently provided from the UAS application specific server to the UAS client. The DAA application policy shall be timestamped and stored at the UAE server.

- The DAA support policy. This policy is used by the UAE layer, and contains information pertaining to the UAE layer. This policy is used by the UAE layer to provide support for DAA applications.

NOTE:	The Detect and Avoid operations performed by the UAS application layer are out of scope of the present specification.

### 7.7.2	Procedures

#### 7.7.2.1	Management of DAA support configuration

Figure 7.7.2.1-1 illustrates the DAA support management procedure where the UAE server receives an application request for managing the DAA configuration parameters from the UAS application specific server.

Pre-condition:

-	The UAV has received its UAS ID from the UAS application specific server.

-	The UAV has performed the UAS UE registration procedure.

Figure 7.7.2.1-1: DAA support management procedure

1.	The UAS application specific server sends to the UAE server a DAA support management request. The request includes the UAV (UAE client) identifier and the DAA policies.

2.	The UAE server shall send to the UAS application specific server a DAA support management response with a positive or negative acknowledgement of the request.

3.	The UAE server shall execute the DAA configuration according to clause 7.7.2.2.

4.	After execution of DAA configuration, the UAE server shall send a DAA support management complete to the UAS application specific server.

#### 7.7.2.2	DAA support configuration procedure

Figure 7.7.2.2-1 illustrates the DAA support configuration procedure. This procedure enables the configuration of the UAE client, based on a request from UAS application specific server to configure DAA policies to the UAE client.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE server has established a UAE session with the respective UAE clients as the UAE clients are successfully registered to the UAE server.

3.	UAE server has performed the DAA support management procedure according to clause 7.7.2.1.

Figure 7.7.2.2-1: DAA support configuration procedure

1.	The UAE server shall send a DAA support configuration request to the UAE client. The UAE client receives a DAA support configuration request from the UAE server that includes the DAA configuration parameters.

2.	The UAE client shall store or remove the DAA configuration parameters as per the information received in step 1.

3.	The UAE client shall send a DAA support configuration response to the UAE server.

### 7.7.3	Information flows

Editor’s Note:	The structure of the DAA policies and the related information elements is FFS.

#### 7.7.3.1	DAA support management request

Table 7.7.3.1-1 describes the information flow DAA support management request from the UAS application specific server to the UAE server.

Table 7.7.3.1-1: DAA support management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the DAA management. This ID can be the USS identifier, when the UAS application specific server is the USS. |
| UAS ID | M | The identification of the UAS for which the DAA support management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| The DAA application policy | O | The DAA application policy. |
| The DAA support policy container (see NOTE) | O | The DAA support policy container consists of the requirements and policy for DAA to be used by the UAE layer. |
| NOTE: If DAA support policy container is not included for a USS, it indicates removal of the DAA support policy related information for this USS. |  |  |

#### 7.7.3.2	DAA support management response

Table 7.7.3.2-1 describes the information flow DAA support management response from the UAE server to the UAS application specific server.

Table 7.7.3.2-1: DAA support management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the DAA support management request. |

#### 7.7.3.3	DAA support management complete

Table 7.7.3.3-1 describes the information flow DAA support management complete from the UAE server to the UAS application specific server.

Table 7.7.3.3-1: DAA support management complete

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of provision of the DAA application policy to the UAS application. |

#### 7.7.3.4	DAA support configuration request

Table 7.7.3.4-1 describes the information flow DAA support configuration request from the UAE server to the UAE client.

Table 7.7.3.4-1: DAA support configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the DAA management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| The DAA application policy | O | The DAA application policy. |
| The DAA support policy container (see NOTE) | O | The DAA support policy container consists of the requirements and policy for DAA. |
| NOTE: If DAA support policy container is not included for a USS, it indicates removal of the DAA support policy related information for this USS. |  |  |

#### 7.7.3.5	DAA support configuration response

Table 7.7.3.5-1 describes the information flow DAA support configuration response from the UAE client to the UAE server.

Table 7.7.3.5-1: DAA support configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of provision of the DAA application policy to the UAS application. |

## * * *   End of Change   * * *