3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230377

e-meeting, 11th – 20th January 2023	(revision of S6-230107_was_ 223621)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0138 | rev | 6 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EEC triggering service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | KI#1 in FS_eEDGEAPP is concluded to address, in the normative work, service provisioning triggering by utilizing application triggering via SMS over NAS or SMS over IP. In this regard, this CR proposes how to enable EEC and EES/ECS to enable such application triggering for the porcedures over EDGE-1 and EDGE-4. This approach does not require EEC to persistently keep the session with ECS or EES (e.g., for receiving the updated EDN configuration information). To support EEC triggering, it necessary to provide from the triggering entity (especially for EES) which service should be triggered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description on enabling EEC triggering service for the procedures over EDGE-1 and EDGE-4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EEC triggering service for procedures over EDGE-1 and EDGE-4 is not supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to KI#1/Sol#3 in TR 23.700-98. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * * (all new text)

8.x	EEC triggering service to initiate procedures over EDGE-1 or EDGE-4

8.x.1	General

The functional entities of the Edge Enabler Layer can utilize the Application Triggering services (or Device Triggering) provided by the 3GPP core network via NEF or SCEF (see clause 4.4.5 in 3GPP TS 23.501 [2], clause 4.13.2, 5.2.6.5.2 in TS 23.502 [3], and clause 4.5.1 TS 23.682 [17]) to enable for ECS or EES to trigger EEC to perform the procedures over EDGE-1 or EDGE-4.

Editor's Note:	It is FFS whether and how the EEC indicates to the ECS or EES that the EEC triggering service is supported. If it is agreed that the EEC indicates such information to the ECS or EES, it is FFS whether and how to describe the detailed EEC operation checking if EEC triggering service via Application Triggering or Device Triggering mechanism is available.

If the EES and ECS knows that EEC triggering service is available for the EEC, the triggering mechanism is used by the ECS and EES for messaging to EEC by invoking application triggering service as specified for Nnef_ Trigger_Delivery service to in clause 4.13.2 of TS 23.502 [3] or the device triggering procedure via T8 in clause 5.17 of TS 23.682 [17].

Editor's Note:	It is FFS how to describe the EEC triggering procedure according to the decision on either having it as a separate procedure or updating the existing procedure

When the EDN configuration information in the ECS is updated, if EEC triggering service is available for the ECS and EEC, the ECS triggers the EEC to initiate service provisioning request by invoking application triggering service as specified for Nnef_ Trigger_Delivery service to in clause 4.13.2 of TS 23.502 [3] or for Device triggering procedure via T8 in clause 5.17 of TS 23.682 [17].

When the EAS profile in the EES is updated, if EEC triggering service is available for the EES and EEC, the EES triggers the EEC to initiate EAS discovery request or EAS by invoking application triggering service as specified for Nnef_ Trigger_Delivery service to in clause 4.13.2 of TS 23.502 [3] or for Device triggering procedure via T8 in clause 5.17 of TS 23.682 [17].

* * * End of Change * * * *