3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230465

e-meeting, 11th – 20th January 2023	(revision of S6-230410)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0167 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture with CAS and CES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The archietecture with CES to support CAS is transposed from TR to the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the architecture for CAS with CES. Describe new reference points. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for new architecture to support ACR btw. CAS and EAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2D (new), 6.3.Y (new), 6.5.X3 (new), 6.5.X4 (new), 6.5.X5 (new), 6.5.X6 (new), 6.5.X8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | KI#11, Sol#24 @MCC, please update the clause numbering if needed (e.g. from 6.2D to 6.2C, or 6.2E …). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

ACR	Application Context Relocation

ACT	Application Context Transfer

AF	Application Function

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECI	Edge and Cloud Interworking

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EEL	Edge Enabler layer

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

GSM	Global System for Mobile Communications

GSMA	GSM Association

LADN	Local Area Data Network

NEF	Network Exposure Function

OP	Operator Platform

OPG	Operator Platform Group

S-EAS	Source Edge Application Server

S-EES	Source Edge Enabler Server

SCEF	Service Capability Exposure Function

SSID	Service Set Identifier

T-EAS	Target Edge Application Server

T-EES	Target Edge Enabler Server

TAI	Tracking Area Identity

* * * First Change * * * *

### 6.2D	Architecture for enabling cloud applications with edge applications, with CES support

Figure 6.2D-1 illustrates the architecture for enabling cloud applications along with the edge applications, when CES is used.

NOTE:	Edge and cloud servers can utilize SEAL NM service but for simplicity such an interaction is not depicted in the figure.

Figure 6.2D-1: Architecture for enabling cloud application with edge applications

Cloud Application Server (CAS) residing in the clould DN may need to interact with the Cloud Enabler Server (CES) for interworking, e.g. for service continuity. The CAS and EAS interaction is Application Data Traffic, which is out-of-scope of this specification.

Editor’s Note:	The ECI and CLOUD reference points re-numbering may be needed.

* * * Next Change * * * *

### 6.3.Y	Cloud Enabler Server (CES)

CES provides supporting functions needed for CASs. The CES does not have service area restriction.

Editor’s Note:	The CES functionalities are FFS, will be completed when detailed procedures with CES are introduced.

* * * Next Change * * * *

### 6.5.X3	ECI-3

ECI-3 enables interaction between CES and ECS.

Editor's note:	What functionalities of EDGE-6 are reused for ECI-3 is FFS.

### 6.5.X4	ECI-4

ECI-3 enables interaction between CES and EES.

Editor's note:	What functionalities of EDGE-9 are reused for ECI-4 is FFS.

### 6.5.X5	CLOUD-1

CLOUD-1 enables interaction between CAS and CES.

Editor's note:	What functionalities of EDGE-3 are reused for CLOUD-1 is FFS.

### 6.5.X6	CLOUD-2

CLOUD-2 enables interaction between CES and CES.

Editor's note:	What functionalities of EDGE-9 are reused for CLOUD-2 is FFS.

### 6.5.X8	CLOUD-4

CLOUD-4 enables interaction between CES and the 3GPP core network.

* * * END of Change * * * *