3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230109

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

Source:	Lenovo

Title:	Predictive slice modification in edge based NSCE deployments

Spec:	3GPP TS 23.435 v0.4.0

Agenda item:	8.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This contribution proposes a new feature for supporting the slice modification in edge based NSCE deployments, based on solution #21 of TR 23.700-99.

2. Reason for Change

This contribution proposes a new feature for supporting the slice modification in edge based NSCE deployments, based on solution #21 of TR 23.700-99. In this feature, the brief feature introduction is provided in clause 4.x, as well as the related procedures (9.x.2, 9.x.3) and IE definition in clause 9.x.4.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v0.4.0.

* * * First Change * * * *

## 4.x	Predictive slice modification in edge based NSCE deployments

This feature addresses scenarios where the NSCE server is deployed at the edge and the migration to different Data Network will require that the ongoing slice is supported at the target area to ensure meeting the application session requirements. The slice parameters monitoring at the target area (e.g. for per NSI/NSSI resource situation) need to be known at the server NSCE server to allow for pro-active slice (or slice subnet) modification trigger to avoid degradation of the application service performance.

* * * Next Change * * * *

## 9.x	Procedure on predictive slice modification in edge based NSCE deployments

### 9.x.1	General

In this feature, the NSCE server initially receives an expected/predicted UE location/mobility change request outside an EDN service area for one or more UEs within the VAL application session (e.g. such session can be an indirect V2V session or a multiplayer gaming session). Then, the source NSCE server checks with 5GS (OAM, 5GC) whether the serving slice is available and can offer the same performance at the target EDN. Thereafter, NSCE server evaluates the need for a slice modification (e.g. a slice lifecycle related trigger change) e.g. a slice subnet resource adaptation to allow for optimizing the application performance at the target area. Based on this decision/recommendation, it provides the action to the OAM and supports the re-mapping of NSCE server for the NSCE client proactively, before UE mobility happens.

### 9.x.2	Procedure

In the procedure shown in Figure 9.x.2-1, a mechanism is provided to allow for slice modification when a vertical application migrates (or is expected to migrate) to a different EDN supported by different NSCE server.

Pre-conditions:

1.	The VAL server has subscribed to the network slice capability enablement server

2.	The VAL client of VAL UE is mapped to Slice#1, and NSCE client of VAL UE has established a connection to NSCE server#1 (S-NSCE server).

3.	The S-NSCE server has already discovered the T-NSCE server and its area of coverage.

4.  The VAL server is subscribed to and received prediction of UE location change

Figure 9.x.2-1: Support for predictive slice modification in distributed NSCE server deployments

- The VAL server sends to S-NSCE server an application service continuity requirement request due to predicted/expected UE or group UE mobility to a target service area covered by a different EDN.

NOTE: Such UE predicted mobility at the VAL server can be based on UE mobility analytics received by NWDAF or can be predicted by the VAL layer (VAL server or VAL UE).

- S-NSCE server sends an application service continuity requirement response to the VAL server as positive or negative acknowledgement.

- S-NSCE server determines to query the underlying 3GPP system on the slice availability and conditions at the target service area (based on step 1 requirement). Such query may be in form of a request/response and include:

  - S-NSCE server interacting with 5GC to query the UE specific information (location, UE connection capabilities) as well as network conditions (network monitoring from NEF) and/or slice related analytics on the slice load (from NWDAF as specified in TS 23.288 [x]).

  - S-NSCE server may also interact with OAM to query on the target slice availability and the up-to-date configured slice parameters e.g. slice RRM policies, modification of the NSI/NSSI resources (see TS 28.531[y], 5.1.12) at the target service area and measurements for the slice at the target area.

- S-NSCE server checks whether new NSCE service area supports slice #1 and if slice #1 offers similar performance in target area. If the current slice doesn't fulfil these requirements, S-NSCE interacts with T-NSCE server (by sending the VAL application service continuity requirement and optionally a proposed action) to negotiate and allow T-NSCE server to determine the need for a slice lifecycle change at the target area and translates this to a trigger action towards OAM (slice provisioning MnS producer).

This trigger action can be the outcome of the negotiation and can be a requested slice modification or the slice #1 creation/instantiation at the target area (this may happen if a group of UEs are moving to the target area and use slice #1, so it may be beneficial to create slice #1 at the target area). Such trigger action can be generated by either the S-NSCE server or the T-NSCE server based on the deployment.

- The S- or T-NSCE server may send the trigger action as a slice modification trigger request to the slice provisioning MnS producer at OAM (e.g. slice modification for network slice) to extend slice availability to the target service area based on the expected/predicted VAL UE or VAL group mobility. As response to the trigger action, the provisioning MnS producer provides a slice modification trigger response to the corresponding NSCE server with a positive or negative result.

- After the slice lifecycle change execution (based on the indication in step 5), the S-NSCE server sends a notification to the VAL server and optionally to the VAL client via S-NSCE client

7.	If the NSCE client needs to be remapped to different NSCE server (due to the expected change of UE location), the NSCE client establishes a new connection with T-NSCE and terminates the one with S-NSCE (in case of subscription-based interaction), or in case of request-based interaction, it updates the mapping at the client side, and maintains the new NSCE server address / ID for the target NSCE area.

### 9.x.3	Information flows

#### 9.x.3.1	General

The following information flows are specified for the predictive slice modification support based on 9.x.2.

#### 9.x.3.2	Application service continuity requirement request

Table 9.x.3.2-1 describes information elements for the Application service continuity requirement request from the VAL server to the NSCE server.

Table 9.x.3.2-1: Application service continuity requirement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Service Continuity Requirement | M | The service continuity requirement which can be the expected or predicted migration of the VAL application or a list of VAL UEs within the application to a target area. |
| Slice identifier | O | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Target Service Area | M | The target area can be represented as the edge service area (including the target DNN/DNAI) or the topological area (e.g. cell ID) or the geographical coordinates / set of waypoints outside the original service area, where the VAL application/ UE(s) is expected or predicted to move. |
| Application QoS requirements | O | The QoS requirements / KPIs for the VAL service |

#### 9.x.3.3	Application service continuity requirement response

Table 9.x.3.3-1 describes information elements for the Application service continuity requirement response from the NSCE server to the VAL server.

Table 9.x.3.3-1: Application service continuity requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement) |

#### 9.x.3.4	slice modification notify

Table 9.x.3.4-1 describes information elements for the slice modification notify message from the NSCE server to the VAL server or the VAL client (via NSCE client).

Table 9.x.4.4-1: slice modification notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The identifier of the VAL application which is expected to be impacted by the slice modification |
| VAL UE ID list | O | The identifiers of the VAL UEs which are expected to be impacted by the slice modification |
| Slice identifier | M | The slice identifier (S-NSSAI, NSI ID or ENSI) which is expected or predicted to modify to extend slice availability to the target service area |
| Target NSCE server ID | M | The identifier of the target NSCE server |
| Target Service Area | M | The target area can be represented as the edge service area (including the target DNN/DNAI) or the topological area (e.g. list of cells/TAs) for which the slice modification applies. |

### 9.x.4	APIs

Table 9.x.4-1 illustrates the NSCALE APIs for the predictive slice modification support feature.

Table 9.x.4-1: List of APIs for the predictive slice modification support feature

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ Service_Continuity_Requirement | Service_Continuity_Requirement | VAL server | Request / Response |
| SS_Slice_Modification_Notify | Slice_Modification_Notify | VAL server or VAL client | Notify |

#### 9.x.4.1	SS_ Service_Continuity_Requirement

##### 9.x.4.1.1	General

API description: This API enables the VAL server to communicate with the network slice capability enablement server for requesting a service continuity requirement over NSCE-S.

##### 9.x.4.1.2	Service_Continuity_Requirement

API operation name: Service_Continuity_Requirement

Description: Providing for Service_Continuity_Requirement to the NSCE server and receiving a response / result.

Known Consumers: VAL server.

Inputs: See subclause 9.x.2 (step 1)

Outputs: See subclause 9.x.2 (step 2)

#### 9.x.4.2	SS_ Slice_Modification_Notify

##### 9.x.4.2.1	General

API description: This API enables the network slice capability enablement server to communicate with the VAL server or VAL UE (NSCE client or VAL client) for notifying the slice modification to extend to the target service area.

##### 9.x.4.2.2	Slice_Modification_Notify

API operation name: Slice_Modification_Notify

Description: Notifying about the slice modification to extend to the target area.

Known Consumers: VAL server.

Inputs: -

Outputs: See subclause 9.x.2 step 6.