3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230110

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

Source:	Lenovo

Title:	Business models and relationships for NSCALE

Spec:	3GPP TS 23.435 v0.4.0

Agenda item:	8.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This contribution provides the NSCALE business relationships based on TR 23.700-99 Annex B.

2. Reason for Change

TS 23.435 skeleton includes clause 5 on the business models and relationships for NSCALE. In TR 23.700-99, such aspects were captured in Annex B. This contribution aims to specify the business models and relationships according to the pre-normative study.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v0.4.0.

* * * First Change * * * *

# 5	Business models and relationships for NSCALE

NSCE layer provides value added services to VAL customers, based on consuming 5GS services related to slicing (from OAM, 5GC) and based on interacting with the VAL UE side. The variety of services and the deployment aspects depend on the different assumptions for the slice owner / provider, the slice customer and the SEAL service provider. With respect to NSCALE, the NSCE server belongs to the SEAL provider (as stated in TS 23.434 clause 5) who can be either the MNO itself, or the vertical customer (e.g. factory owner, automaker X) or the edge/cloud provider (e.g. hyperscaler) who provides such platform services related to slice capability exposure to the vertical customer on top of the MNO. Thus, NSCE can play different roles based on the business models. For example, NSCE server can be:

-	deployed by NOP / MNO

-	deployed by an Edge / Cloud Provider, as a trusted 3rd party. In this case, it is assumed that the NSCE server is acting as a trusted AF to the 3GPP system. Such trustiness can be either assumed according to the service agreement between the SEAL provider and the MNO, or can be provided with existing security mechanisms, where the NSCE server acting as AF can be authenticated and authorized as any other NF as well, then it can be considered trusted (details can be found in TS 33.501 clauses 12 for AF/NEF security aspects and 13 for SBI security aspects).

-	deployed by a vertical industry, which can be the end slice customer

From business perspective, the following business model applies. In Figure 5-1, the different interactions among all the involved entities are provided. More specifically, in this model the end user is the consumer of the applications provided by the vertical/ASP and can have app-level service agreement with vertical/ASP(s).

The end user/UE also has a PLMN subscription arrangement with the MNO. The UE used by the end user is allowed to be registered on the MNO's network. MNO (via OAM) can have a slice SLA with the vertical / ASP, which is optional when vertical customer is the NSC. In addition, due to the involvement of a NSCE service provider, additional agreements can be possible between the NSCE server and VAL/ASP layer and the NOP/MNO(s):

-	the enablement service agreement between VAL/ASP layer and the NSCE service provider include the agreement on the value-added services, which in case on NSaaS these are services related to the consumed slice from NOP. So, the end customer (VAL) subscribes to NSCE server for receiving additional services for optimizing the slice utilization. In case that the NSCE server is a NSP towards VAL customer, then such agreement can relate to slice SLA (for the slice provided by the NSCE server). When the NSCE server is part of the vertical, the enablement service agreement between VAL/ASP layer and the NSCE service provider is internal, out of the SA6’s scope, whereas when the NSCE server is part of the MNO, the enablement service agreement between VAL/ASP layer and the NSCE service provider should still apply.

-	the service agreement between MNO and NSCE service provider is for consuming 5GS services (and being also authorized and trusted to provide additional services on top). Such agreement could be also a slice SLA for the scenarios when NSCE server is the NSC of the MNO (in NSaaS model). When the NSCE server is part of the MNO, the service agreement between MNO and NSCE service provider is internal, based on the MNO’s policy, whereas when the NSCE is part of the vertical, the enablement service agreement between VAL/ASP layer and the NSCE service provider should still apply.

. Hence,

Figure 5-1: Business relationships

* * End of Change * * * *