3GPP TSG-SA WG6 Meeting #52-Bis-e	S6-230132

e-meeting, 11th. – 20th. Jan 2023	(revision of S6-23xxxx)

Source:	China Mobile

Title:	Procedures for Multi-Network slice management capability

Spec:	3GPP TS 23.435

Agenda item:	8.10

Document for:	Approval

Contact:	Jiadi Fu, fujiadi@chinamobile.com

1. Introduction

This contribution proposes text for procedures of Multiple slices management capability.

2. Reason for Change

Procedures of Multiple slices management capability are needed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435.

* * * First Change * * * *

# 4	Overview

Clauses under this clause shall document the functionalities of the Network Slice Capability Exposure for Application Layer Enablement.

## 4.1	Slice API configuration and translation

This functionality is provided to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs. Slice API configuration and translation procedures are specified in clause 9.3.

## 4.2	Application layer network slice lifecycle management

Application layer network slice lifecycle management is provided to the VAL to better meet consumer's requirement without having to interact with 5GS frequently, based on network slice status collected from 5GS and QoE collected from application layer. Application layer network slice lifecycle management procedures are specified in clause 9.4.

## 4.3	Network slice optimization based on AF policy

Network slice optimization based on AF policy optimizes the network slice for the vertical applications by triggering the network slice modification exposed by EGMF defined in SA5. Network slice optimization based on AF policy procedures are specified in clause 9.5

## 4.4	Discovery of management service exposure

This network slice capability enablement feature supports the initial discovery of MnS for a given slice based on VAL server request, and the discovery of new/modified MnS with the required exposure (for example the permissions of the VAL server over the target MnS, e.g. read, write operations). This feature consists of two procedures, which are specified in clause 9.6:

- Discovery of management service capabilities and exposure level based on VAL server request

- Discovery and exposure of new or modified management service capabilities based on changes at OAM.

## 4.5	Information collection from NSCE server(s)

The network slice status collected by the NSCE server could be exposed to other NSCE server(s) if some agreement has been made. Information collection from NSCE server(s) procedures are specified in clause 9.8

## x	Multiple slices combined performance monitoring

- This functionality is provided to the VAL to manage network slice from PNI-NPN and PLMN through NSCE server. It enables the authorized third-party AF to manage its own PNI-NPN (public network integrated non-public network) and its private slice(s) in the PLMN in a combined manner, the NSCE server can make the performance monitoring and analysis for dedicated service from multiple slices of the PNI-NPN owner and offer consolidated service performance report to authorized third-party AF. Coordinated multiple slices performance monitoring capability procedures are specified in clause 9.x.

* * * Second Change * * * *

## 9.X	Multiple slices combined performance monitoring capability

### 9.x.1	General

Based on preferred QoS request from the vertical applications, the performance monitoring and analysis for dedicated service from multiple slices of a PLMN can be made and consolidated service performance report can be offered to authorized third-party AF.

### 9.x.2	Procedure

Figure 9.x.2.1 illustrates the procedure of multiple slices combined performance monitoring capability from VAL server to NSCE server.

Pre-conditions:

1.	The network slice enabler layer is capable to interact with PLMN 5GC or OAM system to handle slices of PLMN and its PNI-NPNs.

2.	PNI-NPNs are deployed as network slices of the PLMN.

Figure 9.x.2.1: Multiple slices combined performance monitoring process

1.	The VAL server initiates multiple slices combined performance monitoring request towards the NSCE server. The request includes VAL server ID, VAL service ID, Network ID. The message also includes application key performance indicator (such as end-to-end delay, throughput, QoE data from UE, etc. ) list, monitoring period and monitoring zone.

2.	Upon receiving the request from the VAL server to manage the network slice performance monitoring report, the NSCE server makes authentication and authorization of the VAL server and if VAL server is not authorized, the NSCE server replies with failure response.

3.	The NSCE server makes lookup of VAL service ID that received from VAL server and get related slice identities (S-NSSAIs allocated in each network) and retrieves the network slice related status information of PNI-NPN slices, such as performance measurements in TS 28.552 [x] and analytics data as specified in 3GPP TS 28.104 [y].

4.	The NSCE server retrieves the network slice related performance information from PLMN 5GC networks (e.g. NWDAF) or OAM, For OAM system, the services defined in clause 11.3 of TS 28.532 [7] and clause 5 of TS 28.552 [x] can be utilized. For CN functions, the services of Nnwdaf_AnalyticsInfo service defined in clause 7.3 of TS 23.288 [4], performance measurements in TS 28.552 [x] and analytics data as specified in 3GPP TS 28.104 [y] can be utilized. The PLMN operator can choose to deploy NSCE-Server acting as the entry of PLMN capability exposure which can be optional.

5.	The NSCE server sends Service QoE data request to NSCE clients in monitoring zone which includes VAL service ID to request the QoE data of specific application.

6.	The NSCE client sends the requested QoE data (e.g. MOS, stalling ratios, etc.) response to NSCE server.

Editor's Note: The mechanism defined in EVEX study will be reused in Data collection from VAL users. The services and APIs to be utilized is FFS.

7.	The NSCE server verifies and analyses performance data of network slice instance that received from both PNI-NPN and PLMN networks as well as QoE data provided by VAL client, then NSCE server make consolidation performance report for specific application among different kinds of networks in specific period of time/location zone.

8.	The NSCE server sends the multiple slices combined performance monitoring response towards VAL server.

### 9.x.3	Information flows

#### 9.x.3.1	General

The following information flows are specified:

-	Multiple slices combined performance monitoring request and response.

#### 9.x.3.2	Multiple slices combined performance monitoring request

Table 9.x.3.2-1 describes information elements for the Multiple slices combined performance monitoring request from the VAL server to the NSCE server.

Table 9.x.3.2-1: Multiple slices combined performance monitoring request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Performance and analytics monitoring metrics | M | The information of performance and analytics monitoring. |
| >VAL service identity | M | Identifier of the VAL service application to be monitored. |
| > Start Time | M | The start time point of the performance monitoring. The |
| > End Time | O | The end time point of the performance monitoring, If the EndTime IE is not included in the request, the End Time is Start Time plus default time duration configured in NSCE server. |
| > Monitoring Zone | O | The preferred location area of the performance monitoring. |
| > PerfList | M | The list of performance to be monitored. |
| >> PerfName | M | The name of the performance to be reported, e.g., the end to end round-trip time or the end to end network slice load. |
| > Requested network identity | O | Indication of the network(s) which are requested. |
| > Requested S-NSSAI(s) | O | Indication of the S-NSSAI(s) which are requested. |

#### 9.x.3.3	Multiple slices combined performance monitoring response

Table 9.x.3.3-1 describes the information elements for the Multiple slices combined performance monitoring response from the NSCE server to the VAL server.

Table 9.x.3.3-1: Multiple slices combined performance monitoring response

| Information element | Status | Description |
| --- | --- | --- |
| status | M | The result of Multiple slices performance monitoring request. |
| >Successful response | CM (NOTE 1) | Indicates that the Multiple slices performance monitoring request is successful. |
| >>VAL service identity | M | Identifier of the VAL service of which the performance monitoring results are reported |
| >>StartTime | M | The start time point of the performance monitoring |
| >>EndTime | M | The end time point of the performance monitoring, If the EndTime IE is not included in the request, the End Time is Start Time plus default time duration configured in NSCE server. |
| >>Monitoring Zone | O | The location area of the performance monitoring. |
| >> PerfList | M | The list of performance to be monitored. |
| >>>PerfName | M | The name of the performance to be reported. |
| >>>PerfValue | M | The corresponding value of the monitored performance. |
| >Failure response | CM (NOTE 2) | Indicates that the Multiple slices performance monitoring request is failed. |
| >>Cause | M | Indicates the cause of performance monitoring request failure. |
| NOTE 1 Information element of Successful reponse shall be present when Multiple slices performance monitoring report exposure monitoring request status is successful. NOTE 2 Information element of Failure response and Cause shall be present when Multiple slices performance monitoring report exposure request status is failed. |  |  |

### 9.x.4	APIs

#### 9.x.4.1	General

Table 9.x.4.1-1 illustrates the API for Multiple slices combined performance monitoring .

Table 9.x.4.1-1: Multiple slices combined performance monitoring

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NSCE_MultiSlices_Combined_PerfMonitoring | MultiSlices_Combined_PerfMonitoring_Request | VAL server | Request /Response |

#### 9.x.4.2	SS_NSCE_MultiSlices_Combined_PerfMonitoring API

API operation name: SS_NSCE_MultiSlices_Monitoring_Report_Exposure_Request

Description: The consumer requests to monitor Multiple slices related performance and analytics for dedicated service.

Inputs: See clause 9.x.3.2.

Outputs: See clause 9.x.3.3.

See clause 9.x.2 for details of usage of this operation.

* * * End of Changes * * * *