3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230133r1

e-meeting, 11th – 20th January 2023

Source:	Deutsche Telekom

Title:	Pseudo-CR on AF policies update

Spec:	3GPP TS 23.435 v.0.4.0

Agenda item:	8.10

Document for:	Approval

Contact:	Vasil Aleksiev <vasil.aleksiev@magenta.at>

1. Introduction

Based on the FS_NSCALE study TR23.700-99 there is network slice optimization based on AF policies as part of TS23.435. This pCR proposes further optimizations of AF policies.

2. Reason for Change

The existing policies are listed in step 1 under figure 9.5.2.1 AF policy provisioning ( to modify a slice based on the performance metric from OAM, based on the network slice load predictions from NWDAF, based on a specific time period). The existing policies do not take in mind the location on which a slice is set. According to TS 28.541 there is coverageArea of a slice profile and respectively one network slice could have several coverage areas defined. TS 23.435 also specifies slice coverage area as the area where the network slice is available.

If the network slice has big coverage area it could be the case that VAL server would have specific requirements to a certain area of interest of that slice. Area of interest is already introduced in section 9.3.4 of TS 23.435.

This pCR proposes to introduce area of interest also for the existing policies, that can be requested from the AF/VAL server.

In solution 20 of the TR 23.700-99 examples of trigger events and expected actions are provided including specific time period. There is already introduced policy of time period with specific time where specific network slice capacity is expected. It is not clear how this slice capacity is expressed, that is why modification is proposed where QoS per UE is introduced. This is existing slice profile parameter used in OAM and also a way to introduce simple requirement from AF/VAL server. New policy is also proposed to be used - to provide separately the need for average QoS per UE.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435 v.0.4.0.

Changed in r1:

Adding “Area of interest” to all the policies. In initial version it was only present in the policy profile.

“Policy of needed average QoS per UE” is changed to “Policy of needed minimum QoS per UE”.

* * * First Change * * * *

#### 9.5.3.2	AF policy provisioning request

Table 9.5.3.2-1 describes information elements for the AF policy provisioning request from the VAL server to the NSCE server.

Table 9.5.3.2-1: AF policy provisioning Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Requested slice information | O | Indication of the slice which is requested. |
| Requested DNN | O | Indication of the DNN which is requested. |
| Indicator of policy harmonization | O | Indicating whether the policy harmonization is requested. |
| Policy | O | The policy profile is defined in Table 9.5.3.2-2. The supported AF policies are listed in Table 9.5.3.2-3 to Table 9.5.3.2-5. |

Table 9.5.3.2-2 describes Policy profile of the AF policy provisioning request.

Table 9.5.3.2-2: Policy profile

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | The name of AF policy. |
| Area of interest | M | The geographical or service area for which the policy profile applies. |
| Trigger event | M | Indicating the event that should be monitored, associated with the threshold of the monitored parameter. |
| Expected action | M | Indicating the excepted actions associated with the updated parameter. |
| Lifetime or number of events | M | Time duration or number of events for which the policy is valid. |

Editor's note: Whether more policies consisting of trigger events and actions could be supported is FFS.

Table 9.5.3.2-3 to Table 9.5.3.2-5 list the supported policies.

Table 9.5.3.2-3: Policy of Max number of PDU sessions/ max number of UEs

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Max number of PDU sessions/ max number of UEs |
| Area of interest | M | The geographical or service area for which the policy profile applies. |
| >Trigger event | M | Threshold PDU sessions/ Threshold UEs (reached utilization of available capacity in %) |
| >Expected action | M | Modification of PDU sessions / max number of UEs (step for increase in %) |
| Lifetime or number of events | M | Time duration |

Table 9.5.3.2-4: Policy of Network slice load prediction

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Network slice load prediction |
| Area of interest | M | The geographical or service area for which the policy profile applies. |
| >Trigger event | M | Network Slice load predictions from NWDAF as defined in TS 23.288[4] clause 6.1.1 (exceeding utilization of available capacity in %) |
| >Expected action | M | Modification of related network slice parameters (step for increase in %) |
| Lifetime or number of events | M | Time duration |

Table 9.5.3.2-5: Policy of Time period and average QoS per UE

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Time period and average QoS per UE |
| Area of interest | M | The geographical or service area for which the policy profile applies. |
| >Trigger event | M | Time/day configuration where specific network slice capacity/QoS per UE is expected, average QoS requested |
| >Expected action | M | Modification of slice capacity (increase or decrease in %) |
| Lifetime or number of events | M | Time duration |

Table 9.5.3.2-x: Policy of needed minimum QoS per UE

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Minimum QoS per UE |
| Area of interest | M | The geographical or service area for which the policy profile applies. |
| >Trigger event | M | Time/day where average QoS per UE is expected, minimum QoS requested |
| >Expected action | M | Modification of slice capacity to have the needed QoS per UE |
| Lifetime or number of events | M | Time duration |

* * * End of Change * * * *