3GPP TSG-SA WG6 Meeting #52-Bis-e	S6-230134

e-meeting, 11th. – 20th. Jan 2023	(revision of S6-23xxxx)

Source:	China Mobile

Title:	Procedures for multiple slices resource optimization capability

Spec:	3GPP TS 23.435

Agenda item:	8.10

Document for:	Approval

Contact:	Jiadi Fu, fujiadi@chinamobile.com

1. Introduction

This contribution proposes text for procedures of multiple slices resource optimization capability.

2. Reason for Change

Procedures of multiple slices management capability are needed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.435.

* * * First Change * * * *

# 4	Overview

Clauses under this clause shall document the functionalities of the Network Slice Capability Exposure for Application Layer Enablement.

## 4.1	Slice API configuration and translation

This functionality is provided to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs. Slice API configuration and translation procedures are specified in clause 9.3.

## 4.2	Application layer network slice lifecycle management

Application layer network slice lifecycle management is provided to the VAL to better meet consumer's requirement without having to interact with 5GS frequently, based on network slice status collected from 5GS and QoE collected from application layer. Application layer network slice lifecycle management procedures are specified in clause 9.4.

## 4.3	Network slice optimization based on AF policy

Network slice optimization based on AF policy optimizes the network slice for the vertical applications by triggering the network slice modification exposed by EGMF defined in SA5. Network slice optimization based on AF policy procedures are specified in clause 9.5

## 4.4	Discovery of management service exposure

This network slice capability enablement feature supports the initial discovery of MnS for a given slice based on VAL server request, and the discovery of new/modified MnS with the required exposure (for example the permissions of the VAL server over the target MnS, e.g. read, write operations). This feature consists of two procedures, which are specified in clause 9.6:

- Discovery of management service capabilities and exposure level based on VAL server request

- Discovery and exposure of new or modified management service capabilities based on changes at OAM.

## 4.5	Information collection from NSCE server(s)

The network slice status collected by the NSCE server could be exposed to other NSCE server(s) if some agreement has been made. Information collection from NSCE server(s) procedures are specified in clause 9.8

## x	Multiple slices coordinated resource optimization

This functionality is provided to the VAL to monitor the slice usage status of multiple slices (PNI-NPN slice(s) and its private slice in the PLMN) of the PNI-NPN owner in a combined manner and make resource adjustment between different slices to realize optimized and efficient resource usage among multiple slices. Coordinated PLMN and PNI-NPN slice resource optimization procedures are specified in clause 9.x.

* * * Second Change * * * *

## 9.X	Multiple slices coordinated resource optimization

### 9.x.1	General

Based on preferred QoS request from the vertical applications, the performance monitoring of multiple networks and resource adjustment between different slices can be made to realize optimized and efficient resource usage among multiple slices.

### 9.x.2	Procedure

Figure 9.x.2.1 illustrates the procedure of multiple slices coordinated resource optimization service from VAL server to NSCE server.

Pre-conditions:

1.	The network slice enabler layer is capable to interact with PLMN 5GC or OAM system to handle slice of PLMN and its PNI-NPNs.

2.	PNI-NPNs are deployed as network slices of the PLMN.

Figure 9.x.2-1 Multiple slices coordinated resource optimization process

1.	The VAL server initiates multiple slices coordinated resource optimization request towards the NSCE server. The request includes VAL server ID, VAL service ID, Network ID. The message also includes preferred optimization zone.

2.	Upon receiving the request from the VAL server to make the network slice resource optimization, the NSCE server makes authentication and authorization of the VAL server and if VAL server is not authorized, the NSCE server replies with failure response.

3.	The NSCE server makes mapping from VAL service ID that received from VAL server to slice identities (S-NSSAIs allocated in multiple networks) and retrieves the network slice related status information from 5GC or OAM of PLMN for PNI-NPN slices, such as NF(s) load in 5GC and network utilization in access network as defined in TS 28.535 [x].

4.	The NSCE server retrieves the network slice related status information from PLMN networks from 5GC or OAM of PLMN networks for PLMN slices, such as NF(s) load in 5GC and network utilization in access network as defined in TS 28.535 [x]. The PLMN operator can choose to deploy NSCE-Server acting as single entry of PLMN capability exposure which can be optional.

5.	The NSCE server verifies and analyses status data of network slices instance as well as network slice performance monitoring response (optional), then NSCE server makes resource optimization decision among different kinds of slices in specific location zone.

6-7.	The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [8] to PNI-NPN slice. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s).

8-9.	The NSCE server determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi request as specified in TS 28.531 [8] to PLMN slice. Based on decision made by NSCE server, the network slice management entity (such as NSMF) performs the corresponding operation(s). The PLMN operator can choose to deploy NSCE-S acting as single entry of PLMN capability exposure which can be optional.

10.	The NSCE server sends the multiple slices coordinated resource optimization response towards VAL server.

### 9.x.3	Information flows

#### 9.x.3.1	General

The following information flows are specified:

-	Multiple slices coordinated resource optimization request and response.

#### 9.x.3.2	Multiple slices coordinated resource optimization request

Table 9.x.3.2-1 describes information elements for the multiple slices coordinated resource optimization request from the VAL server to the NSCE server.

Table 9.x.3.2-1: Multiple slices coordinated resource optimization request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| VAL service identity | M | Identifier of the VAL service application to be monitored |
| Optimization Zone | O | The preferred location area of the performance monitoring. |
| Requested network identity | O | Indication of the network(s) which are requested. |
| Requested S-NSSAI(s) | O | Indication of the S-NSSAI(s) which are requested |

#### 9.x.3.3	Multiple slices coordinated resource optimization response

Table 9.x.3.3-1 describes the information elements for the multiple slices coordinated resource optimization response from the NSCE server to the VAL server.

Table 9.x.3.3-1: Multiple slices coordinated resource optimization response

| Information element | Status | Description |
| --- | --- | --- |
| status | M | The result of multiple slices resource optimization request. |
| Successful response (NOTE) | CM (NOTE 1) | Indicates that the multiple slices resource optimization request is successful. |
| > VAL service identity | M | Identifier of the VAL service application to be monitored. |
| Failure response (NOTE) | CM (NOTE 2) | Indicates that the multiple slices resource optimization request is failed. |
| > Cause | M | Indicates the cause of multiple slices resource optimization request failure. |
| NOTE 1: Information element of Successful reponse shall be present when multiple slices resource optimization request status is successful. NOTE 2 Information element of Failure response and Cause shall be present when multiple slices resource optimization request status is failed. |  |  |

### 9.x.4	APIs

#### 9.x.4.1	General

Table 9.x.4.1-1 illustrates the API for multiple slices coordinated resource optimization.

Table 9.x.4.1-1: Multiple slices coordinated resource optimization

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NSCE_MultiSlices_Optimization | MultiSlices_Optimization | VAL server | Request /Response |

#### 9.x.4.2	SS_NSCE_MultiSlices_Optimization operation

API operation name: SS_NSCE_MultiSlices_Optimization_Request

Description: The consumer subscribes for multiple slices coordinated resource optimization .

Inputs: See clause 9.x.3.2.

Outputs: See clause 9.x.3.3.

See clause 9.x.2 for details of usage of this operation.

* * * End of Changes * * * *