3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230181

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

Source:	Samsung

Title:	Pseudo-CR on adding PIN profile details

Spec:	3GPP TS 23.542 V0.0.0

Agenda item:	8.19

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

3GPP TR 23.700-78 studied the key issues related to application layer support for Personal IoT networks (PIN). As part of the key issue related to the PIN management it is identified to maintain the information related to the PIN at the PIN entities.

2. Reason for Change

This pCR brings in the list of configuration information related to the PIN that needs to be maintained at each of the PIN entities like PIN server, PEMC, PEGC etc., As part of the normative Annex the PIN configuration data is captured.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.542 V0.0.0.

* * * First Change * * * *

#### 8.2.2.1	PIN Profile in a PIN

PIN profile includes information about the static data needed for configuration for the Personal IoT networks.

Table 8.2.2.1-1: PIN Profile

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y |
| Duration | Specifies the time period of how long the PIN can be active | Y | Y | Y |
| Maximum number of PIN elements | Maximum number of PIN elements allowed to join the PIN | Y | Y | N |
| PIN service | List of service that a PIN can provide, including the PINE service or the service that can provided by application client on PINE: > PIN service Provider Identifier > PIN service type > PIN service Feature | Y | Y | N |
| PEMC list | The list of identifiers of the PIN elements which can be allowed to take the role as PEMC (e.g.: PIN client ID, UE GPSI etc.,) and also it contains whether the role is primary or secondary > PIN Element ID > Role | Y | Y | Y |
| PEGC ID list | The list of identifiers of the PIN elements which can be allowed to take the role as PEGC (e.g.: PIN client ID, UE GPSI etc.,) > PIN Element ID | Y | Y | Y |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y |
| PIN server Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y |
| PIN Elements List | List of PIN elements which can be allowed to join the PIN > PIN element ID | Y | Y | Y |

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>