3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230269 REV 2

e-meeting, 11th – 20th January 2023

Source:	Convida Wireless LLC

Title:	PIN modification procedures

Spec:	3GPP TS 23.542 V0.0.0

Agenda item:	8.19

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This contribution adds PIN modification procedures to TS 23.542 based on solutions described in sections 7.3.2.3.7 and 7.10.

Rev 1 indicates new text in track marks, removes comments and adds a single sentence in 8.5.3.1 to describe the clause.

Rev 2 adds an EN and changes IE nomenclature for clarity, removes timestamp

2. Reason for Change

Adds PIN modification procedures involving authorized administrators and PEMC detecting PEGC unavailability.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.542 V0.0.0.

* * * First Change * * * *

### 8.5.3	PIN modification

#### 8.5.3.1	General

This clause describes PIN modification functionality.

#### 8.5.3.2	Procedures

##### 8.5.3.2.1	General

##### 8.5.3.2.2	PIN modification after local PEMC failure

Figure 8.5.3.2.2-1 describes the PIN modification procedure to perform a PEMC role change due to the failure of the PEMC. An authorised administrator is the owner of the PIN and accesses PIN configuration using an application on a UE, which is one of the PEMC for the PIN. The authorised administrator can manage the PIN locally or through the 5G network. This procedure describes a PEMC (e.g. an authorized administrator on a UE) managing the PIN remotely via the 5G network.

The procedure may used e.g. when a PEMC is available on a UE for PIN management by an authorised administrator. When there is a local PEMC failure, the authorized administrator can be enabled to manage the PIN remotely, via the 5G network, using the following steps.

Pre-conditions:

1.	The PIN server has authorised the creation of the PIN.

2.	The authorised administrator is the owner of the PIN and has created the PIN.

3.	The authorised administrator configures PEMC2 to provide PIN management for the PIN. PEMC2 becomes the active PEMC and UE/PEMC1 is the inactive PEMC.

4.	PEMC2, PEGC, PINE-1, PINE-2, and the UE/PEMC1 are members of the PIN. PINE-1 has PEMC capability.

5.  The authorised administrator leaves the local area of the PIN (e.g. in a home) and is able to access the PIN remotely through the 5G network. As a result, the authorised administrator is able to manage the PIN through the 5G network.

Figure 8.5.3.2.2-1: PIN Modification after local PEMC failure

1.	 UE/PEMC1 receives a notification from PEGC that there is a communication failure with PEMC2.

2.	An authorised administrator on UE/PEMC1 sends a PIN_configuration_request to the PIN server through the 5G network. The request includes the security credentials of the authorised administrator, the UE ID, the PIN ID, PIN member ID, authorization type indicating the role change, and a request that PINE-1 be assigned the new PEMC.

3.	The PIN server processes the modification request and checks if the authorised administrator is allowed to modify the PIN. The PIN server verifies PINE-1 has the capability to serve as a PEMC using information in the PIN profile.

4. 	If the authorised administrator is allowed to perform PIN modification, the PIN server sends a PIN_management_request to PINE-1 to assign PINE-1 as the new PEMC and provides PIN profile and dynamic profile information to PINE-1.

Editor’s Note: Usage of PIN profile information and PIN dynamic profile information in the information flows,  and the procedures where each of these IEs are applicable are FFS.

5.	PINE-1 returns an accept response in the PIN_management_response to the PIN server.

6.	The PIN server notifies the other members of the PIN that PINE-1 will be the new PEMC for the PIN and updates the PIN profile and dynamic profile information.

7.	The PIN server sends a PIN_configuration_response to the PIN_configuration_request with updated information for the PIN profile and the dynamic profile information to the UE/PEMC1.

8.	PIN communications resume with PINE-1 serving as the new PEMC.

* * * Next Change * * * *

##### 8.5.3.2.3	PIN modification with PEGC role change

Figure 8.5.3.2.3-1 describes a PIN modification procedure to perform a PEGC role change due to the unavailability of the PEGC. This procedure describes a PEMC detecting the unavailability of a PEGC (e.g. PEGC leaves the local service area of the PIN) and performing a PIN modification with the PIN Server to assign a new PEGC. As part of PIN management, a PEMC subscribes to receive periodic notifications from PEGCs to ensure PIN routing is available for members of the PIN at all times. If a PEMC does not receive the periodic notifications from the PEGC, then the PEMC needs to assign a new PEGC or request the PIN server to assign the new PEGC.Pre-conditions:

1.	The PIN server has authorized the PEMC to create PINs.

2.	The PEMC creates a local PIN with members: PEMC, UE serving as PEGC, PINE1, and PINE2.

3.	The PEMC maintains a PIN profile with information on the capabilities of each PIN member.

4.	PINE2 is a PIN member that also has gateway capability.

Figure 8.5.3.2.3-1: PIN Modification due to PEGC unavailability

1.	PEMC subscribes to receive periodic notifications from PEGC to monitor the availability of PIN routing service provided by PEGC.

NOTE:	The subscription/ notification is for further development.

Editor’s Note: How the PEMC detects the unavailability of PEGC is FFS.

2.	PEGC leaves the local coverage area of the PIN, e.g., leaves the home, and is not available to route PIN communications.

3.	PEMC does not receive a notification from PEGC at the configured interval and determines that PEGC is no longer providing PIN routing capability.

4.	PEMC sends a PIN_configuration_request to the PIN server to select a new PEGC. The request includes the PIN ID, the PEMC ID, the PEGC ID, authorization type indicating the role change, the ID of a PIN member that can serve as the new PEGC (e.g. PINE2), and a timestamp.

5.	The PIN server considers which member of the PIN can serve as the new PEGC, including the PIN member the PEMC provided, and selects a PIN member to serve as the new PEGC. The PIN server sends a PIN_management_request to PINE-2 with PIN profile and dynamic profile information. The dynamic profile information includes PIN traffic routing rules that PINE2 would need to make routing decisions.

6.	PINE-2 sends a PIN_management_response accepting to serve as the new PEGC.

7.	The PIN server sends a PIN_configuration_response with PIN profile and dynamic profile information to the PEMC with the status of the request, the ID of the new PEGC, and PIN traffic routing rules. The PIN server response triggers the PEMC to notify other PIN members of the PEGC role change.

8.	PEMC notifies the other members of the PIN that PINE2 will serve as the new PEGC. The PEMC includes PIN profile and dynamic profile information that includes traffic routing rules applicable to each member.

* * * Next Change * * * *

- 8.5.3.3	Information flows.

- 8.5.3.3.1	General

- 8.5.3.3.2	PIN_configuration_request

Table 8.5.3.3.5-1 shows the informational elements of the PIN configuration request sent by a PIN Element to the PIN server to obtain authorization for the modification of a PIN.

Table 8.5.3.3.2-1: PIN configuration request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN |
| Requester PINE ID | M | The identifier of the PIN Element making the request |
| Authorization type | M | Request for the authorization to modify the configuration of a PIN: PEMC role change, PEGC role change |
| PINE ID of predecessor in the role | M | PINE ID of the element serving in the indicated role and needs to be changed e.g. due to being unavailable. |
| PINE ID of proposed successor in the role | O | PINE ID of the element proposed to assume the role |

- 8.5.3.3.3	PIN_configuration_response

Table 8.5.3.3.3-1 shows the informational elements of the PIN configuration response provided by the PIN server to authorize the modification of a PIN.

Table 8.5.3.3.3-1: PIN configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Response | M | The response (authorize or not authorize) from the PIN Server. |
| PIN profile information | M | The IEs from the PIN profile information that the PIN Server has updated for modifying the PIN |
| Dynamic PIN profile information | M | The IEs from the dynamic PIN profile information that the PIN Server has authorized for modifying the PIN |

- 8.5.3.3.4	PIN_management_request

Table 8.5.3.3.4-1 shows the informational elements of the PIN management request sent to PIN elements to make changes to the configuration of the PIN.

Table 8.5.3.3.4-1: PIN management request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN |
| Requestor ID | M | The identifier of the PIN Server or PIN Element making the request |
| Modification type | M | Request for the modification of the PIN: PEMC assignment, PEGC assignment |
| Dynamic PIN profile information | M | IEs from the dynamic PIN profile information that the PINE needs to operate in the new role |

- 8.5.3.3.5	PIN_management_response

Table 8.5.3.3.5-1 shows the informational elements of the PIN management response received from the PIN element to the PIN modification request.

Table 8.5.3.3.5-1: PIN management response

| Information element | Status | Description |
| --- | --- | --- |
| Response | M | The response (accept or deny) from the PIN Element to the PIN management request. |