3GPP TSG-SA WG6 Meeting #52	S6-223097

Toulouse, France, 14th – 18th November 2022	(Revision of S6-22xxxx)

Source:	InterDigital

Title:	Discussion on DAA for the study FS_eUASAPP

Spec:	N/A

Agenda item:	9.9

Document for:	Discussion and Agreement

Contact:	Atle Monrad, atle.monrad@InterDigital.com

# Introduction

This paper provides information about aspects of Detect And Avoid for SA2 and SA6 in Rel-18, and clarify the scope of SA6’s work. The paper proposes that SA6 come to a common understanding about the introduction of solutions and way forward for conclusions on DAA in SA6.

# 2. Discussion

The Objective of the SID for FS_eUASAPP (SP-220465) is shown in the text box below.

Observation #1:

The objective explicitly identify DAA as functionality where potential solutions shall be studied and enhancements to SEAL shall be considered.

The progress of DAA in SA2 shall be considered for our work.

## 4	Objective

The SA6 objectives of this study item include the following:

1)	analyze requirements in 3GPP TS 22.125 and further identify key issues, develop corresponding architectural requirements and potential enhancements to the application layer architecture as required, including:

a)	support for multiple USS/ UTMs;

b)	support for monitoring by USS/UTM of C2 communication QoS and communication link status including for UAV moving between EPS and 5GS, see 3GPP TS 22.125 clause 6.2;

c)	support for service restriction for UEs onboard of UAV, see 3GPP TS 22.125 clause 6.3;

d)	usage of PC5 communication between UAVs or UAV and UAV-C; and

e)	support for Detect And Avoid (DAA);

2)	identify potential solutions as required, including the information flows and the APIs satisfying the architectural requirements and enhancements identified in bullet 1) above; and

3)	identify potential improvements to SEAL based on the architectural requirements and enhancements identified in bullet 1) above.

For some items, e.g. d) and e) above, SA6 is dependent on progress and decisions by SA2.

Key Issue #4 from TR 23.700-55 v 1.1.0 is shown in the text box below.

Observation #2:

The key issue highlight that UAE-layer and SEAL shall be studied with regards to DAA.

The linkage to work on DAA in SA2 is also highlighted in the key issue.

## 4.4	Key issue #4: Support of detect and avoid services and applications

Development of the Support of Detect and Avoid Mechanism in 3GPP system is being studied in 3GPP TR 23.700-58 [6]. The 5GC enhancement work for DAA includes services for network assisted DAA and direct DAA via PC5.

Considering the stage 1 requirements and also 5GC enhancements for DAA, it is required to study the aspects that UAE layer and SEAL can be supported for DAA.

Further study is also required for UAE layer to support DAA scenarios where UAVs belong to multiple PLMNs.

It is required to study the following:

a)	Whether and how the UAE layer and/or SEAL services can be enhanced to support DAA services and applications for collision avoidance considering the Stage 1 requirements.

b)	How the UAE layer can support DAA scenarios where UAVs belong to multiple PLMNs.

NOTE:	Solutions for DAA will be coordinated with the conclusions of the work outlined by 3GPP TR 23.700-58 [6] and based on requirements for DAA specified in 3GPP TS 22.125 [2].

Key Issue #3 from TR 23.700-58 v 1.1.0 is shown in the text box below.

Observation #3:

The key issue highlights that support of DAA mechanisms in 3GPP shall be studied with regards to DAA. Various technologies and solutions are mentioned (ProSe, C-V2X, Broadcast, Groupcast, Unicast) and NW-assisted and direct solutions.

## 5.3	Key Issue #3 - Support of Detect and Avoid Mechanism in 3GPP system

This key issue focuses on the support of detect and avoid mechanisms in the 3GPP system, based on requirements for DAA defined in TS 22.125 [4]. The following aspects are to be considered:

-	Whether any architectural modifications are required, and what they are, with respect to the current solutions using PC5 (e.g. ProSe, C-V2X), to support direct UAV to UAV communication for the purpose of DAA:

-	For solutions based on direct UAV-to-UAV communication mechanisms, whether only broadcast communications are required, or whether also unicast and/or groupcast communications are required, and whether and how current solutions can be re-used or extended;

-	Whether network-assisted (ground based) DAA solutions are applicable, what solutions may be applicable, and whether a combination of network-assisted and direct solutions should be defined;

-	What assumptions need to be made about considering security solutions outside of 3GPP, e.g. similar to application layer security as in C-V2X. It is assumed security discussion is to be coordinated with SA WG3.

Solutions to Key Issue #3 from TR 23.700-58 v 1.1.0 is shown in the text box below.

Observation #4:

The key issue #3 (Support for DAA in 3GPP) has three proposed solutions in the TR.

## Contents

6.2	Solution #2: Network-assisted DAA	15

6.5	Solution #5: U2X for support of Broadcast Remote ID and direct DAA via PC5	21

6.7	Solution #7: Ground-based DAA for an Area	31

The conclusion of key issue #3 (Support for DAA in 3GPP) in TR 23.700-58 was approved at SA2#153-e and is shown in the text box below.

Observation #5:

## 8.3	Conclusions for Key Issue #3 - Support of Detect and Avoid Mechanism in 3GPP system

It is proposed to progress to normative phase the following:

-	a mechanism for DAA leveraging U2X (UAV-to-everything), with the architectural enhancements identified in solution #5, with the following principles:

-	The detection and resolution of collisions is locally performed between UAVs using direct UAV to UAV communication over PC5.

-	The USS can (optionally) be informed of the collision situation.

NOTE 1:	The UAV may inform the USS of the collision situation over application layer communication. No normative work in SA WG2 is expected to enable this.

-	Both unicast and broadcast mode direct communication over PC5 is supported for DAA.

-	Additionally, may support network-assisted (ground based) DAA solution #7. It is applicable for a specific area, such as a stadium or arena where drones are used.

NOTE 2:	The content of "local policies for DAA" in Solution #7 and how it is communicated between UAV, UAV-C and AAM is out of 3GPP scope.

NOTE 3: 	Whether the UAV UEs served by different PLMNs and with subscriptions to different PLMNs is supported depends on the Rel-18 NR sidelink RAN study conclusion.

It is assumed that security of the U2X solution will be addressed by SA WG3.

Editor's note:	It is FFS how to support the DAA if the UAV does not have ProSe/U2X capability.

The approved conclusion by SA2 frame their work. Note the (optional) information from the client to the USS and the open item for UAVs without ProSe/U2X capability.

# 3. Proposal

To help with the progression of Key Issue #4, it is proposed that SA6 agree to the following.

Proposed Agreement #1:

Introduction of proposed solutions for DAA in TR 23.700-55.

Proposed Agreement #2:

Introduction of a standardized way for the UAE-layer to assist the DAA application with the policy for DAA handling (e.g., USS notification) provided by the USS to the application client. This may also be useful for coordination and alignment of DAA with SA2 conclusion on U2X (PC5) layer based solution for detection/deconfliction.

Proposed Agreement #3:

Introduction of a standardized way to perform coordination of the UAE-layers support for DAA with other functions of the UAE-layer. In particular, a standardized way to utilize SEAL services and coordinate network-based location tracking in relation with client DAA condition reporting.

Proposed Agreement #4:

Introduction of a standardized way to inform the application client if an emergency DAA related situation needs to be handled by the UAE Client (e.g., loss of contact with the serving USS).