3GPP TSG-SA WG6 Meeting #52	S6-223099

Toulouse, France, 14th – 18th November 2022	(revision of S6-22xxxx)

# Source:		InterDigital

# Title:		Overall evaluation and conclusion for KI #4

Spec:		3GPP TR 23.700-55

# Agenda Item:	9.9

# Document for:	Approval

Contact:		Atle Monrad (atle.monrad@interdigital.com)

1. Introduction

Based on the proposed solutions and their evaluations, an overall evaluation and conclusion is proposed introduced for KI 4.

It is proposed to add the following in 3GPP TR 23.700-55 v 1.1.0.

* * * *   Start of changes   * * * *

### 9.2.5	Evaluation of key issue #4: Support of detect and avoid services and applications

Key Issue #4 outlines the following to be investigated:

a)	Whether and how the UAE layer and/or SEAL services can be enhanced to support DAA services and applications for collision avoidance considering the Stage 1 requirements.

b)	How the UAE layer can support DAA scenarios where UAVs belong to multiple PLMNs.

Solution #4 addressesbullet a) including handling of management of the policy for DAA and provision of services from SEAL.

Bullet b) is also supported by Solution #4 by the fact that both the UAS application-layer and the UAE-layer both are PLMN-agnostic.

Solution #4 is selected as the basis for normative work, based on the following combined UAE layer assisted / USS controlled principles:

1)	The DAA capability of the UAE client is provided to the server.

2)	The UAS client, the UAE client and the UAE server are provided with DAA policies from the USS.

3)	The UAS application layer is always in control of the decision for DAA.

4)	If a DAA event is detected, the UAS client can notify the UAE layer, and the UAE server can be provided with DAA client information indicating a detected or resolved flight path conflict with one or more UAVs in proximity.

The UAE server records the DAA event, requests UAE client location information from the SEAL location services, time-stamps the event and forwards the event to the UAS server.

The UAE client may inform the UAS client about lack of connectivity with the UAS server as an emergency DAA situation.

5)	If a DAA event is detected, the UAS server can notify the UAE layer, and the UAE server can be provided with DAA client information indicating a detected flight path conflict with one or more UAVs in proximity.

The UAE server records the DAA event and provides real-time connection status monitoring and location reporting to secure that the UAE-layer is not reporting deviations due to deliberate modification in flight path due to DAA.

# 10	Conclusions

## 10.1	Architecture enhancements

There is no architecture enhancement described in this Technical Report, see subclause 9.1.

## 10.2	Solutions

The conclusions to Key Issue #1, is described in clause 9.2.2. This refer to selection of Solution #1 and solution #2 as the basis for normative work.

The conclusions to Key Issue #2 is described in clause 9.2.3. This refer to selection of Solution #1 and solution #2 as the basis for normative work.

The conclusions to Key Issue #3 is described in clause 9.2.4. This refer to selection of Solution #3 as the basis for normative work.

The conclusions to Key Issue #4 is described in clause 9.2.5. This refer to selection of Solution #4 as the basis for normative work.

* * * *   End of changes   * * * *