3GPP TSG-SA WG6 Meeting #52	S6-223116

Toulouse, France 14th – 18th November 2022	 (revision of)

Source:	Netherlands Police

Title:	pCR Solution on provide interconnection group IDs

Spec:	3GPP TR 23.700-v38 v0.4.0

Agenda item:	9.2

Document for:	Approval

Contact:	kees.verweijAtpolitieDotnl

1. Introduction

This pCR adds a solution for key issue 4, “Group configuration data“. It also addresses the 2nd pre-condition in 3GPP TS 23.280, clause 10.2.7.3:

-	“The MC service group is configured in the MC service user profiles of one or more MC service users in the partner MC system of the MC service group.”

This solutions enables an authorized user to provide a list of interconnection group ID(s) to the authorized user of a partner MC system.

Stage 1 requirements are stated in 3GPP TS 22.280, clause 5.16.4:

[R-5.16.4-005] An MCX Service shall provide a mechanism to allow an authorised MCX User to send configuration information to Partner MCX Service Systems.

[R-5.16.4-002] An MCX Service shall provide a mechanism to allow an authorised MCX User to evaluate and respond to requests for configuration changes from Partner MCX Service Systems.

2. Reason for Change

Adding a solution for key issue 4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-38 v0.3.0.

* * * First Change * * * *

## 7.x	Solution #x: Provide Interconnection Group IDs

### 7.x.1	Solution description

#### 7.x.1.1	General

This solution addresses the following aspects:

-	Key issue 4 – Group configuration data

“The MC service group is configured in the MC service user profiles … “

This solutions enables an authorized user to provide a list of interconnection group ID(s) to a selected authorized user of a partner MC system.

### 7.x.1.2	Information flows

#### 7.x.1.2.1	Group information provision request

Table 7.x.1.2.1-1 describes the information flow of the Group information provision request sent from the primary ACMC to the partner ACMS(s) and partner ACMC(s).

Table 7.x.1.2.1-1: Group information provision request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the primary MC system who triggers the Group information provision request. |
| Functional alias | O | The functional alias of the MC service user at the primary MC system who triggers the Group information provision request. |
| MC service ID | O (see NOTE) | The identity of the MC service user at the partner MC system towards which the Group information provision request is sent |
| Functional alias | O (see NOTE) | The functional alias of the MC service user at the partner MC system towards which the Group information provision request is sent |
| MC service group ID list | M | A list of one or more MC service group IDs |
| Group description per MC service group ID | O | A text description indicating the intended operational use for every MC service group ID in the list |
| ACM transaction identifier | M | Identifies the ACM transaction |
| NOTE: Either the MC service ID or the functional alias must be present. |  |  |

#### 7.x.1.2.2	ACM request stored indication

Table 7.x.1.2.2-1 describes the information flow of the ACM request stored indication from a partner ACMS to the primary ACMS and the primary ACMC.

Table 7.x.1.2.2-1: ACM request stored indication

| Information element | Status | Description |
| --- | --- | --- |
| ACM transaction identifier | M | Identifies the ACM transaction |

#### 7.x.1.2.3	Pending ACM requests notification

Table 7.x.1.2.3-1 describes the information flow of the Pending ACM requests notification from a partner ACMS to the partner ACMC.

Table 7.x.1.2.3-1: Pending ACM requests notification

| Information element | Status | Description |
| --- | --- | --- |
| ACM transaction identifier list | M | A list of one or more pending ACM requests |

#### 7.x.1.2.4	Get pending ACM request

Table 7.x.1.2.4-1 describes the information flow of the Get pending ACM request from a partner ACMC to the partner ACMS.

Table 7.x.1.2.4-1: Get pending ACM request

| Information element | Status | Description |
| --- | --- | --- |
| ACM transaction identifier | M | Identifies the ACM request |

#### 7.x.1.2.5	Group information provision response

Table 7.x.1.2.5-1 describes the information flow of the Group information provision response from a partner ACMC to a partner ACMS, the primary ACMS and the primary ACMC.

Table 7.x.1.2.5-1: Group information provision response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the partner MC system who generates the Group information provision response. |
| Functional alias | O | The functional alias of the MC service user at the partner MC system who generates the Group information provision response. |
| Provision status | M | Indicates the provisioning result |
| ACM transaction identifier | M | Identifies the ACM transaction |

#### 7.x.1.2.6	Pending ACM responses notification

Table 7.x.1.2.6-1 describes the information flow of the Pending ACM responses notification from a primary ACMS to the primary ACMC.

Table 7.x.1.2.6-1: Pending ACM responses notification

| Information element | Status | Description |
| --- | --- | --- |
| ACM transaction identifier list | M | A list of one or more pending ACM responses |

#### 7.x.1.2.7	Get pending ACM response

Table 7.x.1.2.7-1 describes the information flow of the Get pending ACM response from a primary ACMC to the primary ACMS.

Table 7.x.1.2.7-1: Get pending ACM response

| Information element | Status | Description |
| --- | --- | --- |
| ACM transaction identifier | M | Identifies the ACM response |

#### 7.x.1.3	Procedure

##### 7.x.1.3.1	Authorized user providing interconnection group ID(s) to authorized user of a partner MC system

The procedure, which enables an authorized MC service user of a primary MC system to provide a list of interconnection group ID(s) to a selected authorized user of a partner MC system, is shown in figure 7.x.1.3.1-1.

Pre-conditions

-	primary MC system and partner MC system are interconnected

-	both MC systems have implemented an ACMS functionality

-	the primary MC system is the MC service group home system for one or more interconnection groups

-	The MC service user of the primary MC system is authorized to provide a list of interconnection group IDs to a selected authorized user of the partner MC system .

-	The MC service user of the primary MC system has selected an authorized user in the partner MC system to provide a list of interconnection group ID(s) to.

-	At the start of the procedure the selected authorized user of the partner MC System is not logged on.

Figure 7.x.1.3.1-1: Group information provision to an authorized user of a partner system

1.	The ACM client in the primary MC system sends a group information provision request to the ACM server in the primary MC system, providing a list of interconnection  group IDs and the ID of the selected authorized user in the partner MC system.

2.	The ACM server of the primary MC system checks whether the MC service user at ACM client is authorized for the request.

3.	The ACM server of the primary MC system forwards the group information provision request to the ACM server in the partner MC system.

4a. 	The ACM server in the partner MC system verifies the request from the ACM server in the primary MC system and stores the request.

4b.	The ACM server of the partner MC system indicates that the group information provision request has been stored.

5.	The authorized MC service user of the primary MC system logs off.

NOTE 1:	There may be a considerable pause between step 5 and 6.

6.	The selected authorized MC service user of the partner MC system logs on to the ACM client of the partner MC system.

7a.	The ACM server of the partner MC System notifies the ACM client of partner MC System that there is a pending ACM request.

7b.	The ACM client of the partner MC system queries the ACM server of the partner MC system for the pending request.

8.	The ACM server of the partner MC system forwards the stored group information provision request to the ACM client of the partner MC system, informing the selected authorized user of the partner MC System about the interconnection group IDs.

9.	The ACM client of the partner MC system sends a group information provision response to the ACM server of the partner MC system.

10. The ACM server of the partner MC system sends the group information provision response to the ACM server in the primary MC system.

11.	The ACM server of the primary MC system stores the group information provision response.

NOTE 2:	There may be a considerable pause between step 11 and 12.

12.	The authorized user of the primary MC system logs on to the ACM client of the primary MC system.

13a.	The ACM server of MC System A notifies the ACM client of MC System A that there is a pending ACM response.

13b.	The ACM client of MC system A queries the ACM server of MC system A for the pending ACM response.

14.	The ACM server of the primary MC system forwards the group information provision response to the ACM client of primary MC system, informing the authorized user of the primary MC system that the list of interconnection group IDs has been provided to the selected authorized user of the partner MC system.

### 7.x.2	Solution evaluation

This solution enables an authorized user from a primary MC system to provide interconnection group IDs to a selected authorized user of a partner MC system.

* * * End of Changes * * * *