3GPP TSG-SA WG6 Meeting #52	S6-223148

Toulouse, 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0167 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Decoupling of signalling and media paths for MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, UIC, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-10-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The functional models as currently specified require that both signalling and media is processed together by the MCVideo server. For railways, low latency requirements are more important than the need for logging, which requires to have both signalling and data together at the MCVideo server. In the past the requirements with regard to latency and packet reliability were detailed in 3GPP TS 22.289 and 3GPP TR 23.783 and were discussed a number of times. In order to meet such requirements, alternatively, verticals need to be able to manage signalling separately from the media path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR describes the minimum adjustments that are necessary to enable signalling and media to be managed separately. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements regarding very low latency application support cannot be fullfiled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******************************************1st Change******************************************

### 6.1.1	On-network functional model

Figure 6.1.1-1 shows the functional model for the application plane of MCVideo service for on-network operations.

Figure 6.1.1-1: Functional model for application plane of MCVideo service

In the model shown in figure 6.1.1-1, the following apply:

-	The MCVideo server is an instantiation of a MC service server in accordance with 3GPP TS 23.280 [6].

-	The MCVideo server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [9].

-	The MCVideo server controls the signalling reference points and the corresponding media allowing different paths for media and signalling. The MCVideo server determines if it needs to stay in the media path based on the feature(s) involved in the MCVideo communication.

NOTE 1:	If the MC Video server is not in the MCVideo communication media path, any implications regarding MC user data logging are left to implementation.

NOTE 2:	Whether the media distribution function and the transmission control server are within the MCVideo server or outside the MCVideo server is a deployment option.

-	MCVideo-9 carries signalling over multicast bearer between the transmission control server of the MCVideo server and the transmission control participant of the MCVideo UE.

-	MCVideo-4 carries signalling over unicast bearer between the transmission control server of the MCVideo server and the transmission control participant of the MCVideo UE.

******************************************End of Changes *****************************************