3GPP TSG-SA WG6 Meeting #52-e	S6-223149

e-meeting, 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0355 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Private call handling when migrating through multiple MC systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The solution agreed in S6-222948 (CR0353), is offering a generic procedure for private communication towards a migrated MC service user at a partner MC system either by an MC service ID provided in the primary MC system or in the (current) partner MC system. The solution does not allow to use an MC service ID assigned to the MC service user by a (intermediate/previous) partner MC system, i.e., a partner MC system which the MC service has already left towards another partner MC system. The private communication request towards an intermediate/previous partner MC system would fail or even worse, the request may be routed towards another MC service user who is now using the same MC service ID. To overcome the situation, the MC organisation must ensure - that an MC service ID is carefully assigned to a migrated MC service user and not simply reused by another MC service user, also not subsequently re-used for another MC service user later in time - that the relationship between an MC service ID assigned by the partner MC system and the primary MC system is stored by the partner MC system also when the MC service user has left to the partner MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a NOTE indicating that a partner MC system should keep the relationship between the provided MC service ID for an migrated MC service user and the related MC service ID provided by the primary MC system. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Private communication etasblishments fail when the request is sent using an MC service ID provided by an MC system for an migrated MC service user who has left the partner MC system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.6.3.3	Procedure

The procedure for service authorization for migration to a partner MC system is shown in figure 10.6.3.3-1.

Pre-conditions

-	The MC service user wishes to migrate to the partner MC system.

-	The MC service client has been configured with an MC service user profile that contains the necessary parameters needed for connectivity with the partner MC system.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service authorization to take place in the partner system.

Figure 10.6.3.3-1	Service authorization for migration to partner MC system

1.	The MC service client requests migration service authorization with the partner MC service server indicating the selected MC service user profile to be used during migrated MC service. The MC service client provides both the MC service ID provided during user authentication in the partner MC system, and the MC service ID of the MC service user in the primary MC system of the MC service user.

NOTE 1:	The migrating MC service client also provides authentication credentials which are specified in 3GPP TS 33.180 [25].

2.	The partner MC service server performs an initial authorization check to verify that the MC service user is permitted to migrate to the partner MC system from the primary MC system of the MC service user.

NOTE 2:	The criteria for the initial authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request migrated service authorization.

3.	The partner MC service server identifies the primary MC system of the MC service user of the MC service client by use of the MC service ID of the MC service user in the primary MC system of the MC service user, which was presented by the MC service client in step 1, and sends a migration service authorization request to the gateway server in the partner MC system.

4.	The partner MC system gateway server identifies the primary MC system of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to the gateway server of the primary MC system.

5.	The gateway server in the primary MC system of the MC service user identifies the primary MC service server of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to that MC service server.

6.	The primary MC service server of the MC service user performs an authorization check, to verify that migration is permitted to that partner MC system by this MC service user using the indicated MC service user profile.

7.	The primary MC service server marks the MC service user as having migrated, and records the partner MC system as the migrated MC system.

8.	The primary MC service server sends a migration service authorization response to the gateway server in the primary MC system.

9.	The gateway server in the primary MC system sends the migration service authorization response to the gateway server in the partner MC system.

10.	The gateway server in the partner MC system sends the migration service authorization response to the partner MC service server.

11.	The partner MC service server sends the migration service authorization response to the MC service client, confirming that successful migration and service authorization has taken place.

NOTE 3:	The partner MC system needs to store the necessary information related to the migrated MC service users (e.g., MC service ID of migrated MC service users provided by primary MC systems) to allow proper communication redirection once migrated MC service users either migrate to another partner MC system or migrate back to their primary MC system.

NOTE 4:	If topology hiding is not used, the migration service authorization request and migration service authorization response messages are sent between the MC service servers in the primary and partner MC system without the need to be sent via an MC gateway server.

# * * * * End of changes * * * *