3GPP TSG-SA WG6 Meeting #52	S6-223154

Toulouse, France 14th – 18th November 2022	(revision of S6-22abcd)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0325 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ad hoc group call procedures in multiple MCPTT system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2022-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TR 23.700-76 V1.3.0 has defined the procedures for the adhoc group call in multiple MCPTT system. The conclusion in the clause 9 for the Key issue #1 indicates that the procedures in the clause 7.1.3.2 of solution 1 are agreed.This CR is trying to propose the corresponding normative text in the normative work phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new clauses to provide the procedures for the adhoc group call. Added the procedures as defined in the clause 7.1.3.2 of TR 23.700-76 V1.3.0 into new clause 10.19.2.4 of TS 23.379 as a normatie text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group call procedures will be missing in multiple MCPTT systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (10.19.2.4, 10.19.2.4.1, and 10.19.2.4.2) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.19.2.4	Ad hoc group call procedures in multiple MCPTT systems

##### 10.19.2.4.1	Ad hoc group call setup – Participants list provided by the initiator

Figure 10.19.2.4.1-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user involving the MCPTT users from multiple MCPTT systems.

Pre-conditions:

1.	The authorized user at MCPTT client 1 wants to invite MCPTT users at MCPTT client 2, MCPTT client 3 and MCPTT client 4 for the ad hoc group call.

2.	The MCPTT client 1 and MCPTT client 2 are registered to the MCPTT provider 1. The MCPTT client 3 and MCPTT client 4 are registered to MCPTT provider 2.

3.	Number of participants being invited for the call is within the limit.

4.	End-to-End encryption is supported for this ad hoc group call.

Figure 10.19.2.4.1-1: Ad hoc group call setup involving multiple MCPTT systems

1-4a.	Same as described in subclause 10.19.2.3.1.

4b.	The MCPTT server 1 sends the ad hoc group call request towards the MCPTT server 2. This request carries the list of MCPTT users who needs to be invited for this ad hoc group call and are registered with MCPTT server 2. It also carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. While sending the ad hoc group call requests, the MCPTT server shall remove the information elements that are not required to be conveyed to the target MCPTT clients (e.g. Block sharing participants list indicator, MCPTT ID list of the users who are required to acknowledge.

5a-5b.	The MCPTT server 2 sends the ad hoc group call request towards the MCPTT client 3 and MCPTT client 4.

6a-6c.	The receiving MCPTT clients are notified about the incoming ad hoc group call.

7.	The MCPTT client 2 accept the ad hoc group call request and send ad hoc group call response to the MCPTT server 1.

8a-8b.	The MCPTT client 3 and MCPTT client 4 accepts the ad hoc group call request and sends ad hoc group call responses to the MCPTT server 2.

9a-9b.	The MCPTT server 2 forwards the responses to the MCPTT server 1.

10.	The MCPTT server sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

11.	The MCPTT server may notify the initiating MCPTT user of all MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the MCPTT ad hoc group call.

12.	MCPTT client 1, MCPTT client 2, MCPTT client 3 and MCPTT client 4 establish media plane and floor control resources.

NOTE:	How to handle the end-to-end encryption when the MCPTT client 1 does not have the required security related information to call with the participants of the ad hoc group call is in the scope of SA3.

##### 10.19.2.4.2	Ad hoc group call setup – Participants list determined by the MCPTT server

Figure 10.19.2.4.2-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user wherein the list of participants is determined by the MCPTT server based on the citeria received from the MCPTT client and determined MCPTT users are from multiple MCPTT systems.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initate ad hoc group call.

2.	End-to-End encryption is supported for this ad hoc group call.

Figure 10.19.2.4.2-1: Ad hoc group call setup involving multiple MCPTT systems

1-4.	Same as described in subclause 10.19.2.3.1.

5.	The MCPTT server 1 sends the ad hoc group call request towards the MCPTT server 2 containing the details of the criteria to be applied by the MCPTT server 2 for determining the participants list in the partner system and are registered with MCPTT server 2. It also carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested.

6a-6b.	On determining the MCPTT client 3 and MCPTT client 4 as the participants of the adhoc group call in partner system, the MCPTT server 2 at partner system sends the ad hoc group call request towards the MCPTT client 3 and MCPTT client 4.

7.	The MCPTT server 1 sends the ad hoc group call requests towards the MCPTT client 2. While sending the ad hoc group call request, the MCPTT server shall remove the information elements that are not required to be conveyed to the target MCPTT clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group call if end-to-end encryption is requested. The MCPTT server considers the ad hoc group call participants as implicitly affiliated to the ad hoc group.

8a-8c.	The receiving MCPTT clients are notified about the incoming ad hoc group call.

9.	The MCPTT client 2 accept the ad hoc group call request and send ad hoc group call response to the MCPTT server 1.

10a-10b.	The MCPTT client 3 and MCPTT client 4 accepts the ad hoc group call request and sends ad hoc group call responses to the MCPTT server 2.

11a-11b.	The MCPTT server 2 forwards the responses to the MCPTT server 1.

12.	The MCPTT server 1 sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

13.	The MCPTT server 1 may notify the initiating MCPTT user of all MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server 1 more than once during the call when MCPTT users join or leave the MCPTT ad hoc group call.

14.	MCPTT client 1, MCPTT client 2, MCPTT client 3 and MCPTT client 4 establish media plane and floor control resources.

* * * End of Changes * * * *