3GPP TSG-SA WG6 Meeting #52	S6-223204

Toulouse, France 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0092 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MSGin5G UE bulk Configuration over MSGin5G-6 reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is proposed to add the bulk configuration procedure for constrained device over MSGin5G-6 reference point to fulfill the second objective of the Rel-18 5GMARCH2 WID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of the bulk configuration procedure for constrained device over MSGin5G-6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure to fulfill the second objective of the Rel-18 5GMARCH2 WID is still needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.1.X	MSGin5G UE bulk Configuration over MSGin5G-6 reference point

When constrained devices MSGin5G UE-2a and UE-2b, which support an MSGin5G Client, get the MSGin5G Service configuration information via MSGin5G UE-1, the MSGin5G UE-1 may decide to use bulk configuration procedure specified in this clause based on service policy.

Pre-conditions:

1.	MSGin5G UE-2a and MSGin5G UE-2b have discovered and selected MSGin5G UE-1 as the MSGin5G Proxy UE as specified in clause 8.2.8 and connected to the serving network via MSGin5G UE-1 successfully.

The MSGin5G UE-2a and UE-2b send the Get VAL UE configuration request as specified in clause 8.1.2 to MSGin5G UE-1. If the MSGin5G UE-1 decides to use bulk configuration, the MSGin5G UE-1 sends a Get VAL UE configuration response including the information elements as listed in table 8.1.x-1 as a part of VAL UE configuration data to MSGin5G UE-2a and UE-2b.

Table 8.1.x-1: Information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Expected configuration time | O | The expected time when the bulk configuration can be handled and the MSGin5G UEconfiguration Response can be received. |

In the MSGin5G UE bulk configuration procedure, the MSGin5G UE-1 acts as Configuration management client specified in clause 8.1.2, In addition to the information elements listed in table 8.1.2-1, the Get VAL UE configuration request used for bulk Configuration also includes the information elements as listed in table 8.1.x-2.

Table 8.1.x-2: Information in the Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk configuration flag | M | Indicates this request is a enhanced Get VAL UE configuration request used for MSGin5G UE bulk Configuration |
| List of MSGin5G UE IDs | M | List of MSGin5G UE ID of MSGin5G UE-bs which needed to be configured in this bulk configuration request |

In addition to the information elements listed in table 8.1.2-2, the Get VAL UE configuration response used for bulk Configuration also includes the information elements as listed in table 8.1.x-3 as a part of VAL UE configuration data.

Table 8.1.x-3: Information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| List of MSGin5G UE configuration information | M | Each element in this list contains information as specified in Table 8.1.2-2 |

Editor's note:	The enhancement of VAL UE configuration request and response are needed to be captured by eSEAL2 WID and are FFS.

Upon receiving the Get VAL UE configuration response used for bulk Configuration, the MSGin5G UE-includes the MSGin5G UE configuration information in the List of MSGin5G UE configuration information into multiple individual Get VAL UE configuration response and sends them to the corresponding MSGin5G UE-2s.