3GPP TSG-SA WG6 Meeting #52	S6-223244

Toulouse, France 14th – 18th November 2022	(revision of S6-222949)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0126 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of prediction expiration time IE and ACR information procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In service continuity planning in clauses 8.8.2.2 to 8.8.2.6, the ACT phase can be performed for a predicted future UE location and in the end the Post-ACR Clean up phase is left for the time when the UE actually moves to the future location. In Rel. 18 study in TR 23.700-98, clauses 7.21 and 7.6, solution #21 adds a prediction expiration time to the ACR request. This IE is transferred to the T-EAS, so that the T-EAS can use it for handling the ACT. The T-EAS can consider prediction expiration time in deciding whether and for how long to wait for the AC of the UE to connect to it. In other words, if the ACT is performed, and the AC does not connect to the T-EAS by "Prediction expiration time", the EAS can send ACT failure with the appropriate cause to the EES. In that case, the T-EAS can delete the transferred application context. In addition to the new IE, a procedure is required to provide the IE to the T-EES and T-EAS. Also, Rel. 18 study in TR 23.700-98, clause 7.6, solution 6 adds the feature for modifying the parameters of an ACR. Therefore, a procedure is required to provide the modified parameters, e.g. "Prediction expiration time" to the T-EES and T-EAS. Therefore, this paper also introduces such a procedure and the related information flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 8.8.4.4: the optional IE "Prediction expiration time" is added to ACR request. 8.8.3.X: A procedure for communication ACR parameters, e.g. "Prediction expiration time" to the T-EES and T-EAS is specified. 8.8.4.X, 8.8.4.Y: information flows for the procedure are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new features that are introduced and concluded in Rel. 18 study will not work. In Rel. 18 study TR 23.700-98, new features to address service continuity planning are captured in Sol#7 and #21. As per conclusion of TR 23.700-98 these new features are to be addressed in the normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.4.4, 8.8.3.X (new), 8.8.4.X (new), 8.8.4.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev 2 of the CR, two Editor’s notes are deleted. These Editor’s notes are addressed in CRxxx and CRyyy. Also, the event of ACR management notification is clarified as "ACT start ". |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

#### 8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent either from the EEC to the S-EES or T-EES, or by the S-EAS to the S-EES.

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | O | Identifier of the EAS |
| UE identifier | M | The identifier of the UE (i.e. GPSI). |
| ACID | O | The identifier of the AC. |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > Previous T-EAS Endpoint (NOTE 7) | O | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS of the previous ACR. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > Previous EAS notification indication (NOTE 7) | O | Indicates whether to notify the EAS about the cancellation of a previous ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| > Prediction expiration time (NOTE X) | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest |
| > EEC context relocation details | O | Information required for EEC context relocation using the EEC context push or EEC context pull mechanisms. |
| >> EEC Context ID (NOTE 5) | O | Identifier of the EEC Context |
| >> S-EES ID (NOTE 5) | O | Identifier of the EES that provided EEC context ID. |
| >> S-EES endpoint (NOTE 5) | O | The endpoint address (e.g. URI, IP address) of the EES that provided EEC context ID. |
| >> T-EES ID (NOTE 6) | O | Identifier of the T-EES. |
| >> T-EES endpoint (NOTE 6) | O | The endpoint address (e.g. URI, IP address) of the T-EES. |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| NOTE 1: This IE shall be present if the EAS notification indication or previous EAS notification indication indicates that the EAS needs to be informed. NOTE 2: Either ACR initiation or ACR determination shall be included corresponding to the ACR action. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: Void. NOTE 5: This IE may be present only if the request originates from the EEC towards the T-EES. NOTE 6: This IE may be present only if the request originates from the EEC towards the S-EES. NOTE 7: These IEs shall be present when the EEC re-sends the ACR request as described in clause 8.8.1.3 to indicate a previous ACR is to be cancelled. NOTE X: This IE may be included when the ACR is decided and executed for service continuity planning for a predicted/expected UE location. |  |  |

# * * * Change 2 * * * *

#### 8.8.3.X	ACR parameter information procedure

Figure 8.8.3.X-1 illustrates the procedure for sending ACR parameters from S-EES to the T-EES and T-EAS. The procedure may be used by the S-EES at the beginning of the ACR execution to provide ACR parameters, e.g. prediction expiration time, to the T-EES and T-EAS. The procedure may also be used during an ongoing ACR to update ACR parameters.

Pre-condition:

1.	The ACR has been launched to the S-EES.

Figure 8.8.3.X-1: ACR parameter information procedure

1.  The S-EES sends the ACR parameter information to the T-EES.

2.  The T-EES checks if the requestor is authorized for this operation. If authorized, the T-EES processes the request

3.  If the request is authorized and if the T-EAS has subscribed to receive ACR notifications, the EES shall notify the T-EAS by sending an ACR management notification, with "ACT start" event including ACR parameters from the request in step 1, e.g. Prediction expiration time.

NOTE: The T-EAS can use the ACR parameters to handle the ACR. For example, the T-EAS can consider prediction expiration time in deciding whether and for how long to wait for the AC of the UE to connect to it. If the ACT is performed, and the AC does not connect to the T-EAS by "Prediction expiration time", the EAS can send ACT failure with the appropriate cause to the EES. In that case, the T-EAS can delete the transferred application context. If the EAS had included indication of EAS Acknowledgement within ACR management subscribe request, the EAS sends EAS Acknowledgement as a response to the ACR management notification. The EAS can indicate the failure or rejection of the ACT in the Acknowledgement.

4.  The EES responds to the requestor's request with an ACR parameter information response message.

# * * * Change 3 * * * *

#### 8.8.4.X	ACR parameter information

Table 8.8.4.X-1 describes information elements for the ACR parameter information sent from the S-EES to the T-EES.

Table 8.8.4.X-1: ACR parameter information

| Information element | Status | Description |
| --- | --- | --- |
| EES ID | M | Unique identifier of the requesting S-EES. |
| EECID (NOTE 1) | M | Unique identifier of the EEC. |
| ACID (NOTE 1) | M | The identifier of the AC. |
| S-EAS Endpoint (NOTE 1) | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the S-EAS. |
| T-EAS Endpoint (NOTE 1) | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| ACR parameters | M | ACR parameters |
| > Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest |
| NOTE 1: These IEs are used to identify the ACR. |  |  |

Editor’s note: Additional ACR parameters, if required, are FFS.

#### 8.8.4.Y	ACR parameter information response

Table 8.8.4.Y-1 describes information elements for the ACR parameter information response sent from the T-EES to the S-EES.

Table 8.8.4.Y-1: ACR parameter information response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the ACR parameter information was successful. |
| Failure response (NOTE) | O | Indicates that the ACR parameter information failed. |
| > Cause | O | Indicates the cause of ACR parameter information failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

# * * * End of changes * * * *