3GPP TSG-SA WG6 Meeting #52	S6-223279

Toulouse, France 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0141 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL service over 5GS supporting EPS interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The basic procedures e.g., 5G MBS session create, update, delete, data transmission are agreed and introduced in TS 23.434. However, how the VAL service can be supported with the help of SEAL NRM server/client when it provides service both in 5GS and EPS and the 5G system supporting the EPS interwork. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures and information flows for VAL services over 5GS supporting EPS interworking |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEAL cannot support the servic over 5GS supporting EPS interworking. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.z (new), 14.3.z.1 (new), 14.3.z.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/***************************** First Change ****************************/

### 14.3.z	VAL services over 5GS supporting EPS interworking

#### 14.3.z.1	General

VAL services over a 5G system supporting EPS interworking are provided to 5GS/EPS capable VAL UEs as defined in 3GPP TS 23.501 [10], 3GPP TS 23.502 [11], and 3GPP TS 23.503 [12].

This clause describes procedures related to the support of VAL services over a 5G system with EPS interworking.

#### 14.3.z.2	Inter-system mobility

As specified in 3GPP TS 23.501 [10], a dedicated user plane anchor point, i.e. UPF + PGW-U function, is defined for interworking between 5GS and EPS. This enables that the network can directly handle PDU sessions (in 5GS) and PDN connections (in EPS) associated to VAL service sessions of a VAL UE during inter-system mobility.

The inter-system mobility of a VAL UE will be transparent to the NRM server and VAL server. The NRM server will continue interacting with the same control plane functions, e.g. PCF, and the VAL server will continue interacting with the same user plane function, e.g. UPF + PGW-U.

NOTE 1:	For the case that seamless session continuity is required for VAL services,  EPS interworking with N26 (interface between AMF in 5GC and MME in EPC) is required for inter-system change, as described in 3GPP TS 23.501 [10].

The procedure in figure 14.3.z.2-1 describes in a high level inter-system mobility of 5GS/EPS capable VAL UEs with associated VAL service sessions.

Pre-conditions:

-	VAL UE is a 5GS/EPS capable UE.

-	VAL UE is registered to a 5GS supporting EPS interworking for the support of VAL services.

Figure 14.3.z.2-1: VAL services over 5GS with EPS interworking – Inter-system mobility

1.	VAL UE is registered to the VAL server over the 5GS. VAL UE has established a PDU session(s) associated to VAL service sessions.

2.	The 5GS supporting EPS interworking decides that the VAL UE should be handed over to EPS, e.g. due to radio conditions. 5GS handles associated PDU sessions of the VAL UE to establish corresponding PDN connections in EPS, i.e. required default and dedicated EPS bearers associated to VAL service sessions.

NOTE 2:	The interworking procedures with EPS are described in 3GPP TS 23.501 [10], 3GPP TS 23.502 [11], and 3GPP TS 23.503 [12].

3.	VAL UE is registered on EPS. VAL services continue being provided to the VAL UE over EPS via the established PDN connections (default and dedicated bearers associated to corresponding VAL service sessions) in step 2.

/***************************** End of Changes ****************************/