3GPP TSG-SA WG6 Meeting #52	S6-223319

Toulouse, France 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 151 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EDGE-5 – ACR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures and information flows related to ACR requests between the AC and the EEC are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | With this CR: - S-EAS information is added in the ACR request - Details of the ACR response are added - Information flows for ACR request and ACR response are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EDGE-5 ACR related clauses will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.2.1, 8.14.2.4, 8.14.3.10 (new), 8.14.3.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.14.2.1	General

Following procedures are specified for EDGE-5:

-	AC registration request;

-	EAS discovery request;

-	ACR;

-	AC subscription; and

-	AC notification.

Editor's Note:	Additional procedures between AC and EEC for registration update, deregister, subscription update and to unsubscribe are FFS.

* * * Next Change * * * *

#### 8.14.2.4	ACR

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 8.14.2.4-1: ACR request procedure

1.	The AC sends an ACR request to the EEC. The request includes AC profile, AC's security credentials, type of requested operation (i.e., ACR detection, ACR initiation) and S-EAS information. If the request is to initiate the ACR, the request may also include the target EAS information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC process the request from the AC. If the type of requested operation in the request received in step 1 is:

-	ACR detection, then the EEC determines if ACR is required or not. If it is required, the EEC uses one of the EEC initiated ACR scenarios or launches ACR with action "determination", leading to S-EES executed ACR;

-	ACR initiation, then the EEC uses one of the EEC initiated ACR scenarios and initiate ACR. If the request in step 1 also includes target information, the EEC uses it to select the ACR targets;

Once the ACR operation is complete, the EEC sends an ACR response message to the AC. The message includes the ACR result, S-EAS information and if ACR was initiated and successfully completed then the T-EAS information.

* * * Next Change * * * *

#### 8.14.3.10	ACR request

Table 8.14.3.10-1 describes information elements of the ACR request sent by the AC to the EEC.

Table 8.14.3.10-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile for EEC to determine the target EAS. |
| Security credentials | M | Security credentials of the AC |
| Requested ACR action | M | One of ACR detection or ACR initiation request actions |
| S-EAS information | M | ID of the EAS to which the ACR request relates. |
| T-EAS information (NOTE) | O | AC provided target EAS information. |
| NOTE: May be included if the request is to initiate the ACR. |  |  |

#### 8.14.3.11	ACR response

Table 8.14.3.11-1 describes information elements of the ACR response sent by the EEC to the AC.

Table 8.14.3.11-1: ACR response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ACR request was successful. |
| > ACR result | M | Indicates if EEC decided to initiate the ACR and also ACR status if ACR was initiated. |
| > S-EAS information | M | ID of the EAS to which the ACR response relates. |
| > T-EAS information | O | Information of the target EAS that AC should use if ACR was initiated and was successful. |
| Failure response | O | Indicates that the ACR request failed. |
| > Cause | O | Indicates the cause of ACR request failure. |