3GPP TSG-SA WG6 Meeting #52	S6-223320

Toulouse, France 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 152 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EDGE-5 – Subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures and information flows related to AC’s subscription of EEC’s services are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides: - EEC services subscription update and unsubscribe procedures. - Information flows related to EEC services subscription, notification, subscription update and unsubscribe. - Changes ‘AC subscription’ to ‘EEC services subscription’ as the term ’AC subscription’ is not clear and arguably wrong – it is not clear that the AC is subscribing to EEC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EDGE-5 subscription clauses will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.2.1, 8.14.2.5 (new), 8.14.2.5.1 (new), 8.14.2.5.2 (new), 8.14.2.5.3 (new), 8.14.2.5.4 (new) , 8.14.2.5.5 (new), 8.14.3.12 (new), 8.14.3.13 (new), 8.14.3.14 (new), 8.14.3.15 (new), 8.14.3.16 (new), 8.14.3.17 (new), 8.14.3.18 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.14.2.1	General

Following procedures are specified for EDGE-5:

-	AC registration request;

-	EAS discovery request;

-	ACR request; and

-	EEC services subscription.

Editor's Note:	Additional procedures between AC and EEC for registration update, deregister, subscription update and to unsubscribe are FFS.

* * * Next Change * * * *

#### 8.14.2.5	EEC services subscription

##### 8.14.2.5.1	General

Following are supported for EEC services subscription:

-	Subscribe;

-	Notify;

-	Subscription update; and

-	Unsubscribe.

##### 8.14.2.5.2	Subscribe

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 8.14.2.5.2-1: EEC services subscription procedure

1.	The AC sends an EEC services subscription request to the EEC. The request includes AC profile, AC's security credentials, a list of EEC's services that AC requires the EEC to handle, and related parameters as described in 8.14.3.12. If the subscription request includes:

-	EAS discovery or EAS dynamic information subscription, then the request may include a list of EAS discovery filters and a list of EAS dynamic information filters respectively;

-	ACR management, then the request includes a list of S-EAS information and corrosponding type of ACR operations:

-	ACR monitoring, where the EEC monitors the need for ACR and notifies the AC as and when required e.g., on receiving ACR related notifications on EDGE-1 interface.

-	EEC managed ACR, where the EEC monitors the need for ACR. If need for ACR is detected, then the EEC decides and initiates ACR using one of the EEC initiated ACR scenarios. The EEC notifies the AC about the imminent ACR and may include the target information.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC creates the subscription and sends an EEC services subscription response message to the AC. The response includes the list of services that the EEC will handle and related details.

4.	The EEC executes the services e.g., EAS discovery, ACR management, and notifies the AC with information as necessary. The EEC may use locally cached information or configurations while providing services to the AC.

##### 8.14.2.5.3	EEC services notification

Pre-conditions:

1.	The AC has subscribed to the EEC.

Figure 8.14.5.3-1: EEC services notification procedure

1.	An event occurs at the EEC that satisfies the trigger conditions for notifying a AC e.g., EEC detects a need for Application Context Relocation.

2.	The EEC sends an EEC services notification to the AC with relevant information related to the event triggered in step 1.

##### 8.14.2.5.4	EEC services subscription update

Figure 8.14.2.5.4-1 illustrates EEC services subscription update procedure.

Pre-conditions:

1.	The AC has subscribed to the EEC.

Figure 8.14.2.5.4-1: EEC services subscription update procedure

1.	The AC sends an EEC services subscription update request to the EEC. The request includes the subscription ID, AC's security credentials, and may include updated notification related details or updated list of required EEC services.

2.	The EEC checks AC's security credentials and validates the request.

3.	If the request is successfully validated, the EEC updates the subscription and sends a successful subscription update response.

4.	The EEC executes the services e.g., EAS discovery, ACR management, and notifies the AC with information as necessary. The EEC may use locally cached information or configurations while providing services to the AC.

##### 8.14.2.5.5	Unsubscribe

Figure 8.14.2.5.5-1 illustrates the unsubscribe procedure.

Pre-conditions:

1.	The AC has subscribed to the EEC.

Figure 8.14.2.5.5-1: EEC services unsubscribe procedure

1.	The AC sends EEC services unsubscribe request to the EEC. The request includes the subscription ID and AC's security credentials.

2.	The EEC checks AC's security credentials and validates the request.

3.	Upon successful authorization, the EEC sends a successful de-registration response.

* * * Next Change * * * *

#### 8.14.3.12	EEC services subscription request

Table 8.14.3.12-1 describes information elements for the EEC services subscription request sent by the AC to the EEC.

Table 8.14.3.12-1: EEC services subscription request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile of the AC requesting subscription |
| Security credentials | M | Security credentials of the AC. |
| Notification details | M | Details on how the EEC can notify the AC. |
| Required EEC services | M | EEC handled services required by the AC |
| > EAS discovery | O | Indicates that AC requires EEC to handle EAS discovery. |
| >> List of EAS discovery filters | M | List of EAS discovery filters as described in Table 8.5.3.2-2. |
| >>> Preferred ECSP list | O | Preferred ECSP list corresponding to each set of EAS discovery filters. |
| > EAS dynamic information | O | Indicates that the AC requires EAS dynamic information subscription |
| >> List of EAS dynamic information filters | M | List of EAS dynamic information filters as described in Table 8.5.3.4-2. |
| > ACR management | O | Indicates that the AC requires ACR related services |
| >> List of S-EAS information | M | S-EAS information corresponding to the ACR operation |
| >>> ACR operation | O | Indicates if AC requires ACR monitoring or EEC managed ACR corresponding to each S-EAS |

#### 8.14.3.13	EEC services subscription response

Table 8.14.3.13-1 describes information elements for the EEC services subscription response sent by the EEC to the AC.

Table 8.14.3.13-1: EEC services subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EEC services subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Confirmed EEC services | M | List of services EEC confirms to handle |
| >> EAS discovery | O | Indicates that the EEC will handle EAS discovery |
| >> EAS dynamic information | O | Indicates that the EEC will handle EAS dynamic information subscription |
| >> ACR management | O | Indicates that the EEC will handle ACR management |
| >>> List of S-EAS information | M | Information of the S-EASs EEC will handle. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

#### 8.14.3.14	EEC services notification

Table 8.14.3.14-1 describes information elements for the EEC services notification sent by the EEC to the AC.

Table 8.14.3.14-1: EEC services notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the notification. |
| Notification details | M | Information related to the notification |
| > EAS discovery | O | Indicates that the notification relates to EAS discovery |
| >> List of EAS profiles | M | EAS profiles associated with the notification. Each element is described in clause 8.2.4 |
| >>> Lifetime | O | Time interval or duration during which the information in the EAS profile is valid and supposed to be cached in the AC (e.g. time-to-live value for an EAS Endpoint) |
| > EAS dynamic information | O | Indicates that the notification relates to EAS dynamic information subscription |
| >> List of EAS profile | M | EAS profiles associated with the notification. Each element is described in clause 8.2.4 |
| >>> Lifetime | O | Time interval or duration during which the information in the EAS profile is valid and supposed to be cached in the AC (e.g. time-to-live value for an EAS Endpoint) |
| > ACR management | O | Indicates that the notification relates to ACR management |
| >> List of S-EAS information | M | EASID to which the notification relates. |
| >>> ACR detection | O | Indicates that the EEC detected a need for ACR |
| >>> ACR decision | O | EEC’s decision information related to an ACR detection |
| >>> ACR execution status | O | Indicates the status of ACR execution |
| >>>> Status | M | ACR status such as initiated, in-progress, complete |
| >>>> T-EAS information | O | Information of the T-EAS corresponding to the S-EAS |

#### 8.14.3.15	EEC services subscription update request

Table 8.14.3.15-1 describes information elements for the EEC services subscription update request sent by the AC to the EEC.

Table 8.14.3.15-1: EEC services subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | Subscription identifier corresponding to the update request. |
| Security credentials | M | Security credentials of the AC. |
| Notification details | O | Details on how the EEC can notify the AC. |
| Required EEC services | O | EEC handled services required by the AC |
| > EAS discovery | O | Indicates that AC requires EEC to handle EAS discovery. |
| >> List of EAS discovery filters | M | List of EAS discovery filters as described in Table 8.5.3.2-2. |
| >>> Preferred ECSP list | O | Preferred ECSP list corresponding to each set of EAS discovery filters. |
| > EAS dynamic information | O | Indicates that the AC requires EAS dynamic information subscription |
| >> List of EAS dynamic information filters | M | List of EAS dynamic information filters as described in Table 8.5.3.4-2. |
| > ACR management | O | Indicates that the AC requires ACR related services |
| >> List of S-EAS information | M | S-EAS information corresponding to the ACR operation |
| >>> ACR operation | O | Indicates if AC requires ACR monitoring or EEC managed ACR corresponding to each S-EAS |

#### 8.14.3.16	EEC services subscription update response

Table 8.14.3.16-1 describes information elements for the EEC services subscription update response sent by the EEC to the AC.

Table 8.14.3.16-1: EEC services subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EEC services subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Confirmed EEC services | M | List of services EEC confirms to handle |
| >> EAS discovery | O | Indicates that the EEC will handle EAS discovery |
| >> EAS dynamic information | O | Indicates that the EEC will handle EAS dynamic information subscription |
| >> ACR management | O | Indicates that the EEC will handle ACR management |
| >>> List of S-EAS information | M | Information of the S-EASs EEC will handle. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

#### 8.14.3.17	EEC services unsubscribe request

Table 8.14.3.17-1 describes information elements for the EEC services unsubscribe request sent by the AC to the EEC.

Table 8.14.3.17-1: EEC services unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | EEC services subscription identifier provided by the EEC during subscription. |
| Security credentials | M | Security credentials of the AC sending the EEC services unsubscribe request. |

#### 8.14.3.18	EEC services unsubscribe response

Table 8.14.3.18-1 describes information elements for the EEC services unsubscribe response sent by the EEC to the AC.

Table 8.14.3.18-1: EEC services unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the EEC services unsubscribe request was successful. |
| Failure response | O | Indicates that the EEC services unsubscribe request failed. |
| > Cause | M | Provides the cause for EEC services unsubscribe request failure. |