3GPP TSG-SA WG6 Meeting #52	S6-223344

Toulouse, France, 14th – 18th November 2022	(revision of S6-223229, S6-222948)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0353 | rev | 5 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Private call towards a migrated MC user |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2022-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces a generic procedure for a private communication towards a migrated MC service user at a partner MC system. The introduced clause describes the procedure needed for an MC service user to reach another user, where the latter has migrated and receives MC services from a partner MC system. The calling party may not be aware that the target user has migrated, hence it includes within the private call request the MC service ID of the target user provided by the primary MC system. Therefore, the MC service server sends a private call redirection notification towards the calling party to indicate that the target user has migrated, and to provide it with the MC service ID to user 2 provided by the partner MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a generic private call procedure towards a migrated MC service user. The generic procedure is applicable to an MCPTT private call, MCVideo private call, or a one-to-one MCData communication. - Define a private call redirection message sent from the MC service server to the calling MC service user which includes the necessary information (The MC service ID of user 2 obtained from the partner MC system) so that the calling party can reinitiate a private call towards the corresponding MC system, hence the migrated user. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An MC service user is unable to reach another MC service user once the latter has migrated to a partner MC system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.y (new), 10.16.y.1 (new), 10.16.y.2 (new), 10.16.y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA6#52: the information elements in Table 10.16.y.2-1 is updated to include the KMS URL associated to the MC service ID of the target user assigned at the partner MC system. Furthermore, a note is added in clause 10.16.y.3. The additional text is highlighted in green. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.16.y Generic private call procedure towards a migrated MC service user

#### 10.16.y.1 General

This subclause describes a generic private communication procedure towards a migrated MC service user at a partner MC system. This generic private communication can be an MCPTT private call, an MCVideo private call, or a one-to-one MCData communication. For the generic private call, this procedure is used in conjunction with the corresponding MCPTT, MCVideo and MCData procedures described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], or 3GPP TS 23.282 [13], respectively.

The migrated MC service user is reachable by their MC service ID assigned by the primary MC system via redirection done by the primary MC system’s MC service server to the migrated MC system’s MC service server based on the MC service ID provided by the migrated MC system.

#### 10.16.y.2 Private call redirection

Table 10.16.y.2-1 describes the information flow of a private call redirection, which is sent from the MC service server to an MC service client initiating a private call towards a migrated MC service user.

Table 10.16.y.2-1: Private call redirection

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user initiating a private call, i.e., calling party. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the target MC service user (i.e., called party), which the user has obtained from its primary MC system before migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the target MC service user, which the user has obtained from its migrated MC system after Migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| Security domain information (NOTE) | O | The KMS URI associated with the MC service ID of the target MC service user, which the MC service user has obtained from its migrated MC system after migration. |
| Redirection reason | O | The MC service server informs the calling party that the target user has migrated. |
| NOTE: If end-to-end encryption is required, the security domain information is a mandatory information element. |  |  |

#### 10.16.y.3 Procedure

Figure 10.16.y.4-1 presents a generic private communication procedure from MC service user 1 towards a migrated MC service user 2 where the MC system A is the primary MC system of MC service user before migration, and the MC system B is the MC system that the user has migrated

Pre-conditions:

- MC service system A and MC service system B are interconnected.

- MC system A is the primary MC system of MC service user 2 before migration. MC system B is the MC system that MC service user 2 has migrated.

- The MC service server A is aware that MC service user 2 has migrated and is informed of its MC service ID provided by MC system B, as described in clause 10.6.3.3.

Figure 10.16.y.3-1: MC private call towards a migrated MC service user.

- The MC service client 1 initiates a private call request towards MC service user 2 (MC service client 2) who has migrated to MC system B. The private call request includes among others the MC service ID of user 2, which is provided by its primary MC system, as the target, i.e., called party. The private call request is described in clause 10.7.2.1.1 in 3GPP TS 23.379 [16], in clause 7.2.2.2.1 in 3GPP TS 23.281 [12], or the corresponding procedures in 3GPP TS 23.282 [13].

- MC service server A checks that MC service user2 has migrated to MC system B with a new MC service ID assigned by MC system B. If end-to-end encryption is required, the MC service server A determines the KMS URI associated with the MC service ID from the selected user profile of the migrated MC service user 2.

- The MC service server A sends a private call redirection towards the MC service client 1, to inform MC service user 1 that MC service user 2 has migrated and its new MC service ID of MC service user 2 assigned by the migrated MC system. The MC service client 1 releases the private call request at step 1. If end-to-end encryption is required, the MC service server A includes the associated KMS URI of the MC service ID as well.

- The MC service client 1 initiates a private call towards MC service user 2, including the MC service ID of MC service user 2 obtained from MC system B and the KMS URI of the MC service ID of the partner MC system B once end-to-end encryption is required. The initiated private call can either be an MCPTT private call, an MCVideo private call, or a corresponding one-to-one MCData communication, and shall perform the procedures as described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], or 3GPP TS 23.282 [13], respectively.

NOTE:	If end-to-end encryption is required, the migrated MC service user 2 is assumed to have the necessary security information needed to establish a protected call, as defined in 3GPP TS 33.180 [25].

# * * * * End of changes * * * *