3GPP TSG-SA WG6 Meeting #52	S6-223349

Toulouse, France 14th – 18th November 2022	(revision of S6-223155)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0326 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifying participants list of on-going ad hoc group call in single MCPTT system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Kontron Transportation France, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2022-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TR 23.700-76 V1.3.0 has defined the procedures for modifying participants list of on-going ad hoc group call. The conclusion in the clause 9 for the Key issue #4 indicates that the procedures in the clause 7.4 of solution 4 are agreed.This CR is trying to propose the corresponding normative text in the normative work phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new clauses to provide the procedures for modifying participants list of on-going ad hoc group call. Added the procedures as defined in the clause 7.4 of TR 23.700-76 V1.3.0 into new clause 10.19.2.11 to 10.19.2.14 and 10.19.3.1.4 to 10.19.3.1.5 of TS 23.379 as a normatie text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the procedure for modifying participants list of on-going ad hoc group call in single MCPTT system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (10.19.2.11, 10.19.2.12, 10.19.2.13, 10.19.2.14, 10.19.3.1.4, and 10.19.3.1.5) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.19.2.11	Modify ad hoc group call participants request (MCPTT client – MCPTT server)

Table 10.19.2.11-1 describes the information flow Modify ad hoc group call participants request from the MCPTT client to the MCPTT server.

Table 10.19.2.11-1: Modify ad hoc group call participants request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| MCPTT ID list (see NOTE) | O | List of additional MCPTT users to be added to the on-going ad hoc group call |
| MCPTT ID list (see NOTE) | O | List of MCPTT users to be removed from the on-going ad hoc group call |
| NOTE : Either one or both of these information elements is present |  |  |

##### 10.19.2.12	Modify ad hoc group call participants response (MCPTT server – MCPTT client)

Table 10.19.2.12-1 describes the information flow Modify ad hoc group call participants response from the MCPTT server to the MCPTT client.

Table 10.19.2.12-1 Modify Ad hoc group call participants response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |
| MCPTT ID list | O | List of MCPTT users who are not allowed to be added to the on-going ad hoc group call. This list is provided if the operation is partially success. |

##### 10.19.2.13	Ad hoc group call leave request (MCPTT server – MCPTT client)

Table 10.19.2.13-1 describes the information flow ad hoc group call leave request from the MCPTT server to the MCPTT client.

Table 10.19.2.13-1 Ad hoc group call leave request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant leaving the call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call on which call is on-going |
| Reason to leave | O | Carries the reason of why the MCPTT client is being asked to leave the ongoing ad hoc group call |

##### 10.19.2.14	Ad hoc group call leave response (MCPTT client – MCPTT server)

Table 10.19.2.14-1 describes the information flow Ad hoc group call leave response from the MCPTT client to the MCPTT server.

Table 10.19.2.14-1 Ad hoc group call leave response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant leaving the call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group call on which call is on-going |

* * * Next Change * * * *

##### 10.19.3.1.4	Modification of ad hoc group call participants by the initiator

Figure 10.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by the initiator of the ad hoc group call.

Pre-conditions:

1.	The MCPTT client 1 is the initiator of the ad hoc group call.

2.	The MCPTT users on MCPTT client 1, MCPTT client 3 to MCPTT client n are on an ongoing ad hoc group call.

3.	The MCPTT user 1 requests to remove the user of MCPTT client 3 from the ad hoc group call and add user of MCPTT client 2 into the on-going ad hoc group call.

Figure 10.19.3.1.4-1: Modification of ad hoc group call participants by the initiator

1.	The MCPTT client 1 sends the modify ad hoc group call participants request to the MCPTT server in order to remove MCPTT client 3 from the ongoing ad hoc group call and add MCPTT client 2 into it.

2.	The MCPTT server verifies whether the MCPTT client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCPTT server sends modify ad hoc group call participants response to the MCPTT client 1.

4.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCPTT client 3 notifies the user of the ad hoc group call leave request.

6.	The MCPTT client 3 sends the ad hoc group call leave response to the MCPTT server.

7.	The MCPTT server removes the MCPTT client 3 from the ongoing ad hoc group call.

8.	The MCPTT server sends the ad hoc group call request towards MCPTT client 2.

NOTE:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCPTT client 2 notifies the user about the incoming ad hoc group call.

10.	The MCPTT client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCPTT server. The response may also contain a functional alias of the responding MCPTT user, which is verified (valid and activated for the user) by the MCPTT server.

11.	The MCPTT server may notify the initiating MCPTT user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the ad hoc group call.

12.	The MCPTT server may notify the participants about the change in the participants list of on-going ad hoc group call.

##### 10.19.3.1.5	Modification of ad hoc group call participants by the MCPTT server

Figure 10.19.3.1.5-1 below illustrates the modification of ad hoc group call participants procedure by the MCPTT server.

Pre-conditions:

1.	The MCPTT client 1 is the initiator of the ad hoc group call.

2.	MCPTT server determined the participants for the ad hoc group call based on the criteria specified by the MCPTT client 1 while initiating the ad hoc group call.

3.	MCPTT server continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group call.

4.	The MCPTT server detects that the MCPTT client 5 satisfies the criteria and MCPTT client 4 stops to meet the criteria specified by the MCPTT client 1.

Figure 10.19.3.1.5-1: Modification of ad hoc group call participants by the MCPTT server

1.	The ad hoc group call is established and on-going with the participants MCPTT client 1, MCPTT client 2, MCPTT client 3 and MCPTT client 4. The participants list is determined by the MCPTT server based on the criteria specified by the MCPTT client 1 while initiating the call

2.	The MCPTT server detects that the MCPTT client 5 satisfies the criteria specified by the MCPTT client.

3.	The MCPTT server sends the ad hoc group call request to the MCPTT client 5.

4.	The MCPTT client 5 notifies the user about the incoming ad hoc group call.

5.	The MCPTT client 5 accepts the ad hoc group call request and sends the ad hoc group call response to the MCPTT server.

6.	The on-going ad hoc group call is updated by adding MCPTT client 5 which satisfies the criteria specified by the MCPTT client 1.

7.	The MCPTT server detects that the MCPTT client 4 is no more satisfying the criteria to be the participant of the ad hoc group call.

8.	The MCPTT server sends the ad hoc group call leave request to the MCPTT client 4 and removes it from the on-going ad hoc group call.

9.	The MCPTT client 4 notifies the user of the ad hoc group call leave request.

10.	The MCPTT client 4 sends the ad hoc group call leave response to the MCPTT server.

11.	The on-going ad hoc group call is updated by removing MCPTT client 4, which no more satisfies the criteria specified by the MCPTT client 1.

* * * End of Changes * * * *