3GPP TSG-SA6 Meeting #52	S6-223457

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0138 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ADAE functional model in SEAL architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo Future Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2, ADAES |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The new SEAL service, namely ADAES, which is introduced in TS 23.436 / TR 23.700-36, requires the addition of a new clause on the architecture in SEAL spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new clause (clause 17) related to the new ADAE layer, and in particular the functional model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ADAE high level architecture is missing from SEAL specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 17 (new), 17.1 (new), 17.2 (new), 17.2.1 (new), 17.2.2 (new), 17.2.3 (new), 17.3 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

# 17	Application Data Analytics Enablement

## 17.1	General

The application data analytics enablement is a SEAL service that offers value-add application data analytics capabilities which cover stats/predictions for the end-to-end application service.

## 17.2	Functional architecture

### 17.2.1	General

The functional architecture for the application data analytics enablement is based on the generic functional model specified in clause 6.2. It is organized into functional entities to describe a functional architecture which addresses the support for application data analytics enablement aspects for vertical applications.

NOTE: In addition to the generic functional architecture, the internal ADAE functional architecture is described in TS 23.436 [x] clause 5.2.

### 17.2.2	On-network Functional Architecture

For the on-network functional architecture, both service-based representation and reference point representation are provided.

Figure 17.2.2-1 depicts the application data analytics enablement architecture in the non-roaming case, using the reference point representation showing how various entities interact with each other.

Figure 17.2.2-1: Architecture for application data analytics enablement – reference points representation

The application data analytics enablement client communicates with the application data analytics enablement server over the ADAE-UU reference point. The application data analytics enablement client provides the support for application data analytics enablement functions to the VAL client(s) over ADAE-C reference point. The VAL server(s) communicates with the application data analytics enablement server over the ADAE-S reference point. The application data analytics enablement server, acting as AF, may communicate with the 5G Core Network functions (over N33 reference point to NEF and N6 reference point to UPF) and OAM (over ADAE-OAM interface).

Figure 17.2.2-2 exhibits the service-based interfaces for providing and consuming application data analytics enablement services. The application data analytics enablement server could provide service to VAL server and ADAE client through interface SAdae.

Figure 17.2.2-2: Architecture for application data analytics enablement – Service based representation

Figure 17.2.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [x].

Figure 17.2.2-3: Architecture for application data analytics enablement utilizing the 5GS network services based on the 5GS SBA – Service based representation

Figure 17.2.2-4 illustrates the service-based representation including ADAE server as part of the SEAL framework.

Figure 17.2.2-4: SEAL functional model representation using service-based interfaces and including ADAE function

### 17.2.3	Off-network Functional Architecture

Figure 17.2.3-1 illustrates the generic off-network functional model for ADAE.

Figure 17.2.3-1: Generic off-network functional model

In the vertical application layer, the VAL client of UE1 communicates with VAL client of UE2 over VAL-PC5 reference point. An application data analytics enablement client of UE1 interacts with the corresponding application data analytics enablement client of UE2 over ADAE-PC5 reference points. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

## 17.3	Procedures and information flows for application data analytics enablement

The procedures and information flows for application data analytics enablement are presented in TS 23.436 [x].