3GPP TSG-SA WG6 Meeting #52	S6-223459

Toulouse, France 14th – 18th November 2022	(revision of S6-223178)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.222 | CR | 0091 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CAPIF business relationship updates for SNA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA6 has re-evaluated the CAPIF business relationship for SNA use cases in TR 23.700-95. In this study, a new business stakeholder, i.e., resource owner, is introduced in the business relationship. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The business relationships of CAPIF is updated and related texts are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The business relationships including the resource owner, which is required in SNA, is not shown in the CAPIF specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5, 5.1 (new), 5.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

API: The means by which an API invoker can access the service.

API invoker: The entity which invokes the CAPIF or service APIs.

API invoker profile: The set of information associated to an API invoker that allows that API invoker to utilize CAPIF APIs and service APIs.

API exposing function:  The entity which provides the service communication entry point for the service APIs.

API exposing function location: The location information (e.g. civic address, GPS coordinates, data center ID) where the API exposing function providing the service API is located.

CAPIF administrator: An authorized user with special permissions for CAPIF operations.

Common API framework: A framework comprising common API aspects that are required to support service APIs.

Designated CAPIF core function: The CAPIF core function which is configured as the serving CAPIF core function for interconnection.

Northbound API: A service API exposed to higher-layer API invokers.

Onboarding: One time registration process that enables the API invoker to subsequently access the CAPIF and the service APIs.

Resource: The object or component of the API on which the operations are acted upon.

Resource owner: An entity capable of granting access to a protected resource related to the invoked API.

Service API: The interface through which a component of the system exposes its services to API invokers by abstracting the services from the underlying mechanisms.

Serving Area Information: The location information for which the service APIs are being offered to.

Subscriber-aware northbound API access: An API invocation scenario where the API invoker needs an authorization from the resource owner.

PLMN trust domain: The entities protected by adequate security and controlled by the PLMN operator or a trusted 3rd party of the PLMN.

3rd party trust domain: The entities protected by adequate security and controlled by the 3rd party.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 32.240 [6] apply:

Offline charging

Online charging

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AEF	API Exposing Function

AF	Application Function

API	Application Program Interface

AS	Application Server

BM-SC	Broadcast Multicast Service Centre

CAPIF	Common API Framework

CDR	Charging Data Record

CRUD	Create, Read, Update, Delete

DDoS	Distributed Denial of Service

E-UTRA	Evolved Universal Terrestrial Radio Access

EPS	Evolved Packet System

ETSI	European Telecommunications Standards Institute

GS	Group Specification

IP	Internet Protocol

MBMS	Multimedia Broadcast and Multicast Service

MEC	Multi-access Edge Computing

NEF	Network Exposure Function

NGSI	Next Generation Service Interfaces

NR	New Radio

OMA	Open Mobile Alliance

OAM	Operations, Administration and Maintenance

OWSER	OMA Web Services

PC	Protocol Converter

PLMN	Public Land Mobile Network

REST	REpresentational State Transfer

RPC	Remote Procedure Call

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SNA	Subscriber-aware northbound API access

UDDI	Universal Description, Discovery and Integration

URI	Uniform Resource Identifier

WSDL	Web Services Description Language

* * * Next Change * * * *

5	Involved business relationships

5.1	Basic CAPIF business relationships

Figure 5.1-1 shows the typical business relationships in CAPIF.

Figure 5.1-1: Business relationships in CAPIF

The API invoker is typically provided by a 3rd party application provider who has service agreement with a CAPIF provider.

The API provider hosts one or more service APIs and has a service API arrangement with CAPIF provider to offer the service APIs to the API invoker.

The CAPIF provider and the API provider can be part of the same organization (e.g. PLMN operator), in which case the business relationship between the two is internal to a single organization. The CAPIF provider and the API provider can be part of different organizations, in which case the business relationship between the two must exist.

5.2	CAPIF business relationships for SNA

Figure 5.2-1 shows the CAPIF business relationships for the subscriber-aware northbound API access (SNA).

Figure 5.2-1: CAPIF business relationships for SNA

The business relationships the API invoker, the CAPIF provider, and the API provider follow the description in the clause 5.1. In addition to them, the resource owner is an entity capable of granting access to a protected resource related to the resource exposed by the API provider.

NOTE:	In the current release, both the CAPIF provider and the API provider should belong to the same organization (e.g., PLMN operator) and the service API arrangement is not required explicitly.