3GPP TSG-SA WG6 Meeting #52	S6-223272

Toulouse, France 14th – 18th November 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0147 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS instantiation status provisioned by ECS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the KI#9 conclusion, solution #40 proposes that the EEC can obatin the EAS instantiation status (e.g. instantiated or instantiable, but not yet instantiated) in EES profile through service provisioning. Based on the obtained EAS instantiation status, the EEC can perform EES selection to mitigate the wast of EDN resource. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the EAS instantiation status in EES profile and in service provisioning response, and the EEC can perform EES selection based on the obatined EAS instantiation status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution #40 for EAS instantiation status provisioned by ECS will remain and is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6, 8.3.3.3.3, 8.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs (NOTE) | M | List of EASIDs registered with the EES. |
| EAS information of instantiable but not instantiated yet | O | The EAS information(e.g. EASID) which is instantiable but not instantiated yet |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| EES Provider Identifier | O | The identifier of the ECSP that provides the EES Provider. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| NOTE: The corresponding EAS instantiation status of the EASID is instantiated. |  |  |

NOTE:	The list of EES DNAI(s) can include the DNAI(s) of the EAS(s) registered with the EES.

* * * Next Change * * * *

8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDN configuration information as defined in Table 8.3.3.3.3-2. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EASIDs (NOTE 2, 3) | O | List of EASIDs registered with the EES. |
| > EAS information of instantiable but not instantiated yet | O | The EAS information(e.g. EASID) which is instantiable but not instantiated yet |
| > EES Provider identifier | O | The identifier of the EES Provider (such as ECSP) |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.2. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: The corresponding EAS instantiation status of the EASID is instantiated. |  |  |

* * * Next Change * * * *

8.3.3	Service provisioning

8.3.3.1	General

The following clauses specify procedures, information flows and APIs for service provisioning.

8.3.3.2	Procedures

8.3.3.2.1	General

Following procedures are supported for service provisioning:

-	Request-response procedure;

-	Subscribe-notify procedures, including:

-	Subscription procedure;

-	Notification procedure;

-	Subscription update procedure; and

-	Unsubscribe procedure.

8.3.3.2.2	Request-response model

Figure 8.3.3.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization; and

4.	The ECS is configured with ECSP's policy for service provisioning.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.3.3.2.2-1: Service provisioning – Request/Response

1.	The EEC sends a service provisioning request to the ECS. The service provisioning request includes the security credentials of the EEC received during EEC authorization procedure and may include the UE identifier such as GPSI, connectivity information, UE location and AC profile(s) information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.10.2. If AC profile(s) are provided by the EEC, the ECS identifies the EES(s) based on the provided AC profile(s) and the UE location. When AC profiles(s) are not provided, then:

-	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location;

-	ECS identifies the EES(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 2:	Details of the UE-specific service information and how it is available at the ECS is out of scope.

NOTE 3:	Both steps are evaluated prior to sending a response.

The ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints.

3.	If the processing of the request was successful, the ECS responds to the EEC's request with a service provisioning response which includes a list of EDN configuration information, e.g. identification of the EDN, EDN service area, and the required information (e.g. URI, IP address) for establishing a connection to the EES.

If the ECS is not provisioned with any EDN configuration information or is unable to determine the EES information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

If the EDN configuration information includes an LADN DNN as an identifier for the EDN, the EEC considers the LADN as the EDN. Therefore, the service area of EDN is the LADN Service Area which can be discovered using the UE Registration Procedure.

The EEC may cache the service provisioning information (e.g. EES endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the Service provisioning response, then the EEC may cache and reuse the Service provisioning information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

If the ECS provided information regarding the service continuity support of individual EESs, the EEC may take this information into account when selecting an EES for EEC registration, EAS discovery or T-EAS discovery, respectively.

When the EES profile(s) include EAS information of instantiable but not instantiated yet, the EEC can select one EES for EAS discovery to mitigate the waste of EDN resources, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated.

NOTE 4:	If the service provisioning request fails, the EEC can resend the service provisioning request again, taking into account the received failure cause.

NOTE 5:	Even after the EEC establishes a connection to the EES using information received in step 3, the EES can issue AF request to influence traffic routing from EEC to EES as specified in 3GPP TS 23.501 [2] clause 5.6.7.

NOTE 6: If the EAS instantiation fails based on the selected EES, the EEC may retry the EAS discovery request to another EES.